3GPP TSG-SA WG6 Meeting #54-e	S6-231199

17th – 21st April 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0285 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Information flows for roaming and federation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 8.17 describes procedures to support roaming and federation, but the clause does not have description about information flow. To clarify what information is sent during the procedures, "Information flow" clause is added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added clauses for information flows. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Information flows are undefined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.X (new), 8.17.3 (new), 8.17.3.1 (new), 8.17.3.2 (new), 8.17.3.3 (new), 8.17.3.4 (new), 8.17.3.5 (new), 8.17.3.6 (new), 8.17.3.7 (new), 8.17.3.8 (new), 8.17.3.9 (new), 8.17.3.10 (new), 8.17.3.11 (new), 8.17.3.12 (new), 8.17.3.13 (new), 8.17.3.14 (new), 8.17.3.15 (new), 8.17.3.16 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | KI #6, #10, Solution #50 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 7.2.X	Edge Configuration Server ID (ECSID)

The ECSID identifies an ECS and each ECSID is unique within PLMN domain.

* * * First Change * * * *

### 8.17.3	Information flows

#### 8.17.3.1	General

The information flows are specified to support roaming and federation.

#### 8.17.3.2	ECS registration request

Table 8.17.3.2-1 describes information elements for the ECS registration request sent from the ECS to the ECS-ER.

Table 8.17.3.2-1: ECS registration request

| Information element | Status | Description |
| --- | --- | --- |
| ECSID | M | Identifier of the ECS sending the request |
| Security credentials | M | Security credentials of the ECS sending the request |
| ECS configuration information | M | Configuration information of the ECS |
| List of EDN configuration information | M | One or more configuration information of EDN(s) available via the ECS |
| Expiration time | O | Indicates the expiration time of the registration. To maintain an active registration status, a registration update is required before the expiration time. |
| List of partner ECSPs | O | List of partner ECSPs that are allowed to receive the ECS's information |
| DNN | O | DNN required for establish PDU session to the ECS |
| S-NSSAI | O | S-NSSAI required for establish PDU session to the ECS |

#### 8.17.3.3	ECS registration response

Table 8.17.3.3-1 describes information elements for the ECS registration response sent from the ECS-ER to the ECS.

Table 8.17.3.3-1: ECS registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the registration request was successful. |
| > Registration ID | M | Identifier of the ECS registration. |
| > Expiration time | O | Indicates the expiration time of the registration. To maintain an active registration status, a registration update is required before the expiration time. |
| Failure response | O | Indicates that the registration request failed. |
| > Cause | M | Provides the cause for registration request failure. |

#### 8.17.3.4	ECS registration update request

Table 8.17.3.4-1 describes information elements in the ECS registration update request sent by the ECS to the ECS-ER.

Table 8.17.3.4-1: ECS registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Registration ID | M | ECS registration identifier provided by the ECS-ER during ECS registration. |
| Security credentials | M | Security credentials of the ECS sending the request. |
| ECS configuration information | M | Configuration information of the ECS |
| List of EDN configuration information | M | One or more configuration information of EDN(s) available via the ECS |
| Expiration time | O | Indicates the expiration time of the registration. To maintain an active registration status, a registration update is required before the expiration time. |
| List of partner ECSPs | O | List of partner ECSPs that are allowed to receive the ECS's information |
| DNN | O | DNN required for establish PDU session to the ECS |
| S-NSSAI | O | S-NSSAI required for establish PDU session to the ECS |

#### 8.17.3.5	ECS registration update response

Table 8.14.3.5-1 describes information elements in the ECS registration update response sent by the ECS-ER to the ECS.

Table 8.17.3.5-1: ECS registration update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the registration request was successful. |
| > Expiration time | O | Indicates the expiration time of the registration. To maintain an active registration status, a registration update is required before the expiration time. |
| Failure response | O | Indicates that the registration request failed. |
| > Cause | M | Provides the cause for registration request failure. |

#### 8.17.3.6	ECS de-registration request

Table 8.17.3.6-1 describes information elements in the ECS de-registration request sent by the ECS to the ECS-ER.

Table 8.17.3.6-1: ECS de-registration request

| Information element | Status | Description |
| --- | --- | --- |
| Registration ID | M | ECS registration identifier provided by the ECS-ER during ECS registration. |
| Security credentials | M | Security credentials of the ECS sending the de-registration request. |

#### 8.17.3.7	ECS de-registration response

Table 8.17.3.7-1 describes information elements in the ECS de-registration response sent by the ECS-ER to the ECS.

Table 8.17.3.7-1: ECS de-registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the deregistration request was successful. |
| Failure response | O | Indicates that the deregistration request failed. |
| > Cause | M | Provides the cause for deregistration request failure. |

#### 8.17.3.8	ECS discovery request

Table 8.17.3.8-1 describes information elements for the ECS discovery request sent by the ECS to the ECS-ER.

Table 8.17.3.8-1: ECS discovery request

| Information element | Status | Description |
| --- | --- | --- |
| ECSID | M | Identifier of the ECS sending the request |
| Security credentials | M | Security credentials of the ECS. |
| Service provisioning filters | O | Service provisioning filters received from the EEC or the EES |
| UE location | O | Current location of the UE hosting the EEC |
| Serving PLMN (NOTE) | O | PLMNID of the serving PLMN |
| NOTE: This IE shall be included if the UE hosting the EEC is roaming. |  |  |

#### 8.17.3.9	ECS discovery response

Table 8.17.3.9-1 describes information elements for the ECS discovery response sent by the ECS-ER to the ECS.

Table 8.17.3.9-1: ECS discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the ECS discovery request was successful. |
| > Discovered ECS list | M | List of discovered ECS(s). Each element includes the information described below. |
| >> ECS configuration information | M | Configuration information of the ECS |
| >> DNN (NOTE) | O | DNN required for establish an LBO PDU session to the ECS |
| >> S-NSSAI (NOTE) | O | S-NSSAI required for establish an LBO PDU session to the ECS |
| Failure response | O | Indicates that the EAS discovery request failed. |
| > Cause | M | Indicates the cause of EAS discovery request failure. |
| NOTE: These IEs may be included if the UE hosting the EEC is roaming. |  |  |

#### 8.17.3.10	ECS discovery subscription request

Table 8.17.3.10-1 describes information elements for the ECS discovery subscription request sent by the ECS to the ECS-ER.

Table 8.17.3.10-1: ECS discovery subscription request

| Information element | Status | Description |
| --- | --- | --- |
| ECSID | M | Identifier of the ECS sending the request |
| Security credentials | M | Security credentials of the ECS. |
| List of ECS discovery filters | O | List of filters to discover ECS(s). Each element includes the information described below. |
| > Service provisioning filters | O | Service provisioning filters received from the EEC or the EES |
| > UE location | O | Location of the UE hosting the EEC |
| > Serving PLMN (NOTE) | O | PLMNID of the serving PLMN |
| NOTE: This IE shall be included if the UE hosting the EEC is roaming. |  |  |

#### 8.17.3.11	ECS discovery subscription response

Table 8.17.3.11-1 describes information elements for the ECS discovery subscription response sent by the ECS-ER to the ECS.

Table 8.17.3.11-1: ECS discovery subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the ECS services subscription request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O | Indicates that the subscription request failed. |
| > Cause | M | Indicates the cause of subscription request failure |

#### 8.17.3.12	ECS discovery notification

Table 8.17.3.12-1 describes information elements for the ECS discovery notification sent by the ECS-ER to the ECS.

Table 8.17.3.12-1: ECS discovery notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the notification. |
| Notification details | M | Information related to the notification |
| > Discovered ECS list | M | List of discovered ECS(s). Each element includes the information described below. |
| >> ECS configuration information | M | Configuration information of the ECS |
| >> DNN (NOTE) | O | DNN required for establish an LBO PDU session to the ECS |
| >> S-NSSAI (NOTE) | O | S-NSSAI required for establish an LBO PDU session to the ECS |
| NOTE: These IEs may be included if the UE hosting the EEC is roaming. |  |  |

#### 8.17.3.13	ECS discovery subscription update request

Table 8.17.3.13-1 describes information elements for the ECS discovery subscription update request sent by the ECS to the ECS-ER.

Table 8.17.3.13-1: ECS discovery subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the update request. |
| Security credentials | M | Security credentials of the ECS. |
| List of ECS discovery filters | O | List of filters to discover ECS(s). Each element includes the information described below. |
| > Service provisioning filters | O | Service provisioning filters received from the EEC or the EES |
| > UE location | O | Location of the UE hosting the EEC |
| > Serving PLMN (NOTE) | O | PLMNID of the serving PLMN |
| NOTE: This IE shall be included if the UE hosting the EEC is roaming. |  |  |

#### 8.17.3.14	ECS discovery subscription update response

Table 8.17.3.14-1 describes information elements for the ECS discovery subscription update response sent by the ECS-ER to the ECS.

Table 8.17.3.14-1: ECS discovery subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the ECS discovery subscription update request was successful. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O | Indicates that the subscription update request failed. |
| > Cause | M | Indicates the cause of subscription update request failure |

#### 8.17.3.15	ECS discovery unsubscribe request

Table 8.17.3.15-1 describes information elements for the ECS discovery unsubscribe request sent by the ECS to the ECS-ER.

Table 8.17.3.15-1: ECS discovery unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | ECS discovery subscription identifier provided by the ECS-ER during subscription. |
| Security credentials | M | Security credentials of the ECS sending the ECS discovery unsubscribe request. |

#### 8.17.3.16	ECS discovery unsubscribe response

Table 8.17.3.16-1 describes information elements for the ECS discovery unsubscribe response sent by the ECS-ER to the ECS.

Table 8.17.3.16-1: ECS discovery unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the ECS discovery unsubscribe request was successful. |
| Failure response | O | Indicates that the ECS discovery unsubscribe request failed. |
| > Cause | M | Provides the cause for ECS discovery unsubscribe request failure. |