3GPP TSG-SA WG6 Meeting #54-e	S6-231203

Online 17th – 21st April 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0289 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving EN about term alignment between ECSP and OP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-11 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The term "ECSP" and "OP" are used randomly. Since "OP" is a language used in GSMA OPG, the term "ECSP" is more suitable and consistent to use in this 3GPP TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Mainly use ECSP to differentiate two different edge environments/providers. Add a NOTE saying that an OP in GSMA OPG is provided by an ECSP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Both "ECSP" and "OP" are used randomly in the TS and confuse readers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.18.1, 8.18.2, 8.18.2.1, 8.18.2.1.1, 8.18.2.1.2, 8.18.2.2, 8.18.2.4, 8.18.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.18.1	General

In the following clauses, the ECSP-A is a partner of ECSP-B and ECSP-B is the leading ECSP to serve the UE.

NOTE:	An operator platform (OP) in GSMA OPG reference architecture (see Annex D) is provided by an ECSP.

The information and procedure of service provisioning and EAS discovery described in the following clauses are same as clause 8.3.3 and clause 8.5, only differences are described in this clause for edge node sharing.

Editor's note: Reusing existing procedures and information flows (e.g., service provisioning or EAS discovery) for Edge Node Sharing is FFS, it is also FFS how to merge the procedures related with ECS-ER in this clause with 8.17 support of roaming and federation.

### 8.18.2	Procedure

#### 8.18.2.1	Application information sharing between leading ECSP and partner ECSP

Editor's note: The security aspect between ECS-ER (ECSP-B) and ECS-ER (ECSP-A) is FFS and needs to be studied by SA3.

Editor's note: whether more procedures & operations are needed between ECS-ERs is FFS.

##### 8.18.2.1.1	request-response model

Figure 8.18.2.1.1-1: Fetch application information from partner

1)	The ECS-ER (ECSP-B) may fetch application information from its partner ECS-ER (ECSP-A) periodically.

2)	In such a fetch operation, the ECS-ER (ECSP A) provides response which includes a list of EAS IDs and EES information of ECSP-A.

NOTE:	If the ECS (ECSP-B) does not receive EES information of ECSP-A from the published/notified application information, the ECS (ECSP-B) can also fetch it from the ECS (ECSP-A) via the fetch operation.

##### 8.18.2.1.2	Subscribe notify model

The ECS-ER (ECSP-A) subscribes for application information from its partner ECSP-B and receives notifications as shown in figure 8.18.2.1.2-1 and figure 8.18.2.1.2-2.

Pre-conditions:

1.	The federation agreement exists between ECSP-A and ECSP-B;

Figure 8.18.2.1.2-1: Subscribe for application information

1)	Based on service level agreement between ECSP-A and ECSP-B, the ECS-ER (ECSP-B) sends subscription request to ECS-ER (ECSP-A) to receive registered EAS information and its including service area. The request includes required parameters like federation identifier, ECSP identifier, ECS identifier, required application identifiers and security parameters.

2)	Upon receiving the request from ECS-ER (ECSP-B), the ECS-ER (ECSP-A) checks whether ECS-ER (ECSP-B) is authorized to receive registered EAS list or not based on service level agreement.

3)	The ECS-ER (ECSP-A) sends subscription response – which indicates result of the subscription (success or failure) and subscription identity (if the result is success).

Figure 8.18.2.1.2-2: Notification for application information

1)	The list of registered and allowed EAS is changed on the ECS-ER (ECSP-A) – that is either new EAS is registered or EAS registration is updated or EAS is deregistered or new EAS is published or already published EAS is unpublished.

5)	The ECS-ER (ECSP-A) sends notification to the ECS-ER (ECSP-B). The notification includes the registered and allowed EAS between ECSP-A and ECSP-B, EASID and EAS Geographical Service. Upon receiving the notification, the Edge repository (of ECSP-A) stores the information to be used in other procedures (for EAS discovery or get registered EAS information request).

#### 8.18.2.2	Retrieval of EES of partner ECSP from Leading ECSP

Pre-conditions:

1. ECSP-A and ECSP-B have a service level agreement to share edge services.

2. EAS registers in Partner ECSP-A's EES.

Figure 8.18.2.2-1: Service provisioning with EES information retrieval from parnter

1.	The EEC sends service provisioning request to the ECS (ECSP-B).

2.	The ECS (ECSP-B) cannot find any desired EAS ID in the EES profile being registered so it determines to use edge service from its partner ECSP-A.

3.	ECS (ECSP-B) discovers EES from ECS (ECSP-A).

NOTE:	The ECS (ECSP-B) can have SLA with other multiple ECSPs and need to repeat step 3 with more than one partner ECSP in order to discover EES.

4.	The service provisioning response includes the EES (ECSP-A) endpoint and the ECS (ECSP-B) determined EES (ECSP-B) endpoint. The endpoint of EES (ECSP-B) is determined based on SLA with ECSP-A so that the EES (ECSP-B) can be authorized by the EES (ECSP-A). The endpoint of EES (ECSP-A) is sent by EES in a way that is transparent to EEC.8.18.2.3	Service provisioning with the published application information

Pre-conditions:

- EAS registers in Partner ECSP-A's EES.

Figure 8.18.2.3-1: Service provisioning with published application information

1.	The EEC sends service provisioning request to the ECS (ECSP-B).

2.	The ECS (ECSP-B) cannot find any desired EAS ID in the EES profile being registered so it determines to use edge node sharing service from its partner ECSP-A based on its partner's published application information. For the received EAS ID, since the ECS (ECSP-B) has EES information that was published by partner ECSP, the ECS (ECSP-B) discovers EES (i.e. EES(s) from partner) locally.

3.	The ECS(ECSP-B) returns discovered EES(s) of ECSP-A to the EEC.

#### 8.18.2.4	EAS discovery via Leading ECSP

Pre-conditions:

1.	ECSP-1 and ECSP-2 have a service level agreement to share edge services.

2.	EES (ECSP-B) has received EAS discovery request where required EAS is not available with the EES (ECSP-B).

Figure 8.18.2.4-1: EAS discovery for edge node sharing

1.	The EEC sends EAS discovery request to EES (ECSP-B) and may include the endpoint of EES (ECSP-A) in the request.

2.	The EES (ECSP-B) validates the edge service SLA. If EES (ECSP-A) endpoint is included, the EES (ECSP-B) skips step 3-5; otherwise, the EES (ECSP-B) executes for EAS discovery but cannot find any desired EAS so it determines to use edge node sharing service based on edge service SLA.

3.	The EES (ECSP-B) sends Retrieve EES request to the ECS (ECSP-B). The request may include an edge node sharing indication indicating edge node sharing is requested so that the ECS (ECSP-B) can skip checking T-EES(s) registered in the ECS (ECSP-B).

NOTE:	If EES (ECSP-A) information is not carried in EAS discovery request in step 1, the EES (ECSP-B) can determine to retrieve EES (ECSP-A) information from ECS (ECSP-B) as in step 3-5.

4.	For the received EAS ID, the ECS (ECSP-B) discovers T-EES (i.e. EES(s) from partner).

5.	The ECS (ECSP-B) returns discovered EES(s) of ECSP-A in the Retrieve EES response to the ECS (ECSP-B).

6.	The EES (ECSP-B) sends EAS discovery request to the EES (ECSP-A). The EAS discovery request includes the serving MNO information (e.g. MNO name, PLMN ID) which is serving the UE.

7.	The EES (ECSP-A) validates the edge service SLA and returns EAS discovery response including the discovered candidate EAS(s) to the EES (ECSP-B). The candidate EAS(s) are discovered based on the serving MNO information received from the EES (ECSP-B) and allowed MNO information registered by the EAS.

8.	The EEC receives the EAS discovery response sent by the EES (ECSP-B). The EEC may select an EAS for the AC.

Editor's note:	It is FFS whether to add the interaction between EES (ECSP-B) and ECS (ECSP-B) for T-EES retrieval after step 2.

#### 8.18.2.5	EAS discovery via Partner ECSP, EEC triggered

Pre-conditions:

1.	The EEC has received information (e.g. URI, IP address) related to the partner EES;

2.	The EEC has received appropriate security credentials authorizing it to communicate with the parnterEES as specified in clause 8.11;

3.	The partner EES is configured with ECSP's policy for EAS discovery and;

4.	Partner EES is only registered with ECS of lead ECSP.

NOTE:	Details of ECSP's policy are out of scope.

Figure 8.18.2.5-1: EAS discovery via Partner EES, EEC triggered

1.	The EEC sends an EAS discovery request to the EES (partner ECSP). It includes all the information element specified in 8.5.3.2, additionally, the UEs serving MNO information (e.g. MNO name, PLMN ID) is also included.

2.	Upon receiving the request from the EEC, the EES (partner ECSP) performs the authorization check and discovers the suitable EAS(s) as specified in clause 8.5.3.3 and identifies the EAS considering the EAS's allowed MNO information and UE's serving MNO information.

3.	The EES (partner ECSP) sends the EAS discovery response message to EEC as specified in 8.5.3.3.