3GPP TSG-SA WG6 Meeting #54-e	S6-231221

# Online, , 17th Apr 2023 - 26th Apr 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0292 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inclusion of EASID(s) to identify unfulffiled EASID within the ACID for the EEC registration response and EEC registration update response. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | IID Delhi, IIT Bhilai |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | An AC may be served by multiple EASID(s), mentioned as the list of EASID in AC profile. The information element "List of unfulfilled AC information" in table 8.4.2.3.3-1 of the clause 8.4.2.3.3 "EEC registration response" and table 8.4.2.3.5-1 of the clause “EEC registration update response” shall include the list of unfulfilled AC information as well. The AC(s) requirement may be partially fulfilled by the EES, as IE "reason" alone cannot specify which of the services, corresponding to ACID, are not fulfilled. Appending list of unfulfilled EASID(s) in the response may allowanAC to update its AC profile with a reduced list of EASID(s). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Inclusion of EASID(s) corresponding to the unfulfilled ACID in table 8.4.2.3.3-1 of the clause 8.4.2.3.3 "EEC registration request" and table 8.4.2.3.5-1 of the clause "EEC registration update response" enable AC to update its AC profile with a reduced list of EASID(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EEC may not be able to identify the unfulfilled EASID(s) corresponding to the respective unfulfilled ACID as IE "reason" may not provide sufficient information. Inclusion of list of unfulfilled EASID(s) so AC may update its AC profile with a reduced list of EASID(s). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.3.3, 8.4.2.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 8.4.2.3.3 EEC registration response

Table 8.4.2.3.3-1 describes information elements in the EEC registration response from the EES to the EEC.

Table 8.4.2.3.3-1: EEC registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the registration request was successful. |
| > Registration ID | M | Identifier of the EEC registration. |
| > Expiration time (see NOTE) | O | Indicates the expiration time of the registration. To maintain an active registration status, a registration update is required before the expiration time. |
| > EEC context ID | O | Identifier of the EEC Context information available at the EES that performed the registration. |
| > EEC Context Relocation status | O | Indicates whether the EEC context retrieval from the S-EES was successful or not. |
| > Discovered EAS list | O | List of EASs discovered to provide the capabilities required by the AC Profiles. If the request includes the EAS selection request indicator, then Discovered EAS list shall contain only one selected EAS. If the EES selects no EASs, the list may be empty. |
| >> EAS profile | M | Profile of the EAS. Each element is described in clause 8.2.4. |
| > list of unfulfilled AC information | O | List of ACIDs of the AC Profile(s) for which the requirements indicated in the AC profile(s) cannot be fulfilled |
| >> ACID | M | Application Identifier |
| >>>EASID | O | Identifier of the EAS |
| >> reason | O | Reason indicating the cause (e.g. EAS not available, requirements cannot be fulfilled) |
| Failure response | O | Indicates that the registration request failed. |
| > Cause | M | Provides the cause for registration request failure. |
| NOTE: The Expiration time IE shall be included if based on the ECSP policy the EES requires EEC to periodically refresh the EEC registration. |  |  |

END OF FIRST CHANGE

SECOND CHANGE

##### 8.4.2.3.5	EEC registration update response

Table 8.4.2.3.5-1 describes information elements in the EEC registration update response from the EES to the EEC.

Table 8.4.2.3.5-1: EEC registration update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the registration update request was successful. |
| > Expiration time (see NOTE) | O | Indicates the expiration time of the registration. To maintain an active registration status, a registration update is required before the expiration time. |
| > list of unfulfilled AC information | O | List of ACIDs of the AC Profile(s) for which the requirements indicated in the AC profile(s) cannot be fulfilled |
| >> ACID | M | Application Identifier |
| >>>EASID | O | Identifier of the EAS |
| >> reason | O | Reson indicating the cause (e.g. EAS not available, requirements cannot be fulfilled) |
| Failure response | O | Indicates that the registration update request failed. |
| NOTE: The Expiration time IE shall be included if based on the ECSP policy the EES requires EEC to periodically refresh the EEC registration. |  |  |

END OF CHANGES