3GPP TSG-SA WG6 Meeting #54e	S6-231227

e-meeting,  17th – 26th April 2023		(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0194 | rev | - | Current version: | 18.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add “Location service deregistration” procedure and information flow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GFLS |  |  |  |  |  | Date: |  |  | 2023-04-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Location service registration procedure from LM Client to LM Server has been documented and agreed, the Location service deregistration procedure and information flow should also be added in case the LM Client desires to deregister the location service from LM Server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the Location service deregistration procedure and information flow. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5G-enabled fused location service capabilities will not support Location service deregistration for SEAL location management. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.x(new),9.3.2.x(new),9.3.2.y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3	Procedures and information flows for Location management (on-network)

9.3.x	Location service deregistration procedure

Figure 9.3.x-1 illustrates the procedure of client-triggered location service deregistration. By deresgistration, the Location Management Client may terminate the available location services which have registered to the Location Management Server before.

Pre-condition:

The Location Management Client has registered to the Location Management Server.

Figure 9.3.x-1: Location service deregistration procedure

1.	The LM Client decides to trigger the deresgistration request to LM Server to terminate the location services which have registered to the LM Server before.

2.	The LM Client sends location service deregistration request to the LM Server with the identifier of the UE, the optional  VAL service ID and deresgistration cause(e.g. switch off).

3.	The LM Server sends location service deregistration response to the LM Client and deletes the previously stored UE identifier information and related location capabilities(if any).

* * * Next Change * * * *

9.3.2.x	Location service deregistration request

Table 9.3.2.x-1 describes the information flow from the location management client to the location management server for deregistration of the location service.

Table 9.3.2.x-1: Location service deregistration request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or identity of the VAL UE. |
| VAL service ID | O | Identity of the VAL service for which the location service is deregistered. |
| Deregistration cause | O | The reason why VAL UE to trigger the deregistration request (e.g. switch off). |

* * * Next Change * * * *

9.3.2.y	Location service deregistration response

Table 9.3.2.y-1 describes the information flow from the location management server to the location management client for the location service deregistration response.

Table 9.3.2.y-1: Location service deregistration response

| Information element | Status | Description |
| --- | --- | --- |
| Deregistration status | M | It indicated the deregistration result. |

* * * End of Changes * * * *