3GPP TSG-SA WG6 Meeting #54e	S6-231228

e-meeting,  17th – 26th April 2023		(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0195 | rev | - | Current version: | 18.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add location reporting configuration notification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GFLS |  |  |  |  |  | Date: |  |  | 2023-04-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The “Location reporting configuration notification” information flow has been added and approved in CR0148 (S6-230412) in SA6#52-bis-e, however, it was not been included in the newest TS 23.434(V18.4.1).Thus I re-add it in the latest TS 23.434. The reason why to add “Location reporting configuration notification” is sometimes the LM server needs to notify/inform the LM client for updating the Location reporting configuration when it received the new requests/parameters from other entities (the VAL server or other LM client). What’s more, as the requested location access type and positioning method parameters have been introduced in “Location reporting configuration response” information flow, the “Location reporting configuration notification” is the information flow from LMS to notify/inform LMC the Location reporting configuration is update and the update paramaters may include the location access type and positioning method. So both new parameters should also be applicable for “Location reporting configuration notification” information flow. And the procedure for “Location reporting configuration notification” has also been supplemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add “Location reporting configuration notification” information flow and the related procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete sepecification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.1,9.3.2.x(new),9.3.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3.2.1	Location reporting configuration response

Table 9.3.2.1-1 describes the information flow from the location management server to the location management client for the location reporting configuration. This information flow may be sent individually addressed or group addressed on unicast or multicast.

Table 9.3.2.1-1: Location reporting configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or VAL group to which the location reporting configuration is targeted or identity of the VAL UE. |
| Requested location information | O (see NOTE) | Identifies what location information is requested |
| Requested location access type | O (see NOTE) | Identity of the location access type for which the location information is requested, e.g. as described in clause 6.1.6.3.6 and 6.1.6.3.17 of TS 29.572 [51]. |
| Requested positioning method | O (see NOTE) | Identity of the positioning method for which the location information is requested, e.g. as described in clause 6.1.6.3.6 of TS 29.572 [51]. |
| Triggering criteria | O (see NOTE) | Identifies when the location management client will send the location report. The triggering may be e.g., SAI changes, ECGI changes, RAT changes. |
| Minimum time between consecutive reports | O (see NOTE) | Defaults to 0 if absent otherwise indicates the time interval between consecutive reports |
| NOTE: If none of the information element is present, this represents a cancellation for location reporting. |  |  |

* * * Next Change * * * *

9.3.2.x	Location reporting configuration notification

Table 9.3.2.x-1 describes the information flow from the location management server to the location management client for the location reporting configuration notification when LM server needs to deliver the update location reporting configuration to LM client if the LM server received the new requests/parameters from other entities (e.g. the VAL server or other LM client) or based on local configurations. This information flow may be sent individually addressed or group addressed on unicast or multicast.

Table 9.3.2.x-1: Location reporting configuration notification

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or VAL group to which the location reporting configuration is targeted or identity of the VAL UE. |
| Requested location information | O (see NOTE) | Identifies what location information is requested |
| Requested location access type | O (see NOTE) | Identity of the location access type for which the location information is requested, e.g. as described in clause 6.1.6.3.6 and 6.1.6.3.17 of TS 29.572 [51]. |
| Requested positioning method | O (see NOTE) | Identity of the positioning method for which the location information is requested, e.g. as described in clause 6.1.6.3.6 of TS 29.572 [51]. |
| Triggering criteria | O (see NOTE) | Identifies when the location management client will send the location report. The triggering may be e.g., SAI changes, ECGI changes, RAT changes. |
| Minimum time between consecutive reports | O (see NOTE) | Defaults to 0 if absent otherwise indicates the time interval between consecutive reports |
| NOTE: If none of the information element is present, this represents a cancellation for location reporting. |  |  |

* * * Next Change * * * *

9.3.x  Location reporting configuration notification procedure

The location management server may update the location reporting configuration information and inform the location management client of the update ones at any time by triggering a location reporting configuration notification procedure.

Figure 9.3.x-1 illustrates the procedure for location reporting configuration notification.

Figure 9.3.x-1: Location reporting configuration notification procedure

1.	Based on the local configurations or the location information request from other entities (e.g., another location management client, VAL server), the location management server decides to update the location reporting configuration and informs the updated information to the location management client.

2.	The location management server sends location reporting configuration notification message to the location management client(s) which may contain the updated location reporting configuration, e.g. the event triggers, the minimum time between consecutive reports, the requested location access type and positioning methods when VAL UE fetches the UE location report. This message can be sent over a unicast bearer to a specific location management client or as a group message over an MBMS bearer to update the location reporting configuration for multiple location management clients at the same time.

NOTE:	Different location management clients may be given different location reporting criteria.

3.	The location management client stores or updates the new location reporting configuration received in step 2.

* * * End of Changes * * * *