3GPP TSG-SA WG6 Meeting #54-e	S6-231282

17th – 26th April 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0307 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | T-EAS discovery enhancement for composite EAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the composite EAS case, the main EAS may belongs to different composite EAS, accordingly different combination of the composite EAS may provide different service to different AC. Thus the Retrieve T-EES should be based on the AC information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhance the EES profile with the AC information corresponding to the EASID The T-EES determination should be based on the AC information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | only using the EASID to discovery the T-EES cannot ensure that the all the required EAS are available at the T-EES/Target EDN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.6, 8.8.3.3, 8.8.4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.2.6	EES Profile

The EES profile includes information about the EES and the services it provides.

Table 8.2.6-1: EES Profile

| Information element | Status | Description |
| --- | --- | --- |
| EESID | M | The identifier of the EES |
| EES Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EES. This information is provided to the EEC to connect to the EES. |
| EASIDs | M | List of EASIDs registered or expected to be registered with the EES. |
| > ACID | O | Identifier of the AC which the EAS can provide service to. |
| > List of EAS bundle information | O | List of EAS bundles to which the EAS belongs and related bundling requirements. |
| >> Bundle ID | M | A bundle ID as described in clause 7.2.10. |
| Instantiable EAS information | O | The EAS instantiation status per EASID (e.g. instantiated, instantiable but not be instantiated yet). |
| > Instantiation criteria (see NOTE) | O | The criteria upon which EAS can be instantiated (e.g. based on specific date and time). |
| EEC registration configuration | M | Indicates whether the EEC is required to register on the EES to use edge services or not. |
| ECSP ID | O | The identifier of the ECSP that provides the EES. |
| EES Topological Service Area | O | The EES serves UEs that are connected to the Core Network from one of the cells included in this service area. EECs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| List of EES DNAI(s) | O | DNAI(s) associated with the EES. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN, where the EES resides. |
| EES Service continuity support | O | Indicates if the EES supports service continuity or not. This IE indicates which ACR scenarios are supported by the EES, also indicates the EES ability of handling bundled EAS ACR. |
| NOTE: "Instantiation criteria" IE shall be present only when the value of "Instantiable EAS information" IE is "instantiable but not be instantiated yet". |  |  |

NOTE:	The list of EES DNAI(s) can include the DNAI(s) of the EAS(s) registered with the EES.

Editor's Note:	Whether EAS bundle requirement is needed in EES profile is FFS.

* * * NEXT Change * * * *

8.8.3.3	Retrieve T-EES procedure

Figure 8.8.3.3-1 illustrates the procedure for the S-EES to retrieve the T-EES information from the ECS.

Pre-condition:

1.	The S-EES has been pre-configured with the address of the ECS; and

2.	The AC at the UE already has on-going application traffic with the S-EAS.

Figure 8.8.3.3-1: Retrieve T-EES procedure

1.	The S-EES sends the Retrieve EES request (UE location information or UE identity, EASID of the S-EAS, ACID, target DNAI and UE connectivity information) to the ECS in order to identify the T-EES which has an EAS available to serve the given AC in the UE.

2.	If the request contains the UE identity (e.g. GPSI) but the UE location is not known to the ECS, then the ECS interacts with 3GPP core network to retrieve the UE location. The ECS determines T-EES(s) as per the parameters (e.g. EASID, target DNAI) in the request and the UE location information. When the ACID is provided then the ECS can identify the T-EES based on the ACID in the EES profile and in the request message to ensure that the EAS registered to the T-EES can provide service to the AC if the EAS belongs to different EAS composite.

If the ECS does not identify any suitable EES(s) based on EDN configuration available at the ECS and UE’s location, the ECS determines a partner ECS that may satisfy the requirements. Based on ECSP policy, the ECS may use preconfigured or OAM configured information about the partner ECSs or ECS discovery via ECS-ER as specified in clause 8.x.2.3 or both.

If required by the ECSP policies, the ECS may use service provisioning information retrieval procedure as specified in clause 8.X.2.4 to obtain service provisioning information from the partner ECS.

3.	The ECS sends the Retrieve EES response. If the ECS has determined the EDN configuration information, the retrieve EES response includes the list of EDN configuration information to the S-EES. The list of EDN configuration information includes the EDN details with the endpoint information of T-EES(s) as described in table 8.3.3.3.3-2. If the ECS has determined suitable partner ECS(s) instead, the retrieve EES response includes a list of ECS configuration information.

If the retrieve EES response contains a list of ECS configuration information, the S-EES may initiate retrieve T-EES procedure with one or more ECS(s) listed in the retrieve EES response.

NOTE:	The Retrieve EES request initiated by the S-EES can be restricted only to its registered ECS.

* * * NEXT Change * * * *

8.8.4.6	Retrieve EES request

Table 8.8.4.6-1 describes the information elements to retrieve T-EES information from the ECS.

Table 8.8.4.6-1: Retrieve EES request

| Information element | Status | Description |
| --- | --- | --- |
| EESID | M | Unique identifier of the EES. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EASID | M | The identifier of the EAS. |
| ACID(NOTE) | O | The identifier of the AC. |
| Target DNAI | O | The target DNAI information which can be associated with potential T-EES(s) and/or T-EAS(s). |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| UE location | O | The location information of the UE. The UE location is described in clause 7.3.2. |

* * * END of Change * * * *