3GPP TSG-SA WG6 Meeting #53	S6-231304

Athens, Greece 27th February – 3rd March 2023	(revision of S6-230670)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0242 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reliable Edge Computing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TR 23.700-98, Solution #48 Edge server set and edge service set is studied and concluded. It is required to specify the solution in normative work in TS 23.558. The Edge server set is specified by adopting the the mechanism defined in TS 23.434. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added new clause to specify reuse of the roles defined in TS 23.434 - Specified Context synchronization procedure and IE tables - Updated EES profile with EES Set ID - Reuse service provisioning request-response model for discovering equivalent EES from EES Set |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reliable edge computing feature will not be supported in Rel-18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.x (New), 8.x.1 (New), 8.x.1.1 (New), 8.x.2 (New), 8.x.2.1 (New), 8.x.3 (New), 8.x.3.1 (New), 8.x.3.2 (New), 8.2.6, 8.3.3.3.2, 8.3.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Related to KI#23, Sol#48 of TR 23.700-98 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 8.x	Reliability support with Sets

### 8.x.1	General

#### 8.x.1.1	EES reliability

The EEL shall uses the mechanism as defined in clause X of 3GPP TS 23.434 [13] for providing reliability for EES. In this specification, EEC and EAS act as a consumer role, EES acts as a producer role and ECS may act as a registrar role.  An EES set shall be introduced to enable an EES (as a producer) to provide reliable services to the consumers (e.g. EEC, EAS). The EES set is equivalent of AES set as defined in clause X of 3GPP TS 23.434 [13].

Binding information is shared to EEC by EES or ECS in any request-response (e.g. Service provisioning, EEC registration, EAS discovery) or subscribe-notify mechanism (e.g. service provisioning subscription, EAS discovery subscription).

Binding information is shared to EAC by EES in any request-response (e.g. EAS registration, T-EAS discovery) or subscribe-notify mechanism (e.g. ACR management event subscribe).

EEC may subscribe to availability change notifications of EES from the ECS using service provisioning subscribe procedure as defined in clause 8.3.3.2.3, and may discover equivalent EES from EES set using service provisioning request-response procedure as defined in clause 8.3.3.2.2.

Editor’s note:	Reliability aspects for EAS and ECS is FFS.

* * * Next Change * * * *

### 8.x.2	Procedures

#### 8.x.2.1	Context synchronization procedure

The procedure is used to transfer all context (EEC context of all registered EECs, EAS profile of all registered EASs from S-EES to T-EES where both S-EES and T-EES are from same EES set.

Pre-conditions:

1.	The source EES and Target EES are from same EES set.

Figure 8.9.2.2-1: EES Context synchronization procedure

1.	The source EES determines to transfer all context to a targer EES of the same EES set. The source EES determines the target EES and sends request to the target EES to synchronize content of the S-EES. The request includes context details as specified in Table 8.x.3.1-1.

2.	Upon receiving the request from the source EES, the target EES validates the request and verifies the security credentials. The target EES uses the EES Set ID provided to authorize the context to be stored and managed. Then the target EES sends an EEC Context response indicating success.

### 8.x.3	Information flows

#### 8.x.3.1	Content synchronization request

Table 8.x.3.1-1 describes information elements in the Content synchronization request between two EESs.

Table 8.x.3.1-1: Content synchronization request

| Information element | Status | Description |
| --- | --- | --- |
| EES ID | M | Unique identifier of the requesting EES. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EES Set context (NOTE 2) | O | Provides EES context to synchronize |
| > List of EEC Contexts | O | List of registered EEC Contexts to sync with target EES from the same EES Set. |
| > List of EAS Profiles | O | List of registered EAS profiles to sync with target EES from the same EES Set. |
| > List of EDGE-3 subscriptions | O | List of subscriptions IDs over EDGE-3. |
| > EES Set ID | O | Identifies the EES set from which the S-EES requests to synchronize the context T-EES. |

#### 8.x.3.2	Content synchronization response

Table 8.x.3.2-1 describes information elements in the Content synchronization response between two EESs.

Table 8.x.3.2-1: Content synchronization response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the request was successful. |
| Failure response (NOTE) | O | Indicates that the request failed. |
| > Cause | O | Indicates the cause of request failure, mandatory if the request failed. |
| NOTE: This IE shall be included if implicit registration is performed by T-EES. |  |  |

* * * Next Change * * * *

### 8.2.6	EES Profile

The EES profile includes information about the EES and the services it provides.

Table 8.2.6-1: EES Profile

| Information element | Status | Description |
| --- | --- | --- |
| EESID | M | The identifier of the EES |
| EES Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EES. This information is provided to the EEC to connect to the EES. |
| EASIDs | M | List of EASIDs registered or expected to be registered with the EES. |
| > List of EAS bundle information | O | List of EAS bundles to which the EAS belongs and related bundling requirements. |
| >> Bundle ID | M | A bundle ID as described in clause 7.2.10. |
| Instantiable EAS information | O | The EAS instantiation status per EASID (e.g. instantiated, instantiable but not be instantiated yet). |
| > Instantiation criteria (see NOTE) | O | The criteria upon which EAS can be instantiated (e.g. based on specific date and time). |
| EEC registration configuration | M | Indicates whether the EEC is required to register on the EES to use edge services or not. |
| ECSP ID | O | The identifier of the ECSP that provides the EES. |
| EES Topological Service Area | O | The EES serves UEs that are connected to the Core Network from one of the cells included in this service area. EECs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| List of EES DNAI(s) | O | DNAI(s) associated with the EES. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN, where the EES resides. |
| EES Service continuity support | O | Indicates if the EES supports service continuity or not. This IE indicates which ACR scenarios are supported by the EES, also indicates the EES ability of handling bundled EAS ACR. |
| EES Set ID | M | Identifies the EES set in which the EES belongs. |
| List of EESs | O | List of equivalent EESs from the EES set |
| NOTE: "Instantiation criteria" IE shall be present only when the value of "Instantiable EAS information" IE is "instantiable but not be instantiated yet". |  |  |

NOTE:	The list of EES DNAI(s) can include the DNAI(s) of the EAS(s) registered with the EES.

Editor's Note:	Whether EAS bundle requirement is needed in EES profile is FFS.

* * * Next Change * * * *

##### 8.3.3.3.2	Service provisioning request

Table 8.3.3.3.2-1 describes the information elements for service provisioning request from the EEC to the ECS.

Table 8.3.3.3.2-1: Service provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| EECID | M | Unique identifier of the EEC. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| AC Profile(s) | O | Information about services the EEC wants to connect to, as described in Table 8.2.2-1. |
| EEC Service Continuity Support | O | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC. When requesting service provisiojing for T-EES discovery, if the EEC requires that T-EES must support "EEC excuted ACR via T-EES" scenario, then EEC includes only "EEC excuted ACR via T-EES" in this IE |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Connectivity information | O | List of connectivity information for the UE, e.g. PLMN ID, SSID. |
| UE location | O | The location information of the UE. The UE location is described in clause 7.3.2. |
| ECSP identifiers | O | The list of desired EES provider IDs, by the EEC. |
| Equivalent EES discovery | O | Indicates ECS to provide list of equivalent EESs from the EES set. |
| > EES Set ID | O | Identifies the EES set in which the EES belongs. |
| > EESID | O | The identifier of the EES |
| > EES Endpoint | O | The endpoint address (e.g. URI, IP address) of the EES |
| > EES selection indication | O | Indicates ECS to select one equivalent EES from the EES set |

Editor's Note: it is FFS how EEC context pull request will work if S-EES does not support "EEC executed ACR via T-EES" scenario.

Editor's Note: [SA3] Whether the EECID and the UE ID included in request of EDGE-1 & 4 interactions is part of the security credential is SA3's responsibility.

##### 8.3.3.3.3	Service provisioning response

Table 8.3.3.3.3-1 describes the information elements for service provisioning response from the ECS to the EEC.

Table 8.3.3.3.3-1: Service provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the service provisioning request was successful. |
| > List of EDN configuration information | M | List of EDN configuration information as defined in Table 8.3.3.3.3-2. |
| Failure response | O | Indicates that the service provisioning request failed. |
| > Cause | O | Indicates the cause of service provisioning request failure. |
| Redirect | O | Indicates redirection to (an)other ECS(s). |
| > ECS(s) information | M | Endpoint address of ECS(s) to which the UE is redirected for service provisioning. |
| > DNN | O | DNN required for establishing PDU Session to the redirected ECS |
| > S-NSSAI | O | S-NSSAI required for establishing PDU Session to the redirected ECS |

Table 8.3.3.3.3-2: EDN configuration information

| Information element | Status | Description |
| --- | --- | --- |
| EDN connection information (see NOTE 1) | M | Information required by the UE to establish connection with the EDN. |
| > DNN/APN | M | Data Network Name/Access Point Name |
| > S-NSSAI | O | Network Slice information |
| > EDN Topological Service Area | O | The EDN serves UEs that are connected to the Core Network from one of the cells included in this service area. See possible formats in Table 8.2.7-1. |
| List of EESs | M | List of EESs of the EDN. |
| > EESID | M | The identifier of the EES |
| > EES Endpoint | M | The endpoint address (e.g. URI, IP address) of the EES |
| > EASIDs (see NOTE 2) | O | List of EASID registered or expected to be registered with the EES. |
| >> List of EAS bundle information | O | List of EAS bundles to which the EAS belongs and related bundling requirements. |
| >>> Bundle ID or list of EASID | M | A list of EASIDs or a bundle ID as described in clause 7.2.10. |
| > Instantiable EAS information | O | The EAS instantiation status per EASID (e.g. instantiated, instantiable but not be instantiated yet) |
| >> Instantiation criteria (see NOTE 3) | O | The criteria upon which EAS can be instantiated (e.g. based on specific date and time). |
| > ECSP ID | O | The identifier of the ECSP that provides the EES. |
| > EES Topological Service Area | O | The EES serves UEs that are connected to the Core Network from one of the cells included in this service area. EECs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| > EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| > List of EES DNAI(s) | O | DNAI(s) associated with the EES/EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. |
| > EES Service continuity support | O | Indicates if the EES supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EES. |
| > EEC registration configuration | M | Indicates whether the EEC is required to register on the EES to use edge services or not. |
| > Security Credential | O | Indicates the security credential sent by the ECS. The security credential is used by EEC to communicate with the EES as specified in 3GPP TS 33.558 [23], clause 6.2. |
| > EES Set ID | M | Identifies the EES set in which the EES belongs. |
| Lifetime | O | Time duration for which the EDN configuration information is valid and supposed to be cached in the EEC. |
| NOTE 1: If the UE is provisioned or pre-configured with URSP rules by the HPLMN or serving SNPN, the UE handles the precedence between EDN connection info and URSP rules as defined in 3GPP TS 23.503 [12] clause 6.1.2.2.1. EDN connection info is considered to be part of UE Local Configurations. NOTE 2: EAS information is limited to the EEC requested applications. If no AC profiles were present in the service provisioning request, the EAS information is subject to the ECSP policy (e.g. no EAS information or a subset of EAS information related to the EES). NOTE 3: "Instantiation criteria" IE shall be present only when the value of "Instantiable EAS information" IE is "instantiable but not be instantiated yet". |  |  |

* * * End of Change * * * *