3GPP TSG-SA WG6 Meeting #54-e	S6-231311

# E-Meeting, 17th April – 21st April 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0200 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Create_Group service operation in the SS_GroupManagement API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL |  |  |  |  |  | Date: |  |  | 2022-MM-DD |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clauses 10.3.8.2 define the group creation functionality in the SEAL Group Management. The related input and output are present in clauses 10.3.2.26 and 10.3.2.27, respectively. However, clause 10.4.1 does not define a service operation to create a group. That creates functional misalignment between stage 2 and stage 3, i.e., according to stage 2 (23.434) there is no service operation that allows to create any group; however, stage 3 (29.549) implement the group creation functionality. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces the missed Create_Group service operation in the SS_GroupManagement API. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Impossibility to create any group via the SS_GroupManagement API. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4.1, 10.4.2.1, 10.4.2.1, 10.4.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

### 10.4.1	General

Table 10.4.1-1 illustrates the SEAL APIs for group management.

Table 10.4.1-1: List of SEAL APIs for group management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_GroupManagement | Query_Group_Info | VAL server | Request /Response |
|  | Update_Group_Info | VAL server | Request /Response |
|  | Create_Group | VAL server | Request /Response |
| SS_GroupManagementEvent | Subscribe_ Group_Info_Modification | VAL server | Subscribe/Notify |
|  | Notify_Group_Info_Modification | VAL server |  |
|  | Notify_Group_Creation | VAL server |  |

* * * Next Change * * * *

#### 10.4.2.1	General

API description: This API enables the VAL server to communicate with the group management server for the group creation, querying group information, obtaining stored group configuration, modify the group membership and configuration information on the group management server over GM-S.

* * * Next Change * * * *

#### 10.4.2.x	Create_Group operation

API operation name: Create_Group

Description: Create the group.

Known Consumers: VAL server.

Inputs: See subclause 10.3.2.26

Outputs: See subclause 10.3.2.27

See subclause 10.3.8.2 for the details of usage of this API operation.

* * * End of changes * * * *