3GPP TSG-SA WG6 Meeting #54-e	S6-231318

17th – 26th April 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 23.280 | CR | 0383 | rev | <Rev#> | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on subsequent group call on ad hoc group for emergency alert |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon, Hytera |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | IRail, MC_AHGC, enh4MCPTT |  |  |  |  |  | Date: |  |  | 2023-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When an ad hoc group emergency alert is initiated, an ad hoc group is created. Then the subsequent call on this ad hoc group is just a normal group, rather than a ad hoc group call as described in clause 10.10.3.3.1 step 9 which will cause a new ad hoc group being created. This shall be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the incorrect use of ad hoc group call in 10.10.3.3.1 and 10.10.3.3.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missleading to the downstream group |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.10.3.3.1, 10.10.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 10.10.3.3.1	Ad hoc group emergency alert initiation

Figure 10.10.3.3.1-1 illustrates the procedure for the MC service client initiating an ad hoc group emergency alert initiated by an authorized MC service user.

Pre-conditions:

1.	The MC service user at MC service client 1 wants to send an ad hoc group emergency alert to the MC service users who are satisying certain criteria.

2.	All MC service clients belong to the same MC system.

Figure 10.10.3.3.1-1: Ad hoc group emergency alert initiation

1.	The MC service user at the MC service client 1 initiates an ad hoc group emergency alert to an ad hoc group with participants satisfying specific criteria. The MC service user may add additional information describing the reason for sending the alert.

2.	MC service client 1 initiates the ad hoc group emergency alert by sending the ad hoc group emergency alert request containing the details of the criteria to be applied by the MC service server for determining the participants list or the list of participants. MC service client 1 may set its MC service emergency state. The MC service user at MC service client 1 may select a functional alias used for the MC service group emergency alert. If MC service client 1 is in MC service emergency state, the MC service emergency state is retained by the MC service client 1 until explicitly cancelled.

3.	MC service server checks whether the MC service user of MC service client 1 is authorized for initiation of ad hoc group emergency alerts and whether the request is supported. The MC service server checks whether the provided functional alias, if present, can be used and has been activated for the user.

4.	The MC service server determines the list of participants to be invited for the ad hoc group emergency alert based on the information present in the information element Criteria for determining the participants. This information element could carry either criteria or indicator identifying the criteria or combination of both. The MC service server stores the ad hoc group ID together with the list of participants for the duration of the ad hoc group emergency alert. The MC service server considers the alerted participants as implicitly affiliated to the ad hoc group.

Editor's note:	Sending an implicit affiliation indication to the alerted participant in the ad hoc group emergency alert request is FFS.

5.	The MC service server sends the ad hoc group emergency alert response to the MC service user 1 to confirm the ad hoc group emergency alert request is granted. The MC service server adds the ad hoc group ID (generated by the MC service server in the case where the MC service ad hoc group ID created by the MC service client 1 is not acceptable or the case where the MC service ad hoc group ID was not provided by MC service client 1).

6.	The MC service server sends an ad hoc group emergency alert request including the ad hoc group ID towards the MC service clients 2 and 3.

7.	MC service users are notified of the ad hoc group emergency alert. The functional alias of the MC service user initiating the ad hoc group emergency alert and additional information related to the alert may be displayed.

8.	The receiving MC service clients send the ad hoc group emergency alert response to the MC service server to acknowledge the ad hoc group emergency alert.

9.	It is configured, whether any MC service client may setup a subsequent group call over the above ad hoc group, using the ad hoc group ID from the ad hoc group emergency alert, according to 3GPP TS 23.379 [16].

NOTE 1:	MC service client 1 may start the group call over the above ad hoc group after step 5.

NOTE 2:	Sending the emergency alert without making a request to also start an emergency call does not put the group into the in-progress emergency state.

NOTE 3:	Sending the emergency alert does not put the other participants in the group into an MC service emergency state.

The MC service server continuously checks whether other MC service clients meet or if participating MC service clients no longer meet the criteria for the ad hoc group emergency alert.

* * * Next Change * * * *

##### 10.10.3.3.3	Entering an ongoing ad hoc group emergency alert

During an ongoing ad hoc group emergency alert, the MC service server continuously checks if additional MC service users meet the conditions of the ad hoc group emergency alert.

As illustrated in Figure 10.10.3.3.3-1 MC service client is added to the ad hoc group as the MC service client meets the criteria for receiving an ongoing ad hoc group emergency alert.

Pre-conditions:

1.	An ad hoc group emergency alert has been initiated and the related ad hoc group exists.

2.	The criteria for entering an ad hoc group emergency alert are known to the MC service server.

Figure 10.10.3.3.3-1: Entering an ongoing ad hoc group emergency alert

1.	MC service server acquires the latest information of the MC service user at the MC service client and checks whether the criteria for initiating an ad hoc group emergency alert to the MC service client are met.

2.	The MC service server sends an ad hoc group emergency alert request notification to MC service client to notify that it has moved into potential danger. The MC service client is added to the emergency ad hoc group.

3.	MC service client notifies the MC service user about the ad hoc group emergency alert.

4.	The receiving MC service client sends the ad hoc group emergency alert response to the MC service server to acknowledge the ad hoc group emergency alert.

5.	If there is a related emergency call ongoing over the ad hoc group from the ad hoc group emergency alert, the MC service server adds the MC service client as late entry to call according to 3GPP TS 23.379 [16].

##### Editor's note:	It is FFS whether a notification to the ad hoc group emergency alert participants is sent about entering the alert.10.10.3.3.4	Leaving an ad hoc group emergency alert

* * * End of Change * * * *