3GPP TSG-SA WG6 Meeting #54-e	S6-231320

17th – 26th April 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0102 | rev | <Rev#> | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS service area update failure cause handling in MC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon, Hytera |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2023.04.01 |
| Category: | F |  |  |  |  |  | Release: |  |  | 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current MBS session update with service area may fail and application errors listed in Table 6.2.3.2.3.1-3 and table 6.2.7.3-1 is returned as described in TS 29.532. Table 6.2.3.3.3.1-3: Data structures supported by the PATCH Response Body on this resource Data type; P; Cardinality; Response codes; Description. …; …; …. ProblemDetails; O; 0..1; 404 Not Found; When used to represent an unsuccessful MBS session update, the "cause" attribute of the "ProblemDetails" may be used to include one of the following application error codes: - UNKNOWN_MBS_SESSION, if the indicated MBS session is not found in the MB-SMF. - UNKNOWN_MBS_SERVICE_AREA, if the indicated MBS service area is not found.. Table 6.2.7.3-1: Application errors Application Error; HTTP status code; Description. …; …; …. UNKNOWN_MBS_SERVICE_AREA; 404 Not Found; This error indicates an unsuccessful of MBS session update, if the requested MBS service area cannot be found.. The same error code is relayed by the NEF towards the AF as described in Table 5.20.7.3-1: Application errors in TS 29.522. However, several cases may cause this error due to e.g., the MB-SMF is with a limited service area as shown bellow. Or the updated service area may not support 5G MBS at all. With the above cases, the AF/AS is not able to differentiate this error is caused by updated service area is beyound the severing MB-SMF, or there is no 5MBS at all. The MC service server does not know how to handle the error code. The appropriate handling at the MC service server is trying to create a new MBS session with the updated MBS service area, or using unicast PDU session. |
| Summary of change: |  | Introduce the MBS service area update failure handling description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UNKNOWN_MBS_SERVICE_AREA will not be well handled by the ASP |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************** First Changes ********************/

##### 7.3.3.2.x	MBS service area update failure handling

The MBS service area update to the ongoing MBS session may fail, and an error code "UNKNOWN_MBS_SERVICE_AREA" is returned from the core network as described in 3GPP TS 29.532 [xx] and 3GPP TS 29.532 [yy]. This may be caused due to the requested MBS service area are beyond the service area of the serving MB-SMF, or the MBS is not deployed at the updated MBS service area. Upon receiving this "UNKNOWN_MBS_SERVICE_AREA" error code, the MC service server may firstly try to create a new MBS session with the updated service area. If it still fails, the MC service server utilizes unicast PDU session for the group communication in that area.