3GPP TSG-SA WG6 Meeting #54-e	S6-231323

17th – 26th April 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0203 | rev | <Rev#> | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS service area failure cause handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon, Hytera |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph3 |  |  |  |  |  | Date: |  |  | 2023.04.01 |
| Category: | B |  |  |  |  |  | Release: |  |  | 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current MBS session update with service area may fail and application errors listed in Table 6.2.3.2.3.1-3 and table 6.2.7.3-1 is returned as described in TS 29.532. Table 6.2.3.3.3.1-3: Data structures supported by the PATCH Response Body on this resource Data type; P; Cardinality; Response codes; Description. …; …; …. ProblemDetails; O; 0..1; 404 Not Found; When used to represent an unsuccessful MBS session update, the "cause" attribute of the "ProblemDetails" may be used to include one of the following application error codes: - UNKNOWN_MBS_SESSION, if the indicated MBS session is not found in the MB-SMF. - UNKNOWN_MBS_SERVICE_AREA, if the indicated MBS service area is not found.. Table 6.2.7.3-1: Application errors Application Error; HTTP status code; Description. …; …; …. UNKNOWN_MBS_SERVICE_AREA; 404 Not Found; This error indicates an unsuccessful of MBS session update, if the requested MBS service area cannot be found.. The same error code is relayed by the NEF towards the AF as described in Table 5.20.7.3-1: Application errors in TS 29.522. However, several cases may cause this error due to e.g., the MB-SMF is with a limited service area as shown bellow. Also, the updated service area may not support 5G MBS at all. With the above cases, the AF/AS is not able to differentiate this error is caused by updated service area is beyound the severing MB-SMF, or there is no 5MBS at all. The AF/AS does not know how to handle the error code. The SEAL NRM server is designed to hide such compliated handlings from the VAL service. The SEAL NRM server may try to create a new MBS session with the updated MBS service area, or suggest using unicast PDU session to the VAL server. |
| Summary of change: |  | Introduce a new procedure that the NRM server upon receiving the UNKNOWN_MBS_SERVICE_AREA during the MBS session update procedure, will firstly try to create a new MBS session for the updated service area. or it may suggest the VAL server suggest using unicast PDU session to the VAL server when the new MBS session creation is still failed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UNKNOWN_MBS_SERVICE_AREA will not be well handled by the ASP |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.4A.3.X (new), 14.3.2.42, 14.3.2.43 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************** First Changes ********************/

##### 14.3.4A.3.x	MBS service area update failure handling

The MBS service area update to the ongoing MBS session may fail, and an error code "UNKNOWN_MBS_SERVICE_AREA" is returned from the core network as described in 3GPP TS 29.532 [xx] and 3GPP TS 29.532 [yy]. This may be caused due to the requested MBS service area are beyond the service area of the serving MB-SMF, or the MBS are not deployed at the updated MBS service area. Upon receiving this "UNKNOWN_MBS_SERVICE_AREA" error code, the NRM server may firstly try to create a new MBS session with the updated service area. If it still fails, the NRM server suggest using unicast PDU session to the VAL server in that area.

Pre-conditions:

-	The MBS session is created with certain service requirements and optionally with a certain broadcast/multicast service area.

Figure 14.3.4A.3.x-1: MBS service area update failure handling

1.	The NRM server determines to update service area of the existing MBS sesssion. This may be triggered by the MBS session update request with a new service area from the VAL server (step 1a), or the NRM server monitors the UE’s mobility and generates the new service area (step 1b) based on e.g., SEAL LMS mechanism.

2. The NRM servers performs the MBS session update with updated service area towards the 5GC as described in  3GPP TS 23.247 [39]. An application error "UNKNOWN_MBS_SERVICE_AREA" is returned as described in 3GPP TS 29.532 [xx] and 3GPP TS 29.532 [yy].

3.	Upon receiving the "UNKNOWN_MBS_SERVICE_AREA" error code, the NRM determines to create a new MBS session with the new service area obtained in step 1.

4.	The NRM server performs an MBS session creation procedure with the 5GC as described in 3GPP 23.247 [39].

4a.	If MBS session creation is successful, the new MBS session ID and new ingress address are returned. The procedure proceeds with step 5 to step 7.

4b.	If the MBS session creation is failed again, the procedure proceeds with step 7.

5.	The NRM server obtains the new MBS session ID and new ingress address of the new MBS session. The NRM server announces the new session information to the corresponding UEs. The UE may join the MBS session if it is multicast type. The UE may report the MBS listening status to the NRM server.

6.	 The NRM server returns the MBS session update response to the VAL server. The new MBS session ID and ingress address are included.

7.	The NRM server also indicates the VAL server to switch to the new MBS session. If the MBS session creation is failed in step 4, the NRM server indicates the VAL server to use unicast delivery.

# /******************** Next Changes ********************/

#### 14.3.2.42	Multicast/broadcast resource update request

Table 14.3.2.42-1 describes the information flow for the multicast/broadcast resource update request from VAL server to NRM server.

Table 14.3.2.42-1: Multicast/broadcast resource update request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| Identity of the multicast/broadcast resource | M | Identity of the multicast/broadcast resource to be updated |
| Service requirement description | O (NOTE 3) | Indicates the service characteristics, e.g., Media type, Media format, bandwidth requirements, flow description, Application Identifier, Priority indicator, emergency indicator Application service provider. |
| Multicast/Broadcast area information | O(NOTE 3) | Indicate the area where the multicast/broadcast resource is requested for |
| Local MBMS information (NOTE 1, NOTE 2) | O(NOTE 3) |  |
| > MB2-U information | O | IP address, UDP port number of the MB2-U interface |
| > xMB-U information | O | IP address, UDP port number of the xMB-U interface |
| > M1 interface information | O | M1 interface information for local MBMS |
| Local MBMS activation indication (NOTE 1, NOTE 2) | O | Indicates whether to request the NRM server to use Local MBMS information. |
| NOTE 1: The VAL server may provide either the Local MBMS information or the Local MBMS activation indication. This IE is present when the local MBMS is required for VAL services like V2X service. NOTE 2: This information element is only applicable when EPS is specified in the network system indicator IE. NOTE 3: At least one of those IEs shall be present. |  |  |

#### 14.3.2.43	Multicast/broadcast resource update response

Table 14.3.2.43-1 describes the information flow for the Multicast/broadcast resource update response from NRM server to VAL server.

Table 14.3.2.43-1: Multicast/broadcast resource update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of the multicast/broadcast resource update request operation. |
| Identity of the multicast/broadcast resource | M (NOTE 1) | The identity of the multicast/broadcast MBS session, or/and the identity of the MBMS bearer to be updated. |
| New Identity of the multicast/broadcast resource | O | The identity of the new multicast/broadcast MBS session created during update. |
| Session description information | O (NOTE 2) | Indicates Multicast/broadcast related configuration information as defined in 3GPP TS 26.346 [28] (e.g. radio frequency and MBMS Service Area Identities) or/and 3GPP TS 23.247 [39] (e.g. MBS FSA ID(s), Area Session ID, service requirements) |
| NOTE 1: This IE may not be required if the session announcement mode indicates that the request is sent by the NRM server. NOTE 2: If the Result Information element indicates failure then the values of the other information elements shall not present. |  |  |