3GPP TSG-SA WG6 Meeting #54-e	S6-231345

e-meeting, 17th – 26th April 2023	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0390 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating the MC service user criteria in the preconfigured user regroup |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Enh4MCPTT |  |  |  |  |  | Date: |  |  | 2023-04-17 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current version of preconfigured regroup requests, the aspects of defining the MC service user criteria are mentioned to be left for implementation. This however may create issues if the MC service client and MC service server is of different vendors. Therefore, this CR is updating the description of the MC service user criteria. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating the preconfigured regroup requests information element related to MC service user criteria. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There may be issues when deploying multivendor entities. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 10.15.2.1	Preconfigured regroup request (MC service client – MC service server)

Table 10.15.2.1-1 describes the information flow preconfigured regroup request from the MC service client to the MC service server.

Table 10.15.2.1-1 Preconfigured regroup request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requester |
| MC service group ID | O | MC service group ID of the regroup group |
| MC service group ID | M | MC service group ID of the MC service group from which configuration is to be taken |
| MC service group ID list | O (see NOTE) | List of MC service groups to be regrouped into the group regroup group |
| MC service user ID list | O (see NOTE) | List of MC service users to be regrouped into the user regroup group |
| MC service user criteria | O (see NOTE) | Carries the details of criteria which will be used by the MC service server for determining the participants. For example, it can be a location based criteria to determine the MC service user ID list in a particular area, or it could be tags (e.g. “fire”, “medical”, “police”, etc.), or a combination of tags and location.. |
| NOTE: One and only one of these shall be present. |  |  |

# * * * * second change * * * *

#### 10.15.2.2	Preconfigured regroup request (MC service server – MC service client)

Table 10.15.2.2-1 describes the information flow preconfigured regroup request from the MC service server to the MC service client.

Table 10.15.2.2-1 Preconfigured regroup request information elements

| Information Element | Status | Description |  |
| --- | --- | --- | --- |
| MC service ID | M | The MC service ID of the target MC service group member |  |
| MC service group ID | O | MC service group ID of the regroup group |  |
| MC service group ID | M | MC service group ID of the MC service group from which configuration is to be taken |  |
| MC service group ID list | O (see NOTE) | List of MC service groups to be regrouped into the group regroup group |  |
| MC service user ID list | O (see NOTE) | List of MC service users to be regrouped into the user regroup group |  |
| MC service user criteria | O (see NOTE) |  | Carries the details of criteria which will be used by the MC service server for determining the participants. For example, it can be a location based criteria to determine the MC service user ID list in a particular area, or it could be tags (e.g. “fire”, “medical”, “police”, etc.), or a combination of tags and location. |
| NOTE: One and only one of these shall be present. |  |  |  |

# * * * * Third change * * * *

#### 10.15.2.3	Preconfigured regroup request (MC service server – MC service server)

Table 10.15.2.3-1 describes the information flow preconfigured regroup request from the MC service server to the MC service server.

Table 10.15.2.3-1 Preconfigured regroup request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requester |
| MC service group ID | O | MC service group ID of the regroup group |
| MC service group ID | M | MC service group ID of the MC service group from which configuration is to be taken |
| MC service group ID list | O (see NOTE) | List of MC service groups to be regrouped into the group regroup group |
| MC service user ID list | O (see NOTE) | List of MC service users to be regrouped into the user regroup group |
| MC service user criteria | O (see NOTE) | Carries the details of criteria which will be used by the MC service server for determining the participants. For example, it can be a location based criteria to determine the MC service user ID list in a particular area, or it could be tags (e.g. “fire”, “medical”, “police”, etc.), or a combination of tags and location. |
| NOTE: One and only one of these shall be present. |  |  |

# * * * * End of changes * * * *