3GPP TSG-SA WG6 Meeting #54-e	S6-231352

e-meeting, 17th April – 26th April 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0318 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Replacing OP terminology and reusing existing procedures for ENS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-11 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Following Editor’s Notes: Editor's note: The terms "ECSP" and "OP" needs to be aligned in this clause. Editor's note: Reusing existing procedures and information flows (e.g., service provisioning or EAS discovery) for Edge Node Sharing is FFS, it is also FFS how to merge the procedures related with ECS-ER in this clause with 8.17 support of roaming and federation. Most of the proceudres specified for Edge Node Sharing are same as existing procedures specified for federation, service provisioning, EAS discovery and EES retrieval. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Duplicated procedures are removed and references to existing procedures is added: - Following clauses are mapped to 8.17.2.4 (Service provisioning information retrieval) since the purpose of these procedures is to obtain EES and EAS information from the partner ECS (which can also be an ECS-ER of the partner ECSP): 8.18.2. Application information sharing between leading OP and partner OP 8.18.2.1.1 request-response model 8.18.2.1.2 Subscribe notify model 8.18.2.2 Retrieval of EES of partner OP from Leading OP 8.18.2.3 Service provisioning with the published application information - Following clauses reuse the EAS discovery, Retrieve EES and T-EAS discovery clauses, text is updated accordingly. 8.18.2.4 EAS discovery via Leading ECSP 8.18.2.5 EAS discovery via Partner ECSP, EEC triggered 8.18.2.6 EAS discovery via Partner ECSP, EES triggered Also, OP terminology is replaced with EDGEAPP terminology e.g., OP is replaced with ECSP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Duplication of procedrues will lead to erroneous implementations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.18.1, 8.18.2, 8.18.2.1, 8.18.2.1.1, 8.18.2.1.2, 8.18.2.2, 8.18.2.2.1 (NEW), 8.18.2.3, 8.18.2.3.1 (NEW), 8.18.2.3.2 (NEW), 8.18.2.3.3 (NEW), 8.18.2.4, 8.18.2.5, 8.18.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AC	Application Client

ACID	Application Client Identification

ACR	Application Context Relocation

ACT	Application Context Transfer

AEF	API Exposing Function

AF	Application Function

APN	Access Point Name

ASP	Application Service Provider

CAPIF	Common API Framework

DN	Data Network

DNAI	Data Network Access Identifier

DNN	Data Network Name

H-ECS	Home Edge Configuration Server

H-EES	Home Edge Enabler Server

EAS	Edge Application Server

EASID	Edge Application Server Identification

ECI	Edge and Cloud Interworking

ECS	Edge Configuration Server

ECS-ER	Edge Configuration Server – Edge Repository

ECSP	Edge Computing Service Provider

EDN	Edge Data Network

EEC	Edge Enabler Client

EECID	Edge Enabler Client Identification

ETSI	European Telecommunications Standards Institute

EEL	Edge Enabler layer

EES	Edge Enabler Server

EESID	Edge Enabler Server Identification

ENS	Edge Node Sharing

FQDN	Fully Qualified Domain Name

GPSI	Generic Public Subscription Identifier

GSM	Global System for Mobile Communications

GSMA	GSM Association

LADN	Local Area Data Network

MEC	Multi-access Edge Computing

NEF	Network Exposure Function

OP	Operator Platform

OPG	Operator Platform Group

PCF	Policy Control Function

PSA	PDU Session Anchor

S-EAS	Source Edge Application Server

S-EES	Source Edge Enabler Server

SCEF	Service Capability Exposure Function

SEAL	Service Enabler Layer Architecture

SMF	Session Management Function

SSID	Service Set Identifier

T-EAS	Target Edge Application Server

T-EES	Target Edge Enabler Server

TAI	Tracking Area Identity

URSP	UE Route Selection Policy

V-ECS	Visited Edge Configuration Server

V-EES	Visited Edge Enabler Server

* * * Next Change * * * *

## 8.18	Edge Node Sharing

### 8.18.1	General

In the following clauses, ECSP-A is an ECSP partner of ECSP-B and ECSP-B is the leading ECSP which is serving the UE.

### 8.18.2	Procedures

#### 8.18.2.1	General

Following procedures are defined to support ENS:

-	Application information sharing between ECSPs; and

-	EAS discovery for ENS.

#### 8.18.2.2	Application information sharing between ECSPs

Editor's note: The security aspect between ECS-ER (ECSP-B) and ECS-ER (ECSP-A) is FFS and needs to be studied by SA3.

Editor's note: whether more procedures & operations are needed between ECS-ERs is FFS.

##### 8.18.2.2.1	General

To exchange EES and application information between ECSPs for the purpose of ENS, the procedures defined in clause 8.17 are used.

Upon receiving a service provisioning request from the EEC as specified in clause 8.3, if the ECS determines to query a partner ECSP for the purpose of ENS, the ECS uses -

-	the procedures defined in clause 8.17.2.3, if it needs to obtain or subscribe for ECS information of partner ECSP; and

-	the procedures defined in clause 8.17.2.4, to obtain or subscribe for the EES information from the partner ECS.

To obtain EES and application information using

-	a request-response model the procedures defined in clause 8.17.2.4.2.2.

-	a subscribe-notify model the procedures defined in clause 8.17.2.4.2.3.

The service provisioning response to the EEC includes EES information of the partner ECSP along with the EES information of the ECSP serving the UE. The EES of the ECSP serving the UE is determined based on SLA(s) between the ECSPs such that it can be authorized by the EES of the partner ECSP. The endpoint of EES of partner ECSP is provided by the ECS in a way that is transparent to EEC.

#### 8.18.2.3	EAS discovery for ENS

##### 8.18.2.3.1	General

Following procedures are defined for EAS discovery with respect to ENS:

-	EAS discovery via leading ECSP;

-	EEC triggered EAS discovery via partner ECSP; and

-	EES triggered EAS discovery via partner ECSP.

##### 8.18.2.3.2	EAS discovery via leading ECSP

Pre-conditions:

1.	ECSP-1 and ECSP-2 have a service level agreement to share edge services.

Figure 8.18.2.3.2-1: EAS discovery for ENS

1.	The EEC sends EAS discovery request to EES (ECSP-B) as specified in clause 8.5 and may include the endpoint of EES (ECSP-A) in the request, if provided by the ECS as specified in clause 8.18.2.2.1.

2.	If the EAS discovery request contains EES information of a partner ECSP (ECSP-A) or 	the required EAS is not available with the EES the EES may determine to use ENS. If the EAS discovery request contains EES information of a partner ECSP, the EES can use it after validating the information.

NOTE 1:	To validate the information of partner ECSPs, EES (ECSP-B) can use the federation agreement between ECSPs as described in Annex B.

3.	If the EAS discovery request does not contain EES information of a partner ECSP, the EES may use the EES retrieval procedures as specified in clause 8.8.3.3 to obtain EES information of a partner ECSP. The Retrieve EES request may include an ENS indication so that the ECS (ECSP-B) can skip checking T-EES(s) locally registered in the ECS (ECSP-B).

4.	Once EES information of the partner ECSP is available, the EES (ECSPB) sends the EAS discovery request to the EES (ECSP-A) as described in step 3 of clause 8.8.3.2. The EAS discovery request includes the information of the MNO serving the UE (e.g. MNO name, PLMN ID).

The EES (ECSPA) validates the request and returns EAS discovery response including the discovered candidate EAS(s) to the EES (ECSP-B). In addition to step 4 of clause 8.8.3.2, the candidate EAS(s) are discovered based on the serving MNO information received from the EES (ECSP-B) and allowed MNO information registered by the EAS.

NOTE 2:	To validate the EAS discovery request, EES (ECSP-A) can use the federation agreement between ECSPs as described in Annex B.

5.	EES (ECSP-B) provides the discovered information to the EEC as part of the EAS discovery response as specified in clause 8.5.

##### 8.18.2.3.3	EAS discovery via Partner ECSP, EEC triggered

If the EEC has the EES information of the partner ECSP and the required security credentials to communicate with the partner ECSP, the EEC can use the EAS discovery procedures specified in clause 8.5 to directly obtain the EAS information from the EES of the partner ECSP.

##### 8.18.2.3.4	EAS discovery via Partner ECSP, EES triggered

When service continuity is required, the EES can trigger the T-EAS discovery procedures as defined in clause 8.8.3.2 with the EES of the partner ECSP to find an EAS with respect to ENS. The EAS discovery request includes UE's serving MNO information (e.g. MNO name, PLMN ID).

Editor's note: ACR considering all scenarios are FFS.