3GPP TSG-SA WG6 Meeting #54-e	S6-231363

17th – 26th April 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0328 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | More procedures with CES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In case CES is deployed, it can be used to support service exposure to CAS and facilitate EEC context relocation. The CAS discovery can use traditional DNS in case of no ENS. For ENS, given the GSMA OPG reply in S6-2223088 “OP B is serving the end users of Operators B regardless of the region where they are”, this CR also offers a solution with CES facilitation. In case of ENS and there is no EES deployed everywhere in the whole PLMN, the CES endpoint (which is pre-configured in ECS) can be given to EEC. This also implies that the CES can be part of OP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the architecture figure with reference point change. Update reference point explanation. Update ACR scenario with CES. Update ENS scenario with CES. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No procedure support for ACR btw. CAS and EAS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2d, 6.5.16, 6.5.17, 6.5.18, 6.5.19, 6.5.20 (new), 8.8.2B.1, 8.8.2B.2 (new), 8.18.1, 8.18.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 6.2d	Architecture for enabling cloud applications with edge applications, with CES support

Figure 6.2d-1 illustrates the architecture for enabling cloud applications along with the edge applications, when CES is used.

NOTE:	Edge and cloud servers can utilize SEAL NM service but for simplicity such an interaction is not depicted in the figure.

Figure 6.2d-1: Architecture for enabling cloud application with edge applications

Cloud Application Server (CAS) residing in the clould DN may need to interact with the Cloud Enabler Server (CES) for interworking, e.g. for service continuity. The CAS and EAS interaction is Application Data Traffic, which is out-of-scope of this specification.

* * * Next Change * * * *

### 6.5.16	ECI-3

ECI-3 enables interaction between CES and ECS.

ECI-3 supports:

a)	EES retrieval procedure in ENS.

### 6.5.17	ECI-4

ECI-4 enables interaction between CES and EES.

ECI-4 supports:

a)	EEC context relocation procedures.

b)	EAS discovery procedure in ENS.

### 6.5.18	CLOUD-1

CLOUD-1 enables interaction between EEC and CES.

CLOUD-1 supports:

a)	EAS discovery procedure in ENS.

### 6.5.19	CLOUD-2

CLOUD-2 enables interaction between CES and CES.

CLOUD-2 supports:

a)	EEC context relocation procedures.

### 6.5.20	CLOUD-3

CLOUD-1 enables interaction between CAS and CES.

CLOUD-1 supports:

a)	providing access to network capability information (e.g. location information);

b)	requesting the setup of a data session between AC and CAS with a specific QoS and receiving QoS related information; and

c)	service continuity procedures (e.g. ACR status).

* * * Next Change * * * *

#### 8.8.2B.1	General

Since the EAS may have service area restriction, once the UE is moving out of the current edge coverage, to keep service continuity, the application client needs to communicate with the CAS.

CAS(s) are registered in the CES via CLOUD-1 reference point which enables the CES to provide appropriate CAS to S-EAS.

Editor's Note:	Need for CAS registration with CES needs to be confirmed.

DNS can be used in CAS discovery by the EEC or T-CAS discovery by the CAS.

The EES of the source EDN may interact with the CES via ECI-4 reference point and application context is transferred from source EAS to the CAS. Later, if the UE is moving to an area with edge coverage, the CES may interact with the EES in the target EDN via ECI-4 reference point and application context is transferred from the CAS to target EAS.

* * * Next Change * * * *

#### 8.8.2B.2	Enabling ACR with CES – Initiation by EEC using regular EAS Discovery

The scenario described in clause 8.8.2A.2 applies with the following differences:

-	After step 7, the S-EES relocates context to the CES. The relocation procedure re-uses the procedure describes in clause 8.9.2.3.

-	During post-ACR clean up, the CAS triggers ACR status update towards the CES in order to finish ACR.

* * * Next Change * * * *

### 8.18.1	General

In the following clauses, the OP-A is a partner of OP-B and OP-B is the leading OP to serve the UE.

Editor's note: The terms "ECSP" and "OP" needs to be aligned in this clause.

The information and procedure of service provisioning and EAS discovery described in the following clauses are same as clause 8.3.3 and clause 8.5, only differences are described in this clause for edge node sharing.

Editor's note: Reusing existing procedures and information flows (e.g., service provisioning or EAS discovery) for Edge Node Sharing is FFS, it is also FFS how to merge the procedures related with ECS-ER in this clause with 8.17 support of roaming and federation.

The leading ECSP can deploy its EESs in EDNs providing full edge coverage for the UE in the whole PLMN. If the leading ECSP has CES deployed and partial edge coverage in the whole PLMN, the leading ECSP can rely on CES to provide edge service facilitation in ENS. The ECS of the leading ECSP determines whether to provide EES or CES of the leading ECSP to the UE, depending on whether the UE can be served by EES or CES of the leading ECSP.

* * * Next Change * * * *

#### 8.18.2.4	EAS discovery via Leading ECSP

Pre-conditions:

1.	ECSP-1 and ECSP-2 have a service level agreement to share edge services.

2.	EES (ECSP-B) has received EAS discovery request where required EAS is not available with the EES (ECSP-B).

Figure 8.18.2.4-1: EAS discovery for edge node sharing

1.	The EEC sends EAS discovery request to EES (OP-B) and may include the endpoint of EES (OP-A) in the request.

2.	The EES (OP-B) validates the edge service SLA. If EES (OP-A) endpoint is included, the EES (OP-B) skips step 3-5; otherwise, the EES (OP-B) executes for EAS discovery but cannot find any desired EAS so it determines to use edge node sharing service based on edge service SLA.

3.	The EES (OP-B) sends Retrieve EES request to the ECS (OP-B). The request may include an edge node sharing indication indicating edge node sharing is requested so that the ECS (OP-B) can skip checking T-EES(s) registered in the ECS (OP-B).

NOTE:	If EES (OP-A) information is not carried in EAS discovery request in step 1, the EES (OP-B) can determine to retrieve EES (OP-A) information from ECS (OP-B) as in step 3-5.

4.	For the received EAS ID, the ECS (OP-B) discovers T-EES (i.e. EES(s) from partner).

5.	The ECS (OP-B) returns discovered EES(s) of OP-A in the Retrieve EES response to the ECS (OP-B).

6.	The EES (OP-B) sends EAS discovery request to the EES (OP-A). The EAS discovery request includes the serving MNO information (e.g. MNO name, PLMN ID) which is serving the UE.

7.	The EES (OP-A) validates the edge service SLA and returns EAS discovery response including the discovered candidate EAS(s) to the EES (OP-B). The candidate EAS(s) are discovered based on the serving MNO information received from the EES (OP-B) and allowed MNO information registered by the EAS.

8.	The EEC receives the EAS discovery response sent by the EES (OP-B). The EEC may select an EAS for the AC.

Editor's note:	It is FFS whether to add the interaction between EES (OP-B) and ECS (OP-B) for T-EES retrieval after step 2.

During service provisioning, if the ECS (ECSP-B) returns CES (ECSP-B) to the EEC, the procedure is same as figure 8.18.2.4-1 with the difference that the CES (ECSP-B) replaces EES (ECSP-B).

* * * END of Change * * * *