3GPP TSG-SA WG6 Meeting #54-e	S6-231390

17th – 26th April 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0338 | rev | 0 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ACR with CAS - S-EES executed ACR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Service continuity for ACs in the UE require minimum service interruption while switching the application server between Edge and Cloud. To support service continuity, the application context is transferred between Edge Application Server (EAS) and Cloud Application Server (CAS). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Scenario for Enabling ACR with cloud applications - S-EES executed ACR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Scenario details of switching from application server between Edge and Cloud will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.2A.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Key Issue#11, Solution#25 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.8.2A.5	Enabling ACR with CAS - S-EES executed ACR

Figure 8.8.2A.5-1 illustrates the S-EES detecting, deciding and executing ACR from the S-EAS to the CAS. This may include EELManagedACR by S-EES when initiated by S-EAS as per clause 8.8.3.6.

This scenario description is same as described for figure 8.8.2.5-1 except for the following clarifications:

Pre-conditions: Same pre-conditions apply described for figure 8.8.2.5-1, with the clarification that T-EAS is CAS.

Figure 8.8.2A.5-1: Enabling ACR with CAS - S-EES executed ACR

1.	Same as step 1 described for figure 8.8.2.5-1.

Phase I: ACR Detection

2.	Same as step 2 described for figure 8.8.2.5-1.

Phase II: ACR Decision

3.	 Same as step 3 described for figure 8.8.2.5-1.

4	 Same as step 4 described for figure 8.8.2.5-1.

Phase III:	ACR Execution

5.	The S-EES determines the targets via the Discover T-EAS procedure in clause 8.8.3.2. When S-EES determines that no relevant EAS is available for the UE's location it finds out the details of the CAS, e.g. via DNS query/discovery.

6.	The S-EES sends the target information notification to the EEC as described in clause 8.8.3.5.3.

7.	The S-EES may apply the AF traffic influence with the N6 routing information of the CAS in the 3GPP Core Network (if applicable).

8.	The S-EES sends the ACR management notification (e.g. as notification for "ACR facilitation" event or "ACT start" event as described in clause 8.6.3 or due to step 1) to the S-EAS to initiate ACT between the S-EAS and the CAS.

9.	The Application Context is transferred from S-EAS to the CAS at implementation specific time. In the case of EELManagedACR, the S-EES accesses the Application Context from the address as per step 1 and the S-EES either engage in the ACT from S-EAS to the CAS (obtained as per step 5) in a secure way or S-EES shares the storage location of the Application Context with the CAS. Further the CAS accesses the Application Context. The S-EAS may also perform the ACT directly with CAS, the specification of such process is out of scope of the present document.

NOTE 1:	The Application Context is encrypted and protected by the application layer. The S-EES engages in the packet level transport of the Application Context and has no visibility to the content of the Application Context.

NOTE 2:	The S-EAS or CAS can further decide to terminate the ACR, and the CAS can discard the application context based on information received from EEL and/or other methods (e.g. monitoring the location of the UE). It is up to the implementation of the S-EAS and CAS whether and how to make such a decision.

Phase IV:	Post-ACR Clean up

10.	Same as step 12 described for figure 8.8.2.2-1.

11.	If the status in step 10 indicates a successful ACT, the S-EES sends the ACR information notification (ACR complete) message to the EEC to confirm that the ACR has completed as specified in clause 8.8.3.5.3 of TS 23.558.

NOTE 3:	The Application Client mechanism to support switchover of the application traffic to CAS is out of scope of the specification.

* * * End of Changes * * * *