3GPP TSG-SA WG6 Meeting #54-e	S6-231391

17th – 26th April 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0339 | rev | 0 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Common EAS - minimalist solution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The group of UEs may need to connect to the same EAS for better experience of application services and reducing the interaction between EASs when the group of UEs connect to different EASs. Thus the service provisioning and the EAS discovery procedure should be enhanced for the common EAS discovery. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The service provisioning procedure and EAS discovery procedure is enhanced for the common EAS discovery |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Common EAS discovery requirements will remain unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2, 8.2.4, 8.2.6, 8.3.3.2.2, 8.3.3.3.3, 8.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Key Issue#17, New solution |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.2.2	AC Profile

An AC Profile includes information about AC used to determine services and service characteristics required.

Table 8.2.2-1: AC Profile

| Information element | Status | Description |
| --- | --- | --- |
| ACID | M | Identity of the AC. |
| AC Type | O | The category or type of AC (e.g. V2X). This is an implementation specific value. |
| Preferred ECSP list | O | When used in a service provisioning request, this IE indicates to the ECS which ECSPs are preferred for the AC. The ECS may use this information in the selection of EESs. |
| AC Schedule | O | The expected operation schedule of the AC (e.g. time windows) |
| Expected AC Geographical Service Area | O | The expected location(s) (e.g. route) of the hosting UE during the AC's operation schedule. This geographic information can express a geographic point, polygon, route, signalling map, or waypoint set. |
| AC Service Continuity Support | O | Indicates if service continuity support is required or not for the application. The IE indicates which ACR scenarios are supported by the AC and which of these are preferred by the AC, also indicates the AC ability of handling bundled EAS ACR. |
| Simultaneous EAS connectivity information in service continuity | O | Indicates if simultaneous EAS connectivity is needed and the inactive time guidance for keeping connectivity towards the S-EAS. |
| List of EASs | O | List of EAS that serve the AC along with the service KPIs required by the AC |
| > EASID | M | Identifier of the EAS |
| > Expected AC Service KPIs | O | KPIs expected in order for ACs to receive currently required services from the EAS, as described in Table 8.2.3-1 |
| > Minimum required AC Service KPIs | O | Minimum KPIs required in order for ACs to receive meaningful services from the EAS, as described in Table 8.2.3-1 |
| List of required EAS bundle information | O | Information related to the EAS bundles which AC requires. |
| > Bundle ID or list of EASID | M | A list of EASIDs or a bundle ID as described in clause 7.2.10. |
| > EAS bundle requirements | O | Requirements associated with the EAS bundle as described in clause 8.2.10. |
| Application groups information | O | Application group to which the common EAS is desired. |
| > Application Group IDs | M | Identity of groups for which the UE is member of. |

Editor's Note: It is FFS whether and how EAS endpoint information may be included in the AC Profile.

* * * Next Change * * * *

### 8.2.4	EAS Profile

Table 8.2.4-1: EAS Profile

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS. |
| List of EAS bundle information | O | List of EAS bundles to which the EAS belongs and related bundling requirements. |
| > Bundle ID | M | Bundle ID as described in clause 7.2.10. |
| List of application groups information | O | List of application groups to which the EAS is selected as common EAS. Ensuring only one common EAS is selected per group is out of scope of this specification. |
| > Application Group ID | M | Identity of a group of UEs using the same application. |
| ACID(s) | O | Identifies the AC(s) that can be served by the EAS |
| EAS Provider Identifier | O | The identifier of the ASP that provides the EAS. |
| Allowed MNO information | O | Information of the allowed operator (e.g. MNO name, PLMN ID) from which its subscriber can consume the EAS services |
| EAS Type | O | The category or type of EAS (e.g. V2X) |
| EAS description | O | Human-readable description of the EAS |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) |
| EAS Geographical Service Area | O | The geographical service area that the EAS serves. ACs in UEs that are located outside that area shall not be served. |
| EAS Topological Service Area | O | The EAS serves UEs that are connected to the Core Network from one of the cells included in this service area. ACs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table 8.2.5-1 |
| EAS service permission level | O | Level of service permissions e.g. trial, gold-class supported by the EAS |
| EAS Feature(s) | O | Service features e.g. single vs. multi-player gaming service supported by the EAS |
| EAS Service continuity support | O | Indicates if the EAS supports service continuity or not. This IE indicates which ACR scenarios are supported by the EAS, also indicates the EAS ability of handling bundled EAS ACR. |
| EAS Transport layer service continuity support | O | This IE indicates the EAS service continuity support for seamless transport layer (e.g. TCP/TLS/QUIC) relocation |
| General context holding time duration (NOTE) | O | The time duration that the EAS holds the context before the AC connects to the EAS in case of ACR for service continuity planning. It is an indication of the time the EAS holds the application context for a UE to move to its service area after receiving an ACR notification from the EES following an ACR request from the EEC. |
| List of EAS DNAI(s) | O | DNAI(s) associated with the EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN where the EAS resides. |
| List of N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to each EAS DNAI. |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heartbeat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, etc.) |
| NOTE: Since the EASID of the EAS identifies the type of the application (e.g. SA6Video, SA6Game etc) as described in clause 7.2.4, "General context holding time duration" determined by EAS can depend on the EASID (type of the application). |  |  |

Editor's Note:	Whether EAS bundle requirement is needed in EAS profile or EAS discovery filters is FFS.

NOTE:	The EAS Transport layer service continuity support can be used in EAS discovery, e.g. as described in 3GPP TS 23.433 [26] for SEALDD server acting as EAS, which can further support the EAS IP replacement function.

* * * Next Change * * * *

### 8.2.6	EES Profile

The EES profile includes information about the EES and the services it provides.

Table 8.2.6-1: EES Profile

| Information element | Status | Description |
| --- | --- | --- |
| EESID | M | The identifier of the EES |
| EES Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EES. This information is provided to the EEC to connect to the EES. |
| EASIDs | M | List of EASIDs registered or expected to be registered with the EES. |
| > List of EAS bundle information | O | List of EAS bundles to which the EAS belongs and related bundling requirements. |
| >> Bundle ID | M | A bundle ID as described in clause 7.2.10. |
| List of application groups information | O | List of application groups to which the EAS is selected as common EAS as per the information received in the EAS profile. |
| > Application Group ID | M | Identity of a group of UEs using the same application. |
| Instantiable EAS information | O | The EAS instantiation status per EASID (e.g. instantiated, instantiable but not be instantiated yet). |
| > Instantiation criteria (see NOTE) | O | The criteria upon which EAS can be instantiated (e.g. based on specific date and time). |
| EEC registration configuration | M | Indicates whether the EEC is required to register on the EES to use edge services or not. |
| ECSP ID | O | The identifier of the ECSP that provides the EES. |
| EES Topological Service Area | O | The EES serves UEs that are connected to the Core Network from one of the cells included in this service area. EECs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| List of EES DNAI(s) | O | DNAI(s) associated with the EES. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN, where the EES resides. |
| EES Service continuity support | O | Indicates if the EES supports service continuity or not. This IE indicates which ACR scenarios are supported by the EES, also indicates the EES ability of handling bundled EAS ACR. |
| NOTE: "Instantiation criteria" IE shall be present only when the value of "Instantiable EAS information" IE is "instantiable but not be instantiated yet". |  |  |

NOTE:	The list of EES DNAI(s) can include the DNAI(s) of the EAS(s) registered with the EES.

Editor's Note:	Whether EAS bundle requirement is needed in EES profile is FFS.

* * * Next Change * * * *

##### 8.3.3.2.2	Request-response model

Figure 8.3.3.2.2-1 illustrates service provisioning procedure based on request/response model.

Pre-conditions:

1.	The EEC has been pre-configured or has discovered the address (e.g. URI) of the ECS;

2.	The EEC has been authorized to communicate with the ECS;

3.	The UE Identifier is either preconfigured or resulted from a successful authorization; and

4.	The ECS is configured with ECSP's policy for service provisioning.

NOTE 1:	Details of ECSP's policy are out of scope.

Figure 8.3.3.2.2-1: Service provisioning – Request/Response

1.	The EEC sends a service provisioning request to the ECS. The service provisioning request includes the security credentials of the EEC received during EEC authorization procedure and may include the UE identifier such as GPSI, connectivity information, UE location, EEC service continuity support and AC profile(s) information. EEC may provide its desired ECSP identifier(s) in the service provisioning request based on EEC preference. For obtaining the common EAS information from the ECS, the request may include Application groups information i.e. Application Group ID.

2.	Upon receiving the request, the ECS performs an authorization check to verify whether the EEC has authorization to perform the operation. The ECS may utilize the capabilities (e.g. UE location) of the 3GPP core network as specified in clause 8.10.2. If the UE serving PLMN identifier is not provided by the EEC in the connectivity information of the service provisioning request, the ECS may invoke the NEF monitoring event API as described in 3GPP TS 23.502 [43] and 3GPP TS 23.682 [17] to obtain the UE roaming status and serving PLMN identifier. If the UE is roaming, the ECS may use the serving PLMN identifier to determine the roaming partner ECS (i.e. V-ECS) information to be provided to the EEC in the service provisioning response. If AC profile(s) are provided by the EEC, the ECS identifies the EES(s) based on the provided AC profile(s) and the UE location. When AC profiles(s) are not provided, then:

-	if available, the ECS identifies the EES(s) based on the UE-specific service information at the ECS and the UE location;

-	ECS identifies the EES(s) by applying the ECSP policy (e.g. based only on the UE location);

NOTE 2:	Details of the UE-specific service information and how it is available at the ECS is out of scope.

NOTE 3:	Both steps are evaluated prior to sending a response.

If desired ECSP identifier(s) is received, the ECS identifies the EES(s) based on registered ECSP identifier in EES profile.

NOTE 4:	For EEC desired ECSP identifier usage, it is assumed that the ECSP providing the EES and PLMN operator are the same organization and an ECSP providing the EES (desired by the EEC) registers its EES in ECS provided by another ECSP based on service agreement to provide services to EEC.

The ECS also determines other information that needs to be provisioned, e.g. identification of the EDN, EDN service area, EES endpoints.

When the Application groups information (i.e. Application Group ID) is provided in the request, then the ECS identifies the EAS associated to the same Application Group ID, as received in the EES profile.

If ECS does not identify any suitable EES(s) based on EDN configuration available at the ECS and UE's location, the ECS determines a partner ECS that may satisfy the requirements. Based on ECSP policy, the ECS may use preconfigured or OAM configured information about the partner ECSs or ECS discovery via ECS-ER as specified in clause 8.17.2.3 or both.

If required by the ECSP policies, the ECS may use service provisioning information retrieval procedure as specified in clause 8.17.2.4 to obtain seivice provisioning information from the partner ECS.

When the bundle EAS information is provided, then;

-	If bundle EAS information includes EAS bundle identifier, the ECS identifies all the EES(s) providing the same EAS bundle identifier.

-	If bundle EAS information includes a list of EASIDs, the ECS identifies the one or more EES which support all of the EASs within the same DNAI based on the EES DNAI information obtained in the EES profile.

3.	If the processing of the request was successful, the ECS responds to the EEC's request with a service provisioning response. If the ECS has identified the relevant EES(s) information, the service provisioning response includes a list of EDN configuration information, e.g. identification of the EDN, EDN service area, and the required information (e.g. URI, IP address) for establishing a connection to the EES.

When the Application groups information (i.e. Application Group ID) is provided, the ECS returns only one EES (and hence EAS) associated to each of the application group ID received in the request.

The ECS may provide associated EES(s) information (one ore more EES information) in the service provisioning response along with the bundle EAS information.

If the alternative ECS(s) has been identified in step 2, the ECS sends a successful response including the Redirect information element containing the list of ECS(s) configuration information indicating that alternative ECS(s) is available for service provisioning request. The response may include information such as DNN and S-NSSAI for roaming UEs to establish a PDU session with the ECS as specified in 3GPP TS 23.548 [20].

If the ECS is not provisioned with any EDN configuration information or is unable to determine either the EES information or the partner ECS information using the inputs in service provisioning request, UE-specific service information at the ECS or the ECSP's policy, the ECS shall reject the service provisioning request and respond with an appropriate failure cause.

If the service provisioning response contains a list of ECS configuration information, the EEC may initiate service provisioning procedure with one or more ECS(s) provided in the response. If the UE is roaming to a V-PLMN and the ECS configuration information includes V-PLMN ID in the list of Supported PLMN ID(s), the EEC establishes a connection with the V-ECS as specified in 3GPP TS 23.548 [20]. The connection with the V-ECS can be a HR-SBO PDU session or an LBO PDU session based on the information received from the ECS.

If the EDN configuration information includes an LADN DNN as an identifier for the EDN, the EEC considers the LADN as the EDN. Therefore, the service area of EDN is the LADN Service Area which can be discovered using the UE Registration Procedure.

The EEC may cache the service provisioning information (e.g. EES endpoint) for subsequent use and avoid the need to repeat step 1. If the Lifetime IE is included in the Service provisioning response, then the EEC may cache and reuse the Service provisioning information only for the duration specified by the Lifetime IE, without the need to repeat step 1.

If the ECS provided information regarding the service continuity support of individual EESs, the EEC may take this information into account when selecting an EES for EEC registration, EAS discovery or T-EAS discovery, respectively.

If for multiple EES(s), the instantiable EAS information IE for an EAS is not available or the instantiable EAS information IE is set to instantiated or instantiable, the EEC can select one or more such EES to perform EAS discovery. For EAS discovery to mitigate the waste of EDN resources EEC considers the instantiable EAS information and the associated instantiation criteria, the EEC selects one EES, if the EAS instantiation status corresponding to the EASID requested by AC/EEC is instantiable but not yet instantiated (i.e., no instantiated EAS).

NOTE 5:	If the service provisioning request fails, the EEC can resend the service provisioning request again, taking into account the received failure cause.

NOTE 6:	Even after the EEC establishes a connection to the EES using information received in step 3, the EES can issue AF request to influence traffic routing from EEC to EES as specified in 3GPP TS 23.501 [2] clause 5.6.7.

NOTE 7:	If the EAS instantiation fails based on the selected EES, the EEC may retry the EAS discovery request to another EES (e.g. selecting another one EES based on the instantiable EAS information).

* * * Next Change * * * *

##### 8.3.3.3.3	Service provisioning response

Table 8.3.3.3.3-1 describes the information elements for service provisioning response from the ECS to the EEC.

Table 8.3.3.3.3-1: Service provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the service provisioning request was successful. |
| > List of EDN configuration information | M | List of EDN configuration information as defined in Table 8.3.3.3.3-2. |
| Failure response | O | Indicates that the service provisioning request failed. |
| > Cause | O | Indicates the cause of service provisioning request failure. |
| Redirect | O | Indicates redirection to (an)other ECS(s). |
| > ECS(s) information | M | Endpoint address of ECS(s) to which the UE is redirected for service provisioning. |
| > DNN | O | DNN required for establishing PDU Session to the redirected ECS |
| > S-NSSAI | O | S-NSSAI required for establishing PDU Session to the redirected ECS |

Table 8.3.3.3.3-2: EDN configuration information

| Information element | Status | Description |
| --- | --- | --- |
| EDN connection information (see NOTE 1) | M | Information required by the UE to establish connection with the EDN. |
| > DNN/APN | M | Data Network Name/Access Point Name |
| > S-NSSAI | O | Network Slice information |
| > EDN Topological Service Area | O | The EDN serves UEs that are connected to the Core Network from one of the cells included in this service area. See possible formats in Table 8.2.7-1. |
| List of EESs | M | List of EESs of the EDN. |
| > EESID | M | The identifier of the EES |
| > EES Endpoint | M | The endpoint address (e.g. URI, IP address) of the EES |
| > EASIDs (see NOTE 2) | O | List of EASID registered or expected to be registered with the EES. |
| >> List of EAS bundle information | O | List of EAS bundles to which the EAS belongs and related bundling requirements. |
| >>> Bundle ID or list of EASID | M | A list of EASIDs or a bundle ID as described in clause 7.2.10. |
| >> List of application groups information | O | List of application groups to which the EAS is selected as common EAS. |
| >>> Application Group ID | M | Identity of a group of UEs using the same application. |
| > Instantiable EAS information | O | The EAS instantiation status per EASID (e.g. instantiated, instantiable but not be instantiated yet) |
| >> Instantiation criteria (see NOTE 3) | O | The criteria upon which EAS can be instantiated (e.g. based on specific date and time). |
| > ECSP ID | O | The identifier of the ECSP that provides the EES. |
| > EES Topological Service Area | O | The EES serves UEs that are connected to the Core Network from one of the cells included in this service area. EECs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| > EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| > List of EES DNAI(s) | O | DNAI(s) associated with the EES/EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. |
| > EES Service continuity support | O | Indicates if the EES supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EES. |
| > EEC registration configuration | M | Indicates whether the EEC is required to register on the EES to use edge services or not. |
| > Security Credential | O | Indicates the security credential sent by the ECS. The security credential is used by EEC to communicate with the EES as specified in 3GPP TS 33.558 [23], clause 6.2. |
| Lifetime | O | Time duration for which the EDN configuration information is valid and supposed to be cached in the EEC. |
| NOTE 1: If the UE is provisioned or pre-configured with URSP rules by the HPLMN or serving SNPN, the UE handles the precedence between EDN connection info and URSP rules as defined in 3GPP TS 23.503 [12] clause 6.1.2.2.1. EDN connection info is considered to be part of UE Local Configurations. NOTE 2: EAS information is limited to the EEC requested applications. If no AC profiles were present in the service provisioning request, the EAS information is subject to the ECSP policy (e.g. no EAS information or a subset of EAS information related to the EES). NOTE 3: "Instantiation criteria" IE shall be present only when the value of "Instantiable EAS information" IE is "instantiable but not be instantiated yet". |  |  |

* * * Next Change * * * *

#### 8.5.2.2	Request-response model

Pre-conditions:

1.	The EEC has received information (e.g. URI, IP address) related to the EES;

2.	The EEC has received appropriate security credentials authorizing it to communicate with the EES as specified in clause 8.11; and

3.	The EES is configured with ECSP's policy for EAS discovery.

NOTE 1:	Details of ECSP's policy are out of scope.

Figure 8.5.2.2-1: EAS Discovery procedure

1.	The EEC sends an EAS discovery request to the EES. The EAS discovery request includes the requestor identifier [EECID] along with the security credentials and may include EAS discovery filters and may also include UE location to retrieve information about particular EAS(s) or a category of EASs, e.g. gaming applications, or Edge Applications Server(s) available in certain service areas, e.g. available on a UE's predicted or expected route. The request may include an EAS selection request indicator or UE type. For discovering the common EAS information from the EES, the request may include Application groups information i.e. Application Group ID.

2.	Upon receiving the request from the EEC, the EES checks if the EEC is authorized to discover the requested EAS(s). The authorization check may apply to an individual EAS, a category of EASs or to the EDN, i.e. to all the EASs. If UE's location information is not already available, the EES obtains the UE location by utilizing the capabilities of the 3GPP core network as specified in clause 8.10.3. If EAS discovery filters are provided by the EEC, the EES identifies the EAS(s) based on the provided EAS discovery filters and the UE location.

When the bundle EAS information is provided, then;

-	If bundle EAS information includes EAS bundle identifier, the EES identifies all or part of the EAS(s) associated the same EAS bundle identifier.

-	If bundle EAS information includes a list of EASIDs, the EES identifies the EASs which are all or part of the EAS bundle.

When the Application groups information (i.e. Application Group ID) is provided in the request, then the EES identifies the EAS associated to the same Application Group ID, as received in the EAS registration.

If the EEC indicates that service continuity support is required, the EES shall take the indication which ACR scenarios are supported by the AC and the EEC and which of these are preferred by the AC into consideration. The EES may select one EAS and determine whether to perform application traffic influence for this AC based on AC's service KPI or EAS’s service KPI in desired response time, when the EAS does not perform traffic influence in advance.

When EAS discovery filters are not provided, then:

-	if available, the EES identifies the EAS(s) based on the UE-specific service information at the EES and the UE location;

-	EES identifies the EAS(s) by applying the ECSP policy (e.g. based only on the UE location);

NOTE 2:	Details of the UE-specific service information and how it is available at the EES is out of scope.

NOTE 3:	Both steps are evaluated prior to sending a response.

Upon receiving the request from the EEC, if the EEC does not indicate EAS Instantiation Triggering Suppress in the EAS Discovery request, the EES may trigger the EAS management system to instantiate the EAS that matches with EAS discovery filter IEs (e.g. ACID) as in clause 8.12.

Otherwise, upon receiving the request from the EEC, if the EEC indicates EAS Instantiation Triggering Suppress in the EAS Discovery request and the EES supports such capability, the EES may determine Instantiable EAS Information for EAS(s) that are instantiable but not yet instantiated and match the EAS discovery filter IEs. Instantiable EAS Information is provided in the EAS Discovery response and includes the EASID(s) and, for each EASID, the status indicating wether the EAS is instantiated or instantiable but not yet instantiated.

If the EEC provides in the EAS discovery request the EAS selection request indicator, the EES selects EAS satisfying the EAS discovery filter or based on other information (e.g. ECSP policy) as described above (if no EAS discovery filter received), and then provides the selected EAS information to the EEC in the discovered EAS list of EAS discovery response.

NOTE 4:	Without EAS selection request indication, the EES handling is as per R17 procedure.

3.	If the processing of the request was successful, the EES sends an EAS discovery response to the EEC, which includes information about the discovered EASs and Instantiable EAS Information. For discovered EASs, this includes endpoint information. Depending on the EAS discovery filters received in the EAS discovery request, the response may include additional information regarding matched capabilities, e.g. service permissions levels, KPIs, AC locations(s) that the EASs can support, ACR scenarios supported by the EAS, etc. The EAS discovery response may contain a list of EASs and Instantiable EAS Information. This list may be based on EAS discovery filters containing a Geographical or Topological Service Area, e.g. a route, included in the EAS discovery request by the EEC.

When the Application groups information (i.e. Application Group ID) is provided, the EES returns only one EAS associated to each of the application group ID received in the request.

If the EES is unable to determine the EAS information using the inputs in the EAS discovery request, UE-specific service information at the EES or the ECSP policy, the EES shall reject the EAS discovery request and respond with an appropriate failure cause.

If the EEC is not registered with the EES, and ECSP policy requires the EEC to perform EEC registration prior to EAS discovery, the EES shall include an appropriate failure cause in the EAS discovery response indicating that EEC registration is required.

If the UE location and predicted/expected UE locations, provided in the EAS discovery request, are outside the Geographical or Topological Service Area of an EAS, then the EES shall not include that EAS in the discovery response. The discovery response may include EAS(s) that cannot serve the UE at its current location if a predicted/expected UE location was provided in the EAS discovery request.

Upon receiving the EAS discovery response, if the EEC selects an EAS which is instantiated (i.e., an EAS profile was provided), the EEC uses the endpoint information for routing of the outgoing application data traffic to EAS(s), as needed, and may provide necessary notifications to the AC(s). The EEC may use the border or overlap between EAS Geographical Service Areas for service continuity purposes. The EEC may cache the EAS information (e.g. EAS endpoint) for subsequent use and avoid the need to repeat step 1. If the Lifetime IE is included in the response, the EEC may cache the EAS information only for the duration specified by the Lifetime IE.

Upon receiving the EAS discovery response, if the EEC selects an EAS which is not instantiated (i.e. an EAS profile is not provided), the EEC sends the EAS information provisioning request indicating the selected EASID as in clause 8.15.

NOTE 5:	Within the duration specified by the Lifetime IE, the cached EAS Profile can be updated (e.g. according to notifications from the EES for changes of EAS information due to EAS status change) or the cached EAS Profile can be invalidated due to new EAS information discovery (e.g. due to UE mobility). The EEC can update or invalidate the cached EAS information (e.g. on PDU Session Release or Modification Command).

NOTE 6:	The AC can cache the EAS information (e.g. EAS endpoint) for subsequent use. In the case of the cached information needing to be updated or invalidated, the mechanisms for the EEC to notify the AC is up to implementation and is not specified in the current release of the present document.

NOTE 7:	The EEC can use the EAS information provided by the discovery procedure to perform service continuity planning, for example when ultra-low latency ACR is required.

If the EAS discovery request fails, the EEC may resend the EAS discovery request, taking into account the received failure cause. If the failure cause indicated that EEC registration is required, the EEC shall perform an EEC registration before resending the EAS discovery request.

NOTE 8:	As long as a proper EAS (e.g. considering expected AC service KPIs included in EAS discovery request) is discovered and selected by the EES, EEC of a constraint UE can stop sending EAS discovery to rest candidate EES(s), and provide the selected EAS information to AC.

* * * End of Changes * * * *