3GPP TSG-SA6 Meeting #54-e	S6-231468

(revision of S6-231398)

# Online, , 17th Apr 2023 - 26th Apr 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0344 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CAS decided ACR scenario via service provisioning triggering for CESless architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  |  |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adding a procedure for CESless architecture for the CAS to be able to perform ACR to T-EAS combining existing procedures – clause 8.3.3 (service provisioning procedure) and clause 8.8.2.4 (S-EAS decided ACR scenario) where the CAS acts as an S-EAS – along with signaling between the CAS and the EEC via the AC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new procedure is added for CAS decided ACR scenario via service provisioning triggering for CESless architecture in Clause 8.8.2A. The procedure is similar with "S-EAS decided ACR scenario" as specified in clause 8.8.2.4, where the CAS acts like the S-EAS. However, during the ACR execution phase, the CAS needs to know the EES to use for the discovery before continuing with T-EAS discovery. This procedure uses the service provisioning detailed in clause 8.3.3 to get information of the EES following the CAS trigger. Following the provisioning, the necessary informations (EES endpoint information and the UE Identifier) are shared from the EEC to the CAS via the AC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CAS decided ACR scenario remains undefined when the CAS wants to perform an ACR to an T-EAS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.2A.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * Change 1 * * * *

#### 8.8.2A.x	CAS decided ACR scenario via service provisioning triggering

In this scenario, the CAS detects the need for ACR and makes the decision about whether to perform the ACR and starts the ACR at a proper time.

During the ACR execution phase, the CAS needs to determine an EES in the service area of the EEC before continuing with T-EAS discovery. Once the CAS determines the EES, the T-EAS discovery (step 6) and the remaining steps are similar to the "S-EAS decided ACR scenario" as specified in clause 8.8.2.4, where the CAS acts like the S-EAS.

Pre-condition: the CAS has a buisness relationshiop with the ECSP, otherwise, the EES will reject the ACR request during authorization.

Figure 8.8.2A.x-1: CAS decided ACR with EES discovery via service provisioning triggering

1.	The CAS detects the need for ACR. The CAS can detect ACR due to the UE location change.

NOTE 1:	How the CAS detects the need for ACR is outside the scope of this specification.

2.	The CAS makes the decision to perform the ACR.

NOTE 2:	How the CAS determines when to start the ACR is outside the scope of this specification.

3.	The CAS triggers the EEC via the AC to perform service provisioning.

NOTE 3:	How the CAS triggers service provisioning is out of scope.

4.	The EEC performs service provisioning as described in clause 8.3.3. The EEC subscribes to receive ACR information notifications for target information notification events and ACR complete events from the EES, as described in clause 8.8.3.5.2. If the service provisioning in step 4 fails, the CAS will not continue with ACR, and the rest of the steps are skipped. The EEC may perform EEC registration with the EES. If the EEC registration request contains an EEC context ID and source EES Endpoint, the receiver (i.e., target) EES performs an EEC Context Pull relocation (clause 8.9.2.2).

5.	The CAS receives EES endpoint information and the UE Identifier from the EEC via the AC. If the EEC already has the UE ID (GPSI as per clause 7.2.6), and it needs the Edge UE ID to share with an AC/CAS, the EEC retrieves the Edge UE ID using UE Identifier API in clause 8.6.5.

NOTE 4:	Detail on how the AC provides this information to the CAS is out of scope.

6.	Same as step 3 to step 7 in clause 8.8.3.2 where the CAS acts as an S-EAS.

NOTE 5:	When the CAS performs the EAS discovery operation, the EES checks whether the requesting CAS is authorized to perform the discovery operation and may decide to reject or accept the request.

7.	Same as step 8 to 10 in clause 8.8.3.8 where the CAS acts as an S-EAS.

# * * * End of Change * * * *