3GPP TSG-SA WG6 Meeting #54-e	S6-231471

(revision of S6-231221)

# Online, , 17th Apr 2023 - 26th Apr 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0292 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inclusion of IEs in EEC registration update response message to identify unfulfilled EASID(s) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | IIT Delhi, IIT Bhilai |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | An AC may be served by multiple EASID(s), mentioned as the list of EASID in AC profile. The AC should be able to identify the unfulfilled EASID (s) in table 8.4.2.3.5-1. The AC(s) requirement may be partially fulfilled by the EES, as IE "reason" alone cannot specify which of the services, corresponding to ACID, are not fulfilled. Adding Discovered EAS list IE in EEC registration update response present in EEC registration response will help AC to determine unfulfilled EASID (s) and allow an AC to update its AC profile with a reduced list of EASID(s). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Discovered EAS list IE in EEC registration update response present in EEC registration response will help AC to determine unfulfilled EASID (s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EEC may not be able to identify the unfulfilled EASID(s) corresponding to the respective unfulfilled ACID as IE "reason" may not provide sufficient information. Adding Discovered EAS list IE in EEC registration update response present in EEC registration response will help AC to determine unfulfilled EASID (s) |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.3.3, 8.4.2.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 8.4.2.3.5	EEC registration update response

Table 8.4.2.3.5-1 describes information elements in the EEC registration update response from the EES to the EEC.

Table 8.4.2.3.5-1: EEC registration update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the registration update request was successful. |
| > Expiration time (see NOTE) | O | Indicates the expiration time of the registration. To maintain an active registration status, a registration update is required before the expiration time. |
| > Discovered EAS list | O | List of EASs discovered to provide the capabilities required by the AC Profiles. If the EEC registration request includes the EAS selection request indicator, then Discovered EAS list shall contain only one selected EAS. If the EES selects no EASs, the list may be empty. |
| >> EAS profile | M | Profile of the EAS. Each element is described in clause 8.2.4. |
| > list of unfulfilled AC information | O | List of ACIDs of the AC Profile(s) for which the requirements indicated in the AC profile(s) cannot be fulfilled |
| >> ACID | M | Application Identifier |
| >> reason | O | Reson indicating the cause (e.g. EAS not available, requirements cannot be fulfilled) |
| Failure response | O | Indicates that the registration update request failed. |
| NOTE: The Expiration time IE shall be included if based on the ECSP policy the EES requires EEC to periodically refresh the EEC registration. |  |  |

END OF CHANGES