3GPP TSG-SA WG6 Meeting #54-e	S6-231476

e-meeting, 17th April – 26th April 2023	(revision of S6-231350)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0316 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Subscription for service provisioning information retrieval by ECS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Following Editor’s Note: Editor's note: whether and how to specify subscribe-notify model for service provisioning information retrieval is FFS. A subscribe/notify model is required for the Edge Node Sharing scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Subscribe notify model is specified. - A clarification is added in clasue 8.17.2.4.1 that: “ECS sending the request can be an ECS-ER of an ECSP and Partner ECS receiving the request can be the ECS-ER of the partner ECSP.” - Note with respect to V-ECS is moved to the general clause. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete functionality and open editor’s notes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.17.2.4.1, 8.17.2.4.2.1, 8.17.2.4.2.2, 8.17.2.4.2.3 (NEW), 8.17.2.4.2.3.1 (NEW), 8.17.2.4.2.3.2 (NEW), 8.17.2.4.2.3.3 (NEW), 8.17.2.4.2.3.4 (NEW), 8.17.2.4.2.3.5 (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.17.2.4	Service provisioning information retrieval

##### 8.17.2.4.1	General

When required to retrieve Service provisioning information from a partner ECS, the ECS queries the partner ECS by providing information such as required applications, location, service continuity support information and connectivity information. In response the partner ECS provides Service provisioning information of EDNs satisfying the requirements.

The following clauses specify procedures, information flows and APIs for service provisioning information retrieval.

In the following procedures, ECS sending the request can be an ECS-ER of an ECSP and Partner ECS receiving the request can be the ECS-ER of the partner ECSP.

NOTE:	Partner ECS can also be a V-ECS based on the roaming status of the UE for which the service provisioning information is being retrieved.

##### 8.17.2.4.2	Procedures

###### 8.17.2.4.2.1	General

Following procedures are supported for service provisioning:

-	Request-response model; and

-	Subscribe-notify model.

###### 8.17.2.4.2.2	Request-response model

Figure 8.17.2.4.2.2-1 illustrates service provisioning procedure based on request/response model.

Pre-condition(s):

1.	ECS has received a sevice provisioning request from an EEC or T-EES discovery request from an EES.

2.	The ECS has the ECS configuration information of the partner ECS (e.g. through preconfiguration, OAM configuration or ECS discovery).

3.	Required federation agreement as described in Annex B.2-1 exists.

Figure 8.17.2.4.2.2-1: Service provisioning information retrieval – Request/Response

1.	The ECS sends a service provisioning information retrieval request to the partner ECS. The request contains security information of the ECS, Service Provisioning filters received from the EEC or the EES and UE’s current location. If the UE hosting the EEC is roaming in a V-PLMN (determined using the serving PLMN information in the received service provisioning request, or by interacting with the H-PLMN), then the service provisioning infromation retrieval request includes the information of the serving PLMN i.e. the V-PLMN.

2.	The partner ECS authorizes the received request. If authorized, the partner ECS processes the request and gathers the provisioning information of the EDNs that can satisfy the query parameters received in step 1.

3.	The partner ECS sends a service provisioning information retrieval response to the ECS. The response includes provisioning information of the EDNs that can satisfy the query parameters received in step 1 and may include lifetime information. The ECS caches the received information for further use. If the Lifetime IE is included, then the ECS may reuse the retrieved information only for the duration specified by the Lifetime IE, without the need to repeat step 1.

NOTE:	Partner ECS can be a V-ECS based on the roaming status of the UE for which the service provisioning information is being retrieved.

NOTE:	Only the information allowed by the federation agreement is included in the service provisioning information response.

Editor's note:	Information flows for service provisioning information retrieval is FFS. It is FFS whether this procedure and its information flows can be merged to the existing service procedure and flows.

Editor's note: Applicability of this procedure for federation is FFS.

###### 8.17.2.4.2.3	Subscribe-notify model

8.17.2.4.2.3.1	General

8.17.2.4.2.3.2	Subscribe

Figure 8.17.2.4.2.3.2-1 illustrates service provisioning information subscription request between an ECS and its partner ECS.

Pre-condition(s):

1.	The ECS has the ECS configuration information of the partner ECS (e.g. through preconfiguration, OAM configuration or ECS discovery).

2.	Required federation agreement as described in Annex B.2-1 exists.

Figure 8.17.2.4.2.3.2-1: Service provisioning information retrieval – Subscribe

1.	The ECS sends a service provisioning information subscription request to the partner ECS. The request contains security information of the ECS, Notification Target Address (e.g. URL), federation information, Service Provisioning filters.

2.	The partner ECS authorizes the received request. If authorized, the partner ECS creates a subscription and stores the subscription.

3.	The partner ECS sends a service provisioning information subscription response to the ECS. The response includes the subscription identifier and may include the expiration time, indicating when the subscription will automatically expire. To maintain the subscription, the ECS shall send a service provisioning information subscription update request prior to the expiration time. If a service provisioning information subscription update request is not received prior to the expiration time, the partner ECS shall treat the ECS as implicitly unsubscribed.

8.17.2.4.2.3.3	Notify

Figure 8.17.2.4.2.3.3-1 illustrates the service provisioning information notification procedure between an ECS and its partner ECS.

Pre-conditions:

1.	ECS has subscribed for service provisioning information from the partner ECS as specified in clause 8.17.2.4.2.3.2.

Figure 8.17.2.4.2.3.3-1: Service provisioning information notification

1.	An event occurs at the partner ECS that satisfies trigger conditions for notifying (e.g. to provide a change in information about available EASs) a subscribed ECS.

2.	The Partner ECS sends a service provisioning information notification to the ECS. The notification includes updated provisioning information of the EDNs that can satisfy the query parameters received during subscription and may include lifetime information. The ECS caches the received information for further use. If the Lifetime IE is included, then the ECS may reuse the received information only for the duration specified by the Lifetime IE.

NOTE:	Only the information allowed by the federation agreement is included in the service provisioning information notification.

8.17.2.4.2.3.4	Subscription update

Figure 8.17.2.4.2.3.4-1 illustrates service provisioning information subscription update request between an ECS and its partner ECS.

Pre-condition(s):

1.	ECS has subscribed for service provisioning information from the partner ECS as specified in clause 8.17.2.4.2.3.2.

Figure 8.17.2.4.2.3.4-1: Service provisioning information retrieval – Subscription update

1.	The ECS sends a service provisioning information subscription update request to the partner ECS. The request contains security information of the ECS and subscription identifier, and may include updated Notification Target Address (e.g. URL), federation information, Service Provisioning filters, location information and PLMN information.

2.	The partner ECS authorizes the received request. If authorized, the partner ECS updates the stored subscription.

3.	The partner ECS sends a service provisioning information subscription update response to the ECS. The response may include updated expiration time, indicating when the subscription will automatically expire. To maintain the subscription, the ECS shall send a service provisioning information subscription update request prior to the expiration time. If a service provisioning information subscription update request is not received prior to the expiration time, the partner ECS shall treat the ECS as implicitly unsubscribed.

8.17.2.4.2.3.5	Unsubscribe

Figure 8.17.2.4.2.3.5-1 illustrates service provisioning subscription update request between an ECS and its partner ECS.

Pre-condition(s):

1.	ECS has subscribed for service provisioning information from the partner ECS as specified in clause 8.17.2.4.2.3.2.

Figure 8.17.2.4.2.3.5-1: Service provisioning information retrieval – Unsubscribe

1.	The ECS sends a service provisioning information unsubscribe request to the partner ECS. The request contains security information of the ECS and the subscription identifier.

2.	The partner ECS authorizes the received request. If authorized, the partner ECS cancels the subscription.

3.	The partner ECS sends a service provisioning information unsubscribe response to the ECS.