3GPP TSG-SA WG6 Meeting #54-e	S6-231542

17th – 26th April 2023	(revision of S6-231306)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0313 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Edge Node Sharing solution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | KI#22, Solution #44 was studied in TR 23.700-98. In Solution #44 studied procedure to Get list of all registered EAS from partner OP. GSMA OPG has requirement: - Get a list of Applications; for an OP to retrieve the list of published application instances with specific criteria (e.g. edge location, availability zone, etc.) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Specified procedure to Get list of all registered EAS from partner ECSP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | GSMA requirement will not be fulfilled in Rel-18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.18.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | KI#22, Solution #44 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.18.2.4	EAS discovery via Leading ECSP

Pre-conditions:

1.	ECSP-1 and ECSP-2 have a service level agreement to share edge services.

2.	EES (ECSP-B) has received EAS discovery request where required EAS is not available with the EES (ECSP-B).

Figure 8.18.2.4-1: EAS discovery for edge node sharing

1.	The EEC sends EAS discovery request to EES (OP-B) and may include the endpoint of EES (OP-A) in the request.

2.	The EES (OP-B) validates the edge service SLA. If EES (OP-A) endpoint is included, the EES (OP-B) skips step 3-5; otherwise, the EES (OP-B) executes for EAS discovery but cannot find any desired EAS so it determines to use edge node sharing service based on edge service SLA.

3.	The EES (OP-B) sends Retrieve EES request to the ECS (OP-B). The request may include an edge node sharing indication indicating edge node sharing is requested so that the ECS (OP-B) can skip checking T-EES(s) registered in the ECS (OP-B).

NOTE 1:	If EES (OP-A) information is not carried in EAS discovery request in step 1, the EES (OP-B) can determine to retrieve EES (OP-A) information from ECS (OP-B) as in step 3-5.

4.	For the received EAS ID, the ECS (OP-B) discovers T-EES (i.e. EES(s) from partner).

5.	The ECS (OP-B) returns discovered EES(s) of OP-A in the Retrieve EES response to the ECS (OP-B).

6.	The EES (OP-B) sends EAS discovery request to the EES (OP-A). The EAS discovery request includes the serving MNO information (e.g. MNO name, PLMN ID) which is serving the UE.

7.	The EES (OP-A) validates the edge service SLA and returns EAS discovery response including the discovered candidate EAS(s) to the EES (OP-B). The candidate EAS(s) are discovered based on the serving MNO information received from the EES (OP-B) and allowed MNO information registered by the EAS.

NOTE 2:	Upon receiving EAS discovery request from EEC, the EES (OP-B) may send EAS discovery request without any discovery filters. In such case, EES (OP-A) provides list of all registered and allowed EASs in the response message.

8.	The EEC receives the EAS discovery response sent by the EES (OP-B). The EEC may select an EAS for the AC.

Editor's note:	It is FFS whether to add the interaction between EES (OP-B) and ECS (OP-B) for T-EES retrieval after step 2.

* * * End of Change * * * *