3GPP TSG-SA WG6 Meeting #54-e	S6-231321577

17th – 26th April 2023	(revision of S6-231321)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0110 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Revoking notification to other AEFs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon, Hytera |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, eCAPIF |  |  |  |  |  | Date: |  |  | 2023-04-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | One API may be provided by multiple AEFs. When the API invocation is revoked by one of those AEFs, the other ones shall be notfied. However, this is missing in TS 23.222. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Update the clause 8.23.3 to notify other AEFs who provide the same API 2) Upate the clause 8.23.4 to reflect the case that one API may be provided by multiple AEFs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lose access control of the API invocation at certain AEFs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.23.3, 8.23.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************** First Changes ********************/

8.23.3	Procedure for CAPIF revoking API invoker authorization initiated by AEF

Figure 8.23.3-1 illustrates the procedure for revoking API invoker authorization to access service API initiated by the AEF.

Pre-conditions:

1.	The API invoker is authenticated and authorized to use the service API.

2.	The AEF in the CAPIF is configured with the access policy to be applied to the service API invocation corresponding to the API invoker and the service API.

3.	Authorization details of the AEF are available with the CAPIF core function.

API invoker

CAPIF Core function

AEF

2. Revoke API invoker authorization request

1. Trigger for revoking API invoker authorization

3. Invalidate the authorization of the API invoker for service API

4. Revoke API invoker authorization response

6. Revoke API invoker authorization notify

5. Invalidate the authorization of the API invoker for service API

7. Notify other AEFs to invalidate the API invoker authorization corresponding to the service API as described in step 2~4 of clause 8.23.4

Figure 8.23.3-1: Procedure for revoking API invoker authorization initiated by AEF

1.	The AEF triggers the revocation of the API invoker authorization.

2.	The AEF sends revoke API invoker authorization request to the CAPIF core function with the details of the API invoker and the service API.

3.	Upon receiving the information to revoke the API invoker's authorization for service API invocation, the CAPIF core function invalidates the API invoker authorization corresponding to the service API.

4.	The CAPIF core function sends a revoke API invoker authorization response to the AEF.

5.	Upon successful revocation of API invoker authorization corresponding to the service API at the CAPIF core function, the AEF invalidates the API invoker authorization corresponding to the service API.

6.	The CAPIF core function sends a revoke API invoker authorization notify to the API invoker whose authorization to access the service API has been revoked.

7.	Optionally, if the service API is provided by multiple AEFs, the CAPIF core function based on the cause from step 2, may notify the other AEFs to invalidate the API invoker authorization corresponding to the service API as described in step 2 to step 4 of clause 8.23.4.

8.23.4	Procedure for CAPIF revoking API invoker authorization initiated by CAPIF core function

Figure 8.23.4-1 illustrates the procedure for revoking API invoker authorization to access service API initiated by the CAPIF core function.

Pre-conditions:

1.	The API invoker is authenticated and authorized to use the service API.

2.	The AEF in the CAPIF is configured with the access policy to be applied to the service API invocation corresponding to the API invoker and the service API.

Figure 8.23.4-1: Procedure for revoking API invoker authorization initiated by CAPIF core function

1.	The CAPIF core function triggers the revocation of the API invoker authorization.

2.	The CAPIF core function sends revoke API invoker authorization request to the AEF with the details of the API invoker and the service API.

NOTE:	The step 2 can be sent to multiple AEFs if applicable.

3.	Upon receiving the information to revoke the API invoker's authorization for service API invocation, the AEF invalidates the API invoker authorization corresponding to the service API.

4.	The AEF sends a revoke API invoker authorization response to the CAPIF core function.

5.	The CAPIF core function invalidates the API invoker authorization corresponding to the service API.

6.	The CAPIF core function sends a revoke API invoker authorization notify to the API invoker whose authorization to access the service API has been revoked.