3GPP TSG-SA WG6 Meeting #54-e	S6-231580

17th – 26th April 2023	(revision of S6-131326)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0205 | rev | 1 | Current version: | 18.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoS management for network-assisted UE-to-UE communications update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-11 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA2 has defined a new API for the AF to request provisioning of traffic characteristics and monitoring of performance characteristics for a 5G VN group as described in TS 23.501. For UE-to-UE traffic, SA2 defined that the AF can invoke SEAL service as described in clause 14.3.5 of TS 23.434 to handle requirements for UE-to-UE traffic. “ NOTE 2: The AF can invoke SEAL service as described in clause 14.3.5 of TS 23.434 [177] to handle requirements for UE-to-UE traffic, the SEAL service can perform coordinated QoS/resource management for network assisted UE-to-UE communications to guarantee end-to-end QoS fulfilment (primarily for meeting end-to-end latency requirements), e.g. decompose the requirements for UE-to-UE traffic to initial individual QoS parameters for each individual session, and apply QoS adaption for the individual session that receive QoS notification control notifications based on fulfilment/non-fulfilment evaluation on the requirements for UE-to-UE traffic. ” However there are the following problems the procedure described in clause 14.3.5: 1.Only support QoS/resource management or coordination for VAL UE1 to VAL UE2 identified by each UE, not a group of UE, 2.The procedure is not clear how NRM server supports the decomposition of the QoS of UE-to-UE traffic for each individual session, e.g. an uplink network session and a downlink network session, regardless for individual VAL UE 1 to VAL UE 2, or for a group of VAL UEs communication 3. The procedure was not mentioned how NRM server updates with the decomposed QoS parameter for each individual session for 5GC, but it is an input and is meaningful for the produre 14.3.5.3.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new proccedur for group UE E2E qos management Update the information flow in clause 14.3.2.13 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The QoS/resource management or coordination for end-to-end communications is not clear, and unable for SA2 to refer for support of provisioning of traffic characteristics and monitoring of performance characteristics for a 5G VN group. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.5.1, 14.3.5.2.X, 14.3.5.3.X, 14.3.2.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

14.3.5	QoS/resource management for network-assisted UE-to-UE communications

14.3.5.1	General

This feature provides the SEAL NRM support for coordinated QoS/resource management for network assisted UE-to-UE communications. Such capability may be required for guaranteeing end-to-end QoS fulfilment (primarily for meeting end-to-end latency requirements) in network assisted UE to UE communications and may accommodate various vertical-specific application services, e.g.:

-	Network-assisted Command and Control (C2) communications in UASAPP [TS 23.255], where the UAV controller navigates its UAV over the 5GS;

-	Teleoperated Driving (ToD) in eV2XAPP [TS 23.286], where the a V2X UE acting as server may remotely control a further V2X UE over the 5GS;

-	Network-assisted Device-to-Device communications in Factory of the Future (FF) use cases, such as control-to-control communications.

-	5G LAN-Type communication within a 5G VN group as specified in 3GPP TS 23.501 [10].

14.3.5.2	QoS/resource management capability initiation in network assisted UE-to-UE communications

This procedure provides a mechanism for initiating the capability at the NRM server for managing the end-to-end application QoS requirement fulfilment for a network-assisted VAL UE to VAL UE session (comprising a PDU session for each of the constituent links, e.g. VAL UE 1 to PLMN, and PLMN to VAL UE 2). The request may come from NRM client of either of the VAL UEs within the service and will trigger the end-to-end QoS/resource management by the NRM server. The triggering the end-to-end QoS management request can be initiated by the VAL application at the VAL UE, and the conditions may depend on the requirements of the VAL service, e.g. for UAS such triggering may be needed when a UAV is in-flight, or for V2X such trigger may be initiated when a controlled VAL UE enters an urban area, or for 5G LAN-type service such trigger may be initiated by the AF (e.g. on VAL UE1) that manages the corresponding 5G VN group.

# * * * * Next change * * * *

14.3.5.2.X	Procedure

Figure 14.3.5.2.x illustrates the procedure where the NRM server is initiating the end-to-end QoS/resource management capability for network-assisted UE-to-UE communications for a group of UEs.

Pre-conditions:

1.	The NRM client is connected to the NRM server.

2.	The VAL UEs involved in the end-to-end session (VAL UE 1 and a group of VAL UEs) are connected to one or more PLMNs and have ongoing PDU sessions.

Note : The NRM client2 (of VAL UE 2) can be any VAL UEs in the group of VAL UEs.

Figure 14.3.5.2.x end-to-end QoS management request / response for a group of UEs

1.	The NRM server receives from the AF on NRM client 1 (of VAL UE 1) the end-to-end QoS management request for managing the QoS on UE-to-UE traffic for a group of UEs.

2. The NRM server retrieves from 5GC or subscribe to 5GC to obtain additional VAL-UE associated information for each member in the group identified by the VAL group ID, which is account for decomposition. The VAL-UE associated information could be from the 5GC (NEF Monitoring Events as in 3GPP TS 23.502 [11], QoS sustainability analytics as in 3GPP TS 23.288 [34]) or SEAL LMS (on demand location reporting).

3.	The NRM server configures the application QoS parameters by decomposing the end-to-end QoS requirements (UE-to-UE traffic for any two UEs in a group) to application QoS parameters for each individual session (uplink network session for ingress group member and downlink network session for egress group member). The NRM server needs to take the additional VAL-UE associated information into account for evaluation during such decomposition.

4. The NRM/SEAL server, acting as AF, sends to the 5GC a  "Procedures for AF requested QoS for a UE or group of UEs not identified by a UE address" to set the application QoS parameters respectively for uplink network session for each ingress group member in the group and for downlink network session for egress group member.

Additionally, the NRM/SEAL server may activate monitoring of the performance to receive QoS monitoring event notifications from 5GC by setting the (optionally with Alternative QoS Profiles) in the request, in this case the AF may receive the QoS downgrade notification for e.g. latency thus to initiate the NRM-assisted coordinated QoS provisioning for 5G LAN-Type  communication in clause 14.3.5.3.x

5.	The NRM server sends to the NRM client 1an end-to-end QoS management response with a positive or negative acknowledgement of the request.

6.	If the NRF server receives the end-to-end QoS management request including VAL UE2 as any of the List of VAL UEs, then the NRM server may also send a notification to NRM client 2 (of VAL UE 2) to inform about the end-to-end QoS management initiation by the NRM server.

# * * * * Next change * * * *

14.3.5.3.X	Procedure

Figure 14.3.5.3.x illustrates the procedure where the NRM server supports the coordinated QoS provisioning for network-assisted UE-to-UE communications.

Pre-conditions:

1.	NRM server has activated the end-to-end QoS/resource management capability, as described in 14.3.5.2.1

2.	NRM server, acting as AF, has registered to receive QoS monitoring event notifications from 5GC and notifications from VAL UEs (from any UEs in a group), as specified in TS 23.501.

Figure 14.3.5.3.x: NRM-assisted coordinated QoS provisioning for 5G LAN-Type communication

1.	The NRM server receives a trigger event from the 5GC (SMF/NEF), denoting a QoS downgrade notification for the network session of any group member in a group. (described in clause 5.7.2.4.1b of 3GPP TS 23.501 [10]).

2.	The NRM server evaluates the fulfilment/non-fulfilment of the end-to-end QoS based on the trigger event. NRM server may retrieve additional information based on subscription to support its evaluation. This could be from the 5GC (NEF Monitoring Events as in 23.502, QoS sustainability analytics as in TS 23.288 [34]) or SEAL LMS (on demand location reporting for one or both VAL UEs 1 and 2).

Then, the NRM server, determines an action, which is the QoS parameter adaptation of only uplink/downlink network session or both uplink and downlink network sessions for each ingress group member in the group.

3.	The NRM/SEAL server, acting as AF, sends to the 5GC (to SMF via NEF or to PCF via N5) a request for a change of the QoS profile mapped to the impacted network sessions or the update of the PCC rules to apply the new traffic policy (as specified in 3GPP TS 23.502 [11] in clause 4.15.6.14: AF requested QoS for a UE or group of UEs not identified by a UE address procedure).

4.	The NRM server sends an application QoS change notification to the affected NRM clients, to inform on the adaptation of the QoS requirements for the individual session.

# * * * * Next change * * * *

14.3.2.13	end-to-end QoS management request

Table 14.3.2.13-1 describes the information flow end-to-end QoS management request from the NRM client to the NRM server.

Table 14.3.2.13-1: end-to-end QoS management request

| Information element | Status | Description |
| --- | --- | --- |
| List of VAL UEs or VAL Group ID | M | List of VAL UEs or the group of VAL UEs for whom the end-to-end QoS management occurs |
| >VAL UE/user ID | M | Identity of the VAL UE |
| > IP address | O | IP address of the VAL UE |
| VAL service ID | O | The VAL service identity for whom the end-to-end QoS management occurs. |
| End-to-end QoS requirements | O | The application QoS requirements / KPIs (latency, error rate,..) for the end-to-end session. This may optionally include information which will support the NRM server to identify the per session QoS requirements (e.g. a flag indicating the use of HD video for assisting the end-to-end session, a video resolution/encoding required for the HD video). |
| Service area | O | The area where the QoS management request applies. This can be geographical area, or topological area. |
| Time validity | O | The time of validity of the requirement. |