3GPP TSG-SA WG6 Meeting #54-e	S6-231592

17th – 26th April 2023	(revision of S6-231412)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0215 | rev | 1 | Current version: | 18.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Coordinated application-level direct UE-to-UE communications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Provides neccesary changes to include TR 23.745 solutions 1 and 11 from FFApp to SEAL |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduces Ue-to-UE coordination based on application requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FFApp functionality not supported in SEAL |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.2.x1 (new); 14.3.2.x2 (new); 14.3.2.x3 (new); 14.3.2.x4 (new); 14.3.2.x5 (new); 14.3.Y.1 (new); 14.3.Y.2 (new); 14.3.Y.3 (new); |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | n | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | n | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | n | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1 provides corrections to several steps of the procedire in 14.3.Y and to the figure. It also clarifies that the UE-to-UE direct communications enabled are not PC5, but realized over Uu. This corresponds to the requirements and solutions for FFApp, as described in TR 23.745 (see FFA-2 reference point) |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[3]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[4]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[5]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[6]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[7]	3GPP TS 23.286: "Application layer support for V2X services; Functional architecture and information flows".

[8]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[9]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[10]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[11]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[12]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[13]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[14]	3GPP TS 23.002: "Network Architecture".

[15]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[16]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[17]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[18]	3GPP TS 23.203: "Policy and charging control architecture".

[19]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[20]	3GPP TS 26.348: "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point".

[21]	3GPP TS 29.214: "Policy and charging control over Rx reference point".

[22]	3GPP TS 29.468: "Group Communication System Enablers for LTE (GCSE_LTE); MB2 Reference Point; Stage 3".

[23]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[24]	IETF RFC 6733 (October 2012): "Diameter Base Protocol".

[25]	ETSI TS 102 894-2 (V1.2.1): "Intelligent Transport Systems (ITS); Users and applications requirements; Part 2: Applications and facilities layer common data dictionaryMultimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[26]	ETSI TS 102 965 (V1.4.1): "Intelligent Transport Systems (ITS); Application Object Identifier (ITS-AID); Registration".

[27]	ISO TS 17419: "Intelligent Transport Systems - Cooperative systems - Classification and management of ITS applications in a global context".

[28]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[29]	3GPP TS 33.434: "Service Enabler Architecture Layer (SEAL); Security aspects for Verticals".

[30]	3GPP TS 29.549: "Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification; Stage3".

[31]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[32]	IETF RFC 7252: "The Constrained Application Protocol (CoAP)".

[33]	IETF RFC 8323: "CoAP (Constrained Application Protocol) over TCP, TLS, and WebSockets".

[34]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[35]	IEEE Std 802.1Qcc-2018: "Standard for Local and metropolitan area networks - Bridges and Bridged Networks - Amendment: Stream Reservation Protocol (SRP) Enhancements and Performance Improvements".

[36]	IEEE 802.1Q-2018: "IEEE Standard for Local and Metropolitan Area Networks—Bridges and Bridged Networks".

[37]	IEEE Std 802.1CB-2017: "Frame Replication and Elimination for Reliability".

[38]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[39]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[40]		3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[41]		3GPP TS 28.531: "Management and orchestration; Provisioning".

[42]		3GPP TS 28.533: "Management and orchestration; Architecture framework".

[43]		3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[44]		3GPP TS 28.532: "Management and orchestration; Generic management services".

[45]		3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[46]		3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[47]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[48]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".

[49]	3GPP TS 23.436: "Procedures for Application Data Analytics Enablement Service".

[50]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2"

[51]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3"

[52]	3GPP TS 23.256 "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

[x]	3GPP TS 23.545 “Application layer support for Factories of the Future (FF)".

* * * Next Change * * * *

#### 14.3.2.x1	UE-to-UE resource coordination request

Table 14.3.2.x1-1 describes the information flow for the unicast resource request based on application specific requirements. This request from NRM client to NRM server is issued for unicast resources for UE-to-UE application-level direct communications (e.g. over FFA-2 reference point, see TS 23.545[x]).

Table 14.3.2.x1-1: UE-to-UE resource coordination request

| Information element | Status | Description |
| --- | --- | --- |
| Requester VAL UE ID | M | The identity of the source VAL client for which the NRM client performs the request. |
| Requester IP address | M | IP address of the requester |
| Destination VAL user ID or VAL UE ID | M | The identity of the destination VAL user or VAL UE. |
| VAL application specific requirement (see NOTE) | O | VAL application specific requirements for unicast resource (e.g. VAL service ID, Bitrate) |
| UE-to-UE connection coordination context (see NOTE) | O | UE-to-UE connection coordination context of the requester, used for determining the destination and/or resource parameters. See Table 14.3.2.x1-2 |
| NOTE: When this information element is not included, the NRM server considers default VAL service requirements for the unicast resources. |  |  |

Table 14.3.2.x1-2: UE-to-UE connection coordination context

| Information element | Status | Description |
| --- | --- | --- |
| Location | O | Location information of the client. The element may also indicate whether network-determined location is to be used instead. |
| Speed | O | Speed information for the client. |
| Direction | O | Direction information for the client. |
| NOTE: If the UE-to-UE connection coordination context is provided, at least one of the information elements in this table shall be provided. |  |  |

#### 14.3.2.x2	UE-to-UE resource coordination response

Table 14.3.2.x2-1 describes the information flow for the resource response from NRM server to VAL server for unicast resources for UE-to-UE application-level direct communications (e.g. over FFA-2 reference point see TS 23.545[x]).

Table 14.3.2.x2-1: UE-to-UE resource coordination response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of the UE-to-UE resource coordination response operation. |

#### 14.3.2.x3	UE-to-UE resource coordination notification

Table 14.3.2.x3-1 describes the information flow for the notification from NRM server to NRM client of resource coordination having been performed for UE-to-UE application-level direct communications (e.g. over FFA-2 reference point see TS 23.545[x]).

Table 14.3.2.x3-1: UE-to-UE resource coordination notification

| Information element | Status | Description |
| --- | --- | --- |
| Requester UE ID | M | The identity of the source VAL UE ID |
| Destination VAL UE ID | M | The identity of the destination VAL UE ID. |
| VAL service ID | M | Identify of the VAL service for which the resource coordination has been performed |

#### 14.3.2.x4	Get UE-to-UE connection coordination context request.

Table 14.3.2.x4-1 describes the information flow for coordination context request from the NRM server to the NRM client, the coordination applying to UE-to-UE application-level direct communications (e.g. over FFA-2 reference point see TS 23.545[x]).

.

Table 14.3.2.x4-1: Get UE-to-UE connection coordination context request.

| Information element | Status | Description |
| --- | --- | --- |
| Requester UE ID | M | The identity of the source VAL client for which the NRM client performs the request. |
| VAL service ID | M | Identify of the VAL service for which the information is requested. |
| VAL-specific connection coordination context information | O | Additional information required to identify the context data (e.g. device type, device vendor, etc) |

#### 14.3.2.x5	Get UE-to-UE connection coordination context response.

Table 14.3.2.x5-1 describes the information flow for coordination response from the NRM client to the NRM server, the coordination applying to UE-to-UE application-level direct communications (e.g. over FFA-2 reference point see TS 23.545[x]).

Table 14.3.2.x5-1: UE-to-UE connection coordination context response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of the UE-to-UE resource coordination response operation. |
| UE-to-UE connection coordination context (see NOTE) | O | UE-to-UE connection coordination context of the requester, used for determining the destination and/or resource parameters. See Table 14.3.2.x1-2 |
| NOTE: When this information element is not included, the NRM server considers default VAL service requirements for the unicast resources. |  |  |

* * * Next Change * * * *

### 14.3.Y   Coordinated UE-to-UE application-level direct communications based on application requirements.

#### 14.3.Y.1	General

The procedures related to coordinating UE-to-UE application-level direct communication (e.g. over FFA-2 reference point see TS 23.545[x]) are described in the following subclauses. NRM coordinates, via 5GS, the necessary unicast resources based on application service requirements received from the VAL layer.

NOTE: The procedures in this clause are not applicable to PC5 communications.

#### 14.3.Y.2	Coordinated connection establishment with application requirements.

Pre-conditions:

-	NRM client 1 and NRM client 2 are provided configuration information for the VAL clients served e.g. connectivity requirements, etc.

-	Pre-processing determines that network assisted UE-to-UE communications is required. VAL application policies and destination information for NRM clients are available at the NRM server.

Figure 14.3.Y.2-1: Coordination UE-to-UE communications with VAL application requirements

- VAL client 1sends a coordinated resource request to the NRM client on the UE1, requesting to establish connectivity for VAL client 1 on UE 1. The destination VAL client(s) may be hosted on one or multiple UEs (devices).

- The NRM client 1 sends the UE-to-UE resource coordination request (source identity and IP address, destination identities, application requirements) to the NRM server to establish connectivity for VAL client 1 on UE 1. The destination VAL client(s) may be hosted on one or multiple UEs (devices).

3.	The NRM server determines whether VAL client 1 is authorized to connect to VAL client 2 for application-level direct UE-to-UE communications. If VAL client 1 is authorized to connect to VAL client 2, the NRM server performs the retrieve  VAL UE-to-UE connection coordination context procedure as described in clause 14.3.5.Y.3. This step can be skipped if the NRM server is already aware of VAL client 2's context information.

NOTE:	The signaling and functionality for handling the cases when NRM client 2 will be temporarily unavailable for establishing the direct service connection are implementation dependant.

4.	The NRM server  sends the UE-to-UE resource coordination response to NRM client 1.

5.	The NRM server uses VAL client 1's and VAL client 2's context information, their application-level direct UE-to-UE connectivity requirements, location information, and network context as input, checks connectivity service policies, and determines the parameters and patterns for application-level direct UE-to-UE connectivity between  the VAL clients. The NRM server may also determine transport requirements, e.g. QoS requirements, for the 3GPP system (e.g. 5GS).

If network provided location information is used, location information may be obtained from the SEAL location management server. Alternatively, Location Reporting monitoring as described in 23.502[12] may be used. This step may also include a request for direct link status (e.g. PDU Session Status, UE reachability, etc. as described in 23.502 [12]).  This action may be skipped if  the clients provide location information or if there are no location requirements for establishing the application-level direct UE-to-UE connectivity.

If the NRM server determines that UE-to-UE application-level direct connectivity is not authorized or not possible with the given connectivity requirements, it skips step 5 and proceeds to steps 6 and 7, informing each NRM client accordingly.

6.	The NRM server may request the 3GPP system to establish or modify the 3GPP system level connectivity that enables the application-level direct UE-to-UE connection for VAL client 1 and VAL client 2 services, e.g. via modification of existing radio bearers. NRM server provides the necessary information (e.g. identifiers of VAL client 1 and VAL client 2, transport requirements) in this request message.

7.	a. The NRM server notifies NRM client 1 of the established UE-to-UE connection b. The NRM server notifies the NRM client 2 of the established UE-to-UE connection.

8.	The NRM client 1 notifies VAL client 1 of the established application-level direct UE-to-UE connection.

9.   The NRM client 2 notifies VAL client 2 of the established application-level direct UE-to-UE connection.

#### 14.3.Y.3	Retrieve VAL UE-to-UE connection coordination context.

The procedure for retrieving VAL application-level direct UE-to-UE  connection coordination context by the NRM Server from the NRM client is described in figure 14.3.Y.3-1.

Pre-conditions:

-	The NRM  server has received UE-to-UE resource coordination request.

Figure 14.3.Y.3-1: Retrieve VAL UE-to-UE connection coordination context.

- The NRM server requests the VAL application-level direct UE-to-UE connection coordination context data from the NRM client. The request provides information about the counterpart UE VAL application applicable.

- Using the request information and local policies, the NRM client determines whether context information is to be provided for establishing application-level direct UE-to-UE  connectivity to the counterpart UE for the VAL application indicated. If the NRM client determines that context information is to be provided, it responds to the NRM server and provides the VAL application-level direct UE-to-UE connection coordination context data for the VAL client served.

* * * End of Changes * * * *