3GPP TSG-SA WG6 Meeting #54-e bis	S6-231596

e-meeting, 17th – 26th April  2023		(revision of S6-231382 rev 2)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0330 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Common EAS support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless LLC, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | “Common EAS” services are those provided to group of UEs which need to connect to the same EAS e.g. for better user experience Service provisioning and the EAS discovery procedure need to be enhanced for the Common EAS feature to be supported . |  |  |  |  |  |  |  |  |
| Summary of change: |  | The service provisioning procedure and EAS discovery procedure are enhanced for Common EAS support |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Common EAS feature will remain under specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.4; 7.2.x (new); 8.2.x (new); 8.3.3.2.2; 8.3.3.3.2, 8.3.3.3.3, 8.5.2.2, 8.5.3.2, 8.15.1; 8.15.2.1; 8.15.2.2; 8.15.3.2; 8.x (new); 8.x.1 (new); 8.x.2 (new); 8.x.3 (new); 8.x.3.1 (new); 8.x.3.2 (new); 8.x.3.3 (new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Key Issue#17 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S6-231382 rev 1 corrects Table 8.2.X-1 to include AC KPIs (clause 8.2.3) instead of EAS KPIs (clause 8.2.5) S6-231382 rev 2: - Simplified clause 7.2.X changes and added an EN about the relationship between Application group ID and EAD IF or ACID. - Changed Application Group Profile from being provided within AC Profile to being provided alongside AC profile during provisioning and discovery requests. This impacts clauses: 8.2.2 (change to 8.2.2 no longer needed) 8.3.3.2 and 8.5.3.2 (new changes for each clause). It also impacts some of the descriptions in 8.3.3.2.2 and 8.5.2.2 - In clause 8.2.x EAS ID is made mandatory within the List of Common EAS criteria and an EN is added to clarify that it is FFS whether a Group ID can be associated with a single EAS ID. - Nomenclature alignement and editorials as needed. S6-231596 adds clarifications for CR relative to ECS-ER. It also clarifies the assumptions that Application Group ID is unique per EAS ID and can include a single ACID |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 6.6.2.4	ECS

The following cardinality rules apply for ECSs:

a)	One or more ECS(s) may be deployed to support one EDN;

b)	One ECS may be deployed to support one or more EDN(s);

c)	One or more ECS(s) may be deployed by a PLMN operator; and

d)	One or more ECS(s) may be deployed by an ECSP.

The following cardinality rules apply to the ECS that supports edge repository functionality and acts as an ECS-ER:

a)	One ECS-ER may be deployed by a PLMN operator; and

b)	One ECS-ER may be deployed by an ECSP.

The following cardinality rules apply to the ECS that supports central repository functionality:

a)	One ECS with Central Repository functionality  may be deployed by a PLMN operator.

* * * Next Change * * * *

### 6.3.4	Edge Configuration Server (ECS)

ECS provides supporting functions needed for the EEC to connect with an EES.

Functionalities of ECS are:

a)	provisioning of Edge configuration information to the EEC. The Edge configuration information includes the following:

1)	the information for the EEC to distinguish amongst the EESs (e.g. EDN service area); and

2)	the information for establishing a connection with EESs (such as URI);

b)	 providing  the T-EES information to the S-EES;

NOTE:	The ECS can be deployed in the MNO domain or can be deployed in 3rd party domain by service provider.

c)	supporting the functionalities of registration (i.e., registration, update, and de-registration) for the EES(s);

d)	supporting the functionalities of API invoker and API exposing function as specified in 3GPP TS 23.222 [6];

e)	interacting with 3GPP Core Network for accessing the capabilities of network functions either directly (e.g. via PCF) or indirectly (i.e. SCEF/NEF/SCEF+NEF).

f)	providing service provisioning information to a partner ECS; and

g)	retrieving service provisioning information from a partner ECS.

If the ECSP is required to support service provisioning for a certain EAS whose information is not within the preconfiguration or OAM configuration, the ECS can use Edge repository functions to support ECS discovery via ECS-ER. To support functions of the edge repository the ECS supports the following functionality(ies):

a)	registering and providing edge deployment information to ECS-ER.

To support functions of the edge repository the functionalities of the enhanced ECS i.e. ECS-ER, in addition to ECS functionalities, are:

a)	supporting the functionalities of registration (i.e., registration, update, and de-registration) for the ECS(s);

b)	receiving and storing information about edge computing resources from the ECS(s);

c)	receiving and storing information about edge computing resources from other ECS-ER(s); and

d)	providing information about Edge computing resources to other ECS-ER(s) of the federation.

An ECS can be enhanced to support discovery of a common EAS for a set of ACs. This functional enhancement is termed Central Repository. In addition to ECS functionalities, ECSs with Central Repository support the following functions:

a) 	storing and providing information for determining common EAS.

NOTE: The Central Repository functionality may be also implemented within ECS-ER.

Editor’s Note: It is FFS whether further alignment of the text is necessary to fully describe the case in which Central Repository is implemented within ECS-ER

* * * NEXT Change * * * *

### 7.2.X	Application group ID

Application group ID uniquely identifies a group of  UEs using the same application. It is allocated (either dynamically or pre-configured) by the ASP. How the ASP assigns the Application group ID is out of scope of this specification.

NOTE 1:  In this version of specification, Application group ID is assumed to be unique per EAS ID. Ensuring the uniqueness of the Application group ID per EAS ID is out of scope of this specification.

NOTE 2:  In this version of specification, an Application group is assumed to correspond to a single ACID.

* * * NEXT Change * * * *

### 8.2.X	Application group profile

An Application group profile includes information about the Application group used to determine common EAS.

Table 8.2.X-1: Application group profile

| Information element | Status | Description |
| --- | --- | --- |
| Application group ID | M | Identity of a group of UEs using the same application. |
| Expected Group Geographical Service Area. | O | The expected location(s) (e.g. route) of the group of UEs during the AC's operation schedule. This geographic information can express a geographic point, polygon, route, signalling map, or waypoint set. |
| List of Common EAS criteria (NOTE 1) | O | List of common EAS criteria required by the Application group. |
| > EASID | M | Identifier of the EAS |
| > Expected Application Group Service KPIs | O | KPIs (as shown in Table 8.2.3-1) expected in order to receive the required group services from the EAS for the entire application group. |
| > Minimum Application Group Service KPIs | O | Minimum KPIs (as shown in Table 8.2.3-1) required in order to receive meaningful group services from the EAS, for the entire application group. |
| NOTE 1: When the Application group profile is provisioned to the ACs, it is assumed that this IE is present only if the EAS ID and/or the EAS KPIs are different than those required for providing application services for the individual AC, without a group. Otherwise, this IE shall be absent from provisioning. |  |  |

* * * NEXT Change * * * *

##### 8.3.3.2.2	Request-response model

Figure 8.3.3.2.2-1 illustrates service provisioning procedure based on request/response model.

Pre-conditions:

1.	The EEC has been pre-configured or has discovered the address (e.g. URI) of the ECS;

2.	The EEC has been authorized to communicate with the ECS;

3.	The UE Identifier is either preconfigured or resulted from a successful authorization; and

4.	The ECS is configured with ECSP's policy for service provisioning.

NOTE 1:	Details of ECSP's policy are out of scope.

Figure 8.3.3.2.2-1: Service provisioning – Request/Response

1.	The EEC sends a service provisioning request to the ECS. The service provisioning request includes the security credentials of the EEC received during EEC authorization procedure and may include the UE identifier such as GPSI, connectivity information, UE location, EEC service continuity support and AC profile(s) information. EEC may provide its desired ECSP identifier(s) in the service provisioning request based on EEC preference.

2.	Upon receiving the request, the ECS performs an authorization check to verify whether the EEC has authorization to perform the operation. The ECS may utilize the capabilities (e.g. UE location) of the 3GPP core network as specified in clause 8.10.2. If the UE serving PLMN identifier is not provided by the EEC in the connectivity information of the service provisioning request, the ECS may invoke the NEF monitoring event API as described in 3GPP TS 23.502 [43] and 3GPP TS 23.682 [17] to obtain the UE roaming status and serving PLMN identifier. If the UE is roaming, the ECS may use the serving PLMN identifier to determine the roaming partner ECS (i.e. V-ECS) information to be provided to the EEC in the service provisioning response. If AC profile(s) are provided by the EEC, the ECS identifies the EES(s) based on the provided AC profile(s) and the UE location.

When one or more Application group profiles are provided in the request, the ECS processing to determine the corresponding EESs is as follows:

-  If the Central Repository functionality is supported and EES information corresponding to the Application group ID is available, then the ECS uses the Application group ID to retrieve the list of one or more common EESs stored in the Central Repository.

-  If the Central Repository functionality is not supported or EES information corresponding to the Application group ID is not available at the Central Repository, then the ECS determines EESs corresponding to the Application group Profile based on other request parameters present, e.g. group service area. If EESs corresponding to the Application group Profile cannot be determined, the ECS proceeds using the remaining information in the request to identify corresponding EESs.

NOTE 2: Determination of common EES(s) is subject to local policies and the ECS implementation. The local policies or the ECS implementation when Central Repository functionality is not supported may allow caching and reuse of common EES information provided to previous service provisioning requests.

When AC profiles(s) are not provided, then:

-	if available, the ECS identifies the EES(s) based on the UE-specific service information at the ECS and the UE location;

-    ECS identifies the EES(s) by applying the ECSP policy (e.g. based only on the UE location);

NOTE 3:	Details of the UE-specific service information and how it is available at the ECS is out of scope.

NOTE 4:	Both steps are evaluated prior to sending a response.

If desired ECSP identifier(s) is received, the ECS identifies the EES(s) based on registered ECSP identifier in EES profile.

NOTE 5:	For EEC desired ECSP identifier usage, it is assumed that the ECSP providing the EES and PLMN operator are the same organization and an ECSP providing the EES (desired by the EEC) registers its EES in ECS provided by another ECSP based on service agreement to provide services to EEC.

The ECS also determines other information that needs to be provisioned, e.g. identification of the EDN, EDN service area, EES endpoints.

If ECS does not identify any suitable EES(s) based on EDN configuration available at the ECS and UE's location, the ECS determines a partner ECS that may satisfy the requirements. Based on ECSP policy, the ECS may use preconfigured or OAM configured information about the partner ECSs or ECS discovery via ECS-ER as specified in clause 8.17.2.3 or both.

If required by the ECSP policies, the ECS may use service provisioning information retrieval procedure as specified in clause 8.17.2.4 to obtain service provisioning information from the partner ECS.

When the bundle EAS information is provided, then;

-	If bundle EAS information includes EAS bundle identifier, the ECS identifies all the EES(s) providing the same EAS bundle identifier.

-	If bundle EAS information includes a list of EASIDs, the ECS identifies the one or more EES which support all of the EASs within the same DNAI based on the EES DNAI information obtained in the EES profile.

3.	If the processing of the request was successful, the ECS responds to the EEC's request with a service provisioning response. If the ECS has identified the relevant EES(s) information, the service provisioning response includes a list of EDN configuration information, e.g. identification of the EDN, EDN service area, and the required information (e.g. URI, IP address) for establishing a connection to the EES. When an identified EES is based on Application group profile information included in the request, then the Application group ID is included in the response message with the corresponding EES(s).

The ECS may provide associated EES(s) information (one or more EES information) in the service provisioning response along with the bundle EAS information.

If the alternative ECS(s) has been identified in step 2, the ECS sends a successful response including the Redirect information element containing the list of ECS(s) configuration information indicating that alternative ECS(s) is available for service provisioning request. The response may include information such as DNN and S-NSSAI for roaming UEs to establish a PDU session with the ECS as specified in 3GPP TS 23.548 [20].

If the ECS is not provisioned with any EDN configuration information or is unable to determine either the EES information or the partner ECS information using the inputs in service provisioning request, UE-specific service information at the ECS or the ECSP's policy, the ECS shall reject the service provisioning request and respond with an appropriate failure cause.

If the service provisioning response contains a list of ECS configuration information, the EEC may initiate service provisioning procedure with one or more ECS(s) provided in the response. If the UE is roaming to a V-PLMN and the ECS configuration information includes V-PLMN ID in the list of Supported PLMN ID(s), the EEC establishes a connection with the V-ECS as specified in 3GPP TS 23.548 [20]. The connection with the V-ECS can be a HR-SBO PDU session or an LBO PDU session based on the information received from the ECS.

If the EDN configuration information includes an LADN DNN as an identifier for the EDN, the EEC considers the LADN as the EDN. Therefore, the service area of EDN is the LADN Service Area which can be discovered using the UE Registration Procedure.

The EEC may cache the service provisioning information (e.g. EES endpoint) for subsequent use and avoid the need to repeat step 1. If the Lifetime IE is included in the Service provisioning response, then the EEC may cache and reuse the Service provisioning information only for the duration specified by the Lifetime IE, without the need to repeat step 1.

If the ECS provided information regarding the service continuity support of individual EESs, the EEC may take this information into account when selecting an EES for EEC registration, EAS discovery or T-EAS discovery, respectively.

If for multiple EES(s), the instantiable EAS information IE for an EAS is not available or the instantiable EAS information IE is set to instantiated or instantiable, the EEC can select one or more such EES to perform EAS discovery. For EAS discovery to mitigate the waste of EDN resources EEC considers the instantiable EAS information and the associated instantiation criteria, the EEC selects one EES, if the EAS instantiation status corresponding to the EASID requested by AC/EEC is instantiable but not yet instantiated (i.e., no instantiated EAS).

NOTE 5:	If the service provisioning request fails, the EEC can resend the service provisioning request again, taking into account the received failure cause.

NOTE 6:	Even after the EEC establishes a connection to the EES using information received in step 3, the EES can issue AF request to influence traffic routing from EEC to EES as specified in 3GPP TS 23.501 [2] clause 5.6.7.

NOTE 7:	If the EAS instantiation fails based on the selected EES, the EEC may retry the EAS discovery request to another EES (e.g. selecting another one EES based on the instantiable EAS information).

* * * NEXT Change * * * *

##### 8.3.3.3.2	Service provisioning request

Table 8.3.3.3.2-1 describes the information elements for service provisioning request from the EEC to the ECS.

Table 8.3.3.3.2-1: Service provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| EECID | M | Unique identifier of the EEC. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| AC Profile(s) | O | Information about services the EEC wants to connect to, as described in Table 8.2.2-1. |
| > List of Application group profiles | O | List of Application group profile information as defined in Table 8.2.x -1 |
| EEC Service Continuity Support | O | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC. When requesting service provisioning for T-EES discovery, if the EEC requires that T-EES must support "EEC executed ACR via T-EES" scenario, then EEC includes only "EEC executed ACR via T-EES" in this IE |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Connectivity information | O | List of connectivity information for the UE, e.g. PLMN ID, SSID. |
| UE location | O | The location information of the UE. The UE location is described in clause 7.3.2. |
| ECSP identifiers | O | The list of desired EES provider IDs, by the EEC. |

Editor's Note: it is FFS how EEC context pull request will work if S-EES does not support "EEC executed ACR via T-EES" scenario.

Editor's Note: [SA3] Whether the EECID and the UE ID included in request of EDGE-1 & 4 interactions is part of the security credential is SA3's responsibility.

* * * NEXT Change * * * *

##### 8.3.3.3.3	Service provisioning response

Table 8.3.3.3.3-1 describes the information elements for service provisioning response from the ECS to the EEC.

Table 8.3.3.3.3-1: Service provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the service provisioning request was successful. |
| > List of EDN configuration information | M | List of EDN configuration information as defined in Table 8.3.3.3.3-2. |
| Failure response | O | Indicates that the service provisioning request failed. |
| > Cause | O | Indicates the cause of service provisioning request failure. |
| Redirect | O | Indicates redirection to (an)other ECS(s). |
| > ECS(s) information | M | Endpoint address of ECS(s) to which the UE is redirected for service provisioning. |
| > DNN | O | DNN required for establishing PDU Session to the redirected ECS |
| > S-NSSAI | O | S-NSSAI required for establishing PDU Session to the redirected ECS |

Table 8.3.3.3.3-2: EDN configuration information

| Information element | Status | Description |
| --- | --- | --- |
| EDN connection information (see NOTE 1) | M | Information required by the UE to establish connection with the EDN. |
| > DNN/APN | M | Data Network Name/Access Point Name |
| > S-NSSAI | O | Network Slice information |
| > EDN Topological Service Area | O | The EDN serves UEs that are connected to the Core Network from one of the cells included in this service area. See possible formats in Table 8.2.7-1. |
| List of EESs | M | List of EESs of the EDN. |
| > EESID | M | The identifier of the EES |
| > EES Endpoint | M | The endpoint address (e.g. URI, IP address) of the EES |
| > List of Application group IDs | O | List of Application group IDs of the groups the EES serves as common EES. |
| > EASIDs (see NOTE 2) | O | List of EASID registered or expected to be registered with the EES. |
| >> List of EAS bundle information | O | List of EAS bundles to which the EAS belongs and related bundling requirements. |
| >>> Bundle ID or list of EASID | M | A list of EASIDs or a bundle ID as described in clause 7.2.10. |
| > Instantiable EAS information | O | The EAS instantiation status per EASID (e.g. instantiated, instantiable but not be instantiated yet) |
| >> Instantiation criteria (see NOTE 3) | O | The criteria upon which EAS can be instantiated (e.g. based on specific date and time). |
| > ECSP ID | O | The identifier of the ECSP that provides the EES. |
| > EES Topological Service Area | O | The EES serves UEs that are connected to the Core Network from one of the cells included in this service area. EECs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| > EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| > List of EES DNAI(s) | O | DNAI(s) associated with the EES/EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. |
| > EES Service continuity support | O | Indicates if the EES supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EES. |
| > EEC registration configuration | M | Indicates whether the EEC is required to register on the EES to use edge services or not. |
| > Security Credential | O | Indicates the security credential sent by the ECS. The security credential is used by EEC to communicate with the EES as specified in 3GPP TS 33.558 [23], clause 6.2. |
| Lifetime | O | Time duration for which the EDN configuration information is valid and supposed to be cached in the EEC. |
| NOTE 1: If the UE is provisioned or pre-configured with URSP rules by the HPLMN or serving SNPN, the UE handles the precedence between EDN connection info and URSP rules as defined in 3GPP TS 23.503 [12] clause 6.1.2.2.1. EDN connection info is considered to be part of UE Local Configurations. NOTE 2: EAS information is limited to the EEC requested applications. If no AC profiles were present in the service provisioning request, the EAS information is subject to the ECSP policy (e.g. no EAS information or a subset of EAS information related to the EES). NOTE 3: "Instantiation criteria" IE shall be present only when the value of "Instantiable EAS information" IE is "instantiable but not be instantiated yet". |  |  |

* * * NEXT Change * * * *

#### 8.5.2.2	Request-response model

Pre-conditions:

1.	The EEC has received information (e.g. URI, IP address) related to the EES;

2.	The EEC has received appropriate security credentials authorizing it to communicate with the EES as specified in clause 8.11; and

3.	The EES is configured with ECSP's policy for EAS discovery.

NOTE 1:	Details of ECSP's policy are out of scope.

Figure 8.5.2.2-1: EAS Discovery procedure

1.	The EEC sends an EAS discovery request to the EES. The EAS discovery request includes the requestor identifier [EECID] along with the security credentials and may include EAS discovery filters and may also include UE location to retrieve information about particular EAS(s) or a category of EASs, e.g. gaming applications, or Edge Applications Server(s) available in certain service areas, e.g. available on a UE's predicted or expected route. The request may include an EAS selection request indicator or UE type.

2.	Upon receiving the request from the EEC, the EES checks if the EEC is authorized to discover the requested EAS(s). The authorization check may apply to an individual EAS, a category of EASs or to the EDN, i.e. to all the EASs. If UE's location information is not already available, the EES obtains the UE location by utilizing the capabilities of the 3GPP core network as specified in clause 8.10.3. For the  EAS discovery filters provided by the EEC not containing Application group profile, the EES identifies the EAS(s) based on the provided EAS discovery filters and the UE location.

When the bundle EAS information is provided, then;

-	If bundle EAS information includes EAS bundle identifier, the EES identifies all or part of the EAS(s) associated the same EAS bundle identifier.

-	If bundle EAS information includes a list of EASIDs, the EES identifies the EASs which are all or part of the EAS bundle.

If the EEC indicates that service continuity support is required, the EES shall take the indication which ACR scenarios are supported by the AC and the EEC and which of these are preferred by the AC into consideration. The EES may select one EAS and determine whether to perform application traffic influence for this AC based on AC's service KPI or EAS’s service KPI in desired response time, when the EAS does not perform traffic influence in advance.

If the List of AC characteristics is provided and includes Application group profiles, the EES checks whether it already serves the corresponding Application group IDs. If the EES already serves the Application group ID, then it provides the Common EAS already in use as result corresponding to the Application group profile filter. If the EES does not already serve the Application group ID, the EES identifies common EAS(s) based on the other IEs  in the Application group profile, e.g. Expected Group Geographical Service Area . If the Application group profile does not include the List of Common EAS criteria IE, the EES determines Common EAS based on local policies.

NOTE 2:	Determination of common EAS(s) is subject to local policies which may allow reuse of common EAS information provided to previous discovery requests including the requested Application group ID.

NOTE 3: This version of specification does not detail whether and how Common EAS information which  may have been previously received via common EAS announcement procedures is used in determining EAS discovery results.

When EAS discovery filters are not provided, then:

-	if available, the EES identifies the EAS(s) based on the UE-specific service information at the EES and the UE location;

-	EES identifies the EAS(s) by applying the ECSP policy (e.g. based only on the UE location);

NOTE 2:	Details of the UE-specific service information and how it is available at the EES is out of scope.

NOTE 3:	Both steps are evaluated prior to sending a response.

Upon receiving the request from the EEC, if the EEC does not indicate EAS Instantiation Triggering Suppress in the EAS Discovery request, the EES may trigger the EAS management system to instantiate the EAS that matches with EAS discovery filter IEs (e.g. ACID) as in clause 8.12.

Otherwise, upon receiving the request from the EEC, if the EEC indicates EAS Instantiation Triggering Suppress in the EAS Discovery request and the EES supports such capability, the EES may determine Instantiable EAS Information for EAS(s) that are instantiable but not yet instantiated and match the EAS discovery filter IEs. Instantiable EAS Information is provided in the EAS Discovery response and includes the EASID(s) and, for each EASID, the status indicating whether the EAS is instantiated or instantiable but not yet instantiated.

If the EEC provides in the EAS discovery request the EAS selection request indicator, the EES selects EAS satisfying the EAS discovery filter or based on other information (e.g. ECSP policy) as described above (if no EAS discovery filter received), and then provides the selected EAS information to the EEC in the discovered EAS list of EAS discovery response.

NOTE 4:	Without EAS selection request indication, the EES handling is as per R17 procedure.

3.	If the processing of the request was successful, the EES sends an EAS discovery response to the EEC, which includes information about the discovered EASs and Instantiable EAS Information. For discovered EASs, this includes endpoint information. Depending on the EAS discovery filters received in the EAS discovery request, the response may include additional information regarding matched capabilities, e.g. service permissions levels, KPIs, AC locations(s) that the EASs can support, ACR scenarios supported by the EAS, etc. The EAS discovery response may contain a list of EASs and Instantiable EAS Information. This list may be based on EAS discovery filters containing a Geographical or Topological Service Area, e.g. a route, included in the EAS discovery request by the EEC. When the discovered EAS is based on Application group profile information included in the request, then the Application group ID is included in the response message with the corresponding EAS.

If the EES is unable to determine the EAS information using the inputs in the EAS discovery request, UE-specific service information at the EES or the ECSP policy, the EES shall reject the EAS discovery request and respond with an appropriate failure cause.

If the EEC is not registered with the EES, and ECSP policy requires the EEC to perform EEC registration prior to EAS discovery, the EES shall include an appropriate failure cause in the EAS discovery response indicating that EEC registration is required.

If the UE location and predicted/expected UE locations, provided in the EAS discovery request, are outside the Geographical or Topological Service Area of an EAS, then the EES shall not include that EAS in the discovery response. The discovery response may include EAS(s) that cannot serve the UE at its current location if a predicted/expected UE location was provided in the EAS discovery request.

Upon receiving the EAS discovery response, if the EEC selects an EAS which is instantiated (i.e., an EAS profile was provided), the EEC uses the endpoint information for routing of the outgoing application data traffic to EAS(s), as needed, and may provide necessary notifications to the AC(s). The EEC may use the border or overlap between EAS Geographical Service Areas for service continuity purposes. The EEC may cache the EAS information (e.g. EAS endpoint) for subsequent use and avoid the need to repeat step 1. If the Lifetime IE is included in the response, the EEC may cache the EAS information only for the duration specified by the Lifetime IE.

Upon receiving the EAS discovery response, if the EEC selects an EAS which is not instantiated (i.e. an EAS profile is not provided), the EEC sends the EAS information provisioning request indicating the selected EASID as in clause 8.15.

NOTE 5:	Within the duration specified by the Lifetime IE, the cached EAS Profile can be updated (e.g. according to notifications from the EES for changes of EAS information due to EAS status change) or the cached EAS Profile can be invalidated due to new EAS information discovery (e.g. due to UE mobility). The EEC can update or invalidate the cached EAS information (e.g. on PDU Session Release or Modification Command).

NOTE 6:	The AC can cache the EAS information (e.g. EAS endpoint) for subsequent use. In the case of the cached information needing to be updated or invalidated, the mechanisms for the EEC to notify the AC is up to implementation and is not specified in the current release of the present document.

NOTE 7:	The EEC can use the EAS information provided by the discovery procedure to perform service continuity planning, for example when ultra-low latency ACR is required.

If the EAS discovery request fails, the EEC may resend the EAS discovery request, taking into account the received failure cause. If the failure cause indicated that EEC registration is required, the EEC shall perform an EEC registration before resending the EAS discovery request.

NOTE 8:	As long as a proper EAS (e.g. considering expected AC service KPIs included in EAS discovery request) is discovered and selected by the EES, EEC of a constraint UE can stop sending EAS discovery to rest candidate EES(s), and provide the selected EAS information to AC.

* * * NEXT Change * * * *

#### 8.5.3.2	EAS discovery request

Table 8.5.3.2-1 describes information elements for the EAS discovery request. Table 8.5.3.2-2 provides further detail about the EAS Discovery Filter information element.

Table 8.5.3.2-1: EAS discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The ID of the requestor (e.g. EECID) |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EAS discovery filters | O | Set of characteristics to determine required EASs, as detailed in Table 8.5.3.2-2. |
| UE location | O | The location information of the UE. The UE location is described in clause 7.3.2. |
| Serving MNO information (NOTE 2) | O | The serving MNO information (e.g. MNO name, PLMN ID) which is serving the subscriber. |
| Target DNAI (NOTE 1) | O | Target DNAI information which can be associated with potential T-EAS(s) |
| EEC Service Continuity Support | O | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC or, if this message is sent by the EEC to discover a T-EAS, which ACR scenario(s) are intended to be used for the ACR. |
| EES Service Continuity Support (NOTE 1) | O | The IE indicates if the S-EES supports service continuity or not. The IE also indicates which ACR scenarios are supported by the S-EES or, if the EAS discovery is used for an S-EES executed ACR according to clause 8.8.2.5, which ACR scenario is to be used for the ACR. |
| EAS Service Continuity Support (NOTE 1) | O | The IE indicates if the S-EAS supports service continuity or not. The IE also indicates which ACR scenarios are supported by the S-EAS or, if the EAS discovery is used for an S-EAS decided ACR according to clause 8.8.2.4, which ACR scenario is to be used for the ACR. |
| EAS Instantiation Triggering Suppress | O | Indicates to the EES that EAS instantiation triggering should not be performed for the current request. |
| EAS selection request indicator | O | Indicates the request for EAS selection support from the EES (e.g., for constrained device). |
| NOTE 1: This IE shall not be included when the request originates from the EEC. NOTE 2: This IE shall be included if edge node sharing is used. |  |  |

Table 8.5.3.2-2: EAS discovery filters

| Information element | Status | Description |
| --- | --- | --- |
| List of AC characteristics (NOTE 1) | O | Describes the ACs for which a matching EAS is needed. |
| > AC profile (NOTE 2) | M | AC profile containing parameters used to determine matching EAS. AC profiles are further described in Table 8.2.2-1. |
| >> Application group profile | O | Application group profile as described in Table 8.2.X-1 |
| List of EAS characteristics (NOTE 1, NOTE 3) | O | Describes the characteristic of required EASs. |
| > EASID | O | Identifier of the required EAS. |
| > Bundle ID or list of EASID | O | A list of EASIDs or a bundle ID as described in clause 7.2.10. |
| > EAS provider identifier | O | Identifier of the required EAS provider |
| > EAS type | O | The category or type of required EAS (e.g. V2X) |
| > EAS schedule | O | Required availability schedule of the EAS (e.g. time windows) |
| > EAS Geographical Service Area | O | Location(s) (e.g. geographical area, route) where the EAS service should be available. |
| > EAS Topological Service Area | O | Topological area (e.g. cell ID, TAI) for which the EAS service should be available. See possible formats in Table 8.2.7-1. |
| > Service continuity support | O | Indicates if the service continuity support is required or not. |
| > Service permission level | O | Required level of service permissions e.g. trial, gold-class |
| > Service feature(s) | O | Required service features e.g. single vs. multi-player gaming service |
| NOTE 1: Either "List of AC characteristics" or "List of EAS characteristics" shall be present. NOTE 2: "Preferred ECSP list" IE shall not be present. NOTE 3: The "List of EAS characteristics" IE must include at least one optional IE, if used as an EAS discovery filter. |  |  |

* * * NEXT Change * * * *

## 8.15	EAS Information provisioning

### 8.15.1	General

EAS information provisioning procedure allows the EEC to exchange information with the EES about selected EAS or ACR scenario selection.

When service continuity is required, service continuity scenarios may be combined to perform ACR detection in one or more of the EEC, the EES and the EAS; the related procedures are specified in clauses 8.15.2, 8.6.3 and referred to in clause 8.8.2. The selection of ACR scenario(s) may be performed by the EEC or the EES for a given AC and the selected EAS from the common supported ACR scenarios of AC, EEC, selected EES and selected EAS. The selection of ACR scenario(s) for each EAS in EAS bundles may be performed separately by the EEC or the EES for a given AC and the selected EAS(s) in a bundle, based on the service continuity support by AC, EEC, EES and EAS(s). In addition, the EEC or EES can further determine the one or more ACR scenario(s) for the selected EAS based on the AC service KPI.

Editor's Note:	It is FFS whether the ACR scenario selection for EAS bundles is needed to extend to multiple EES scenario, depending on the feedback from SA5.

NOTE:	How to select ACR scenario(s) at the EEC or EES considering AC service KPI is implementation specific.

Information about instantiable EAS may be provided to the EEC as in clause 8.3.3 and 8.5. Triggering the instantiation of an EAS by the EEC may be announced to the EES by including the selected EASID in the EAS information provisioning request without including the selected EAS endpoint.

The EAS information provisioning request types supported are:

-	"ACR scenario selection announcement". Inform the EES about the EAS that has been selected by the EEC and may provide the selected ACR scenario list to the EES. For the EAS bundles scenario, the selected bundled EAS(s) and the selected ACR scenario list for EAS bundles by EEC are included and notified to the EES.

-	"ACR scenario selection request". Inform the EES to perform ACR scenario selection. For the EAS bundles scenario, the request may inform the EES to determine the ACR scenario list for EAS bundles.

- 	“EAS selection”. Inform the EES of EAS selection and start of application services.

### 8.15.2	Procedure

#### 8.15.2.1	General

#### 8.15.2.2	EAS Information provisioning

Pre-conditions:

1.	The EEC has performed service provisioning procedure

2.	The EEC has performed the EAS discovery procedure

Figure 8.15.2.2-1: EAS information provisioning

1.	The EEC sends the EAS information provisioning request to the EES:

a-	"ACR scenario selection announcement". The request may include ACR scenario list selected by the EEC, EEC security credentials, selected EASID, selected EAS endpoint, EECID and ACID. For the EAS bundles scenario, the request may include the ACR scenario list for EAS bundles selected by the EEC.

b-	"ACR scenario selection request". The request may include AC profile, EEC service continuity support, EEC security credentials, EECID and ACID.

c- " EAS selection". Informs the EES of EAS selection. If the selected EAS is for Common EAS services, the request includes the corresponding Application group ID and may contain a list of EESs received in the service provisioning response message.

If the EEC has selected an uninstantiated EAS, the EEC includes the selected EASID without including the selected EAS endpoint in the request.

2.	Upon receiving the request from the EEC, the EES validates the EEC information request and verifies if the EEC is authorized for this operation.

a-	"ACR scenario selection announcement". The EES may send the ACR Selection notification to the selected EAS if the EAS has subscribed and if EES allows EEC based ACR scenario selection. Otherwise, EES may respond with status failure and include appropriate reason. For the EAS bundles scenario, the EES may send the ACR selection notification to the bundled EAS(s).

b-	"ACR scenario selection request". The EES selects the ACR scenario list and may send the ACR Selection notification to the selected EAS if the EAS has subscribed. The EES may include the ACR scenario list in the EAS information provisioning response. For the EAS bundles scenario, the EES selects the ACR scenario list for EAS bundles based on the AC/EEC/EES/EAS(s) service continuity support, and sends the ACR scenario list to the bundled EAS(s).

c- " EAS selection". The EES determines that the EAS is providing services to the AC with the indicated ACID.

If the request contains selected EAS ID and selected EAS Endpoint, the EES may apply the EAS traffic influence with the N6 routing information of the EAS in the 3GPP Core Network, based on application KPIs and if the EAS traffic influence was not done before (e.g. neither in EAS discovery procedure nor the EAS perform traffic influence).

If the request is for selected EAS and contains Application group ID, the EES  considers itself as common EES for the corresponding Application group. For each Application group it services, the EES maintains locally an association between its own EES ID and the Application group ID, i.e. the (EES ID, Application group ID) tuple, until it determines that the Common EAS no longer provides services to any member of this group.

If the ECS provides Common Repository support, the EES provides the (EES ID, Application group ID) tuple to inform the ECS that it serves the corresponding Application group.

NOTE 1: The EES procedure to inform the ECS that it serves the Application group does not need to be performed if the Application group ID has been previously associated with the EES ID, e.g. for other group members.

Editor’s Note: How the EES determines whether ECS provides Common Repository support is FFS.

Editor’s Note: How the EES informs ECS that it serves an Application group ID is FFS. Proposals for using the EES registration update have been discussed and are FFS.

If the ECS does not provide Common Repository support, and based on local policies, the EES may perform the common EAS announce procedure as detailed in clause 8.X.using the list of EESs provided in the request. If the request message does not contain a list of EESs for a certain Application group ID, then the EES determines the other EESs to which to announce the common EAS.

Editor’s Note: It is FFS whether clause 8.8.3.3 of TS 23.558 can be used to determine target EESs for Common EAS announcement if the message does not include a list of EESs for a group.

Editor’s Note: It is FFS how disassociation between EES and Application group ID.is distributed from the EES to ECS or other EESs to which it was announced.

If the request contains the selected EASID and the selected EAS endpoint is not included, the EES verifies if instantiation of EAS is needed and may trigger the EAS management system to instantiate the EAS as in clause 8.12.

When the request contains EEC Service Continuity Support IE and the EEC context has been established, the EES includes the IE into the EEC context described in Table 8.2.8-1.

NOTE 1:	EES can also influence the EAS traffic in advance.

NOTE 2:	It is up to the AC to decide when to connect to the selected EAS (either immediately or wait for a while) once the AC knows the selected EAS.

3.	If the processing of the request was successful, the EES sends an EAS information provisioning response to the EEC indicating a successful status. If an EEC context has been established, the response also includes the list of selected ACR scenario(s) into the session context IE within EEC context as described in Table 8.2.8-2; otherwise, the EES shall indicate a failure status and include appropriate reasons. If the EES has triggered EAS instantiation based on the EAS information provisioning request and obtained the newly instantiated EAS information, the response contains information about the newly instantiated EAS, including the EAS endpoint information.

The EEC, EES and EAS (or the bundled EAS(s)) use the selected ACR scenario list to determine if they should perform ACR detection and/or ACR decision.

Upon receiving the EAS information provisioning response, if the response includes instantiated EAS information, the EEC uses the endpoint information to subscribe to ACR event notification, as needed, and provides necessary notifications to the AC.

NOTE 3:	Other ACR selection criteria are out of scope of the current specification.

NOTE 4:	The common supported ACR scenarios is decided as part of the EAS discovery and selection procedure.

* * * NEXT Change * * * *

#### 8.15.3.2	EAS information provisioning request

Table 8.15.3.2-1 describes the information elements for EAS information provisioning request from the EEC to the EES.

Table 8.15.3.2-1: EAS information provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| EECID | M | The identifier of the EEC. |
| ACID | M | The identifier of the AC. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Selected EAS ID(s) | O | The identifier(s) of the selected EAS or the selected EAS(s) for EAS bundles, which is either instantiated or instantiable. |
| Selected EAS Endpoint(s) | O | The endpoint(s) of the selected EAS or the selected EAS(s) for EAS bundles |
| Request type | O | Request types: - ACR scenario selection announcement - ACR scenario selection request - EAS selection |
| Selected ACR scenario list (see NOTE 1) | O | The list of ACR scenarios (or the list of ACR scenarios for EAS bundles) selected by the EEC |
| AC Profile (see NOTE 2, NOTE 3) | O | AC Profile as described in Table 8.2.2-1 |
| Application group ID (NOTE 4) | O | Identity of the Application group of associated with the common EAS. |
| > List of EES(s) | O | List of EES information (e.g. address information) corresponding to the Application group ID, as provided to the EEC by ECS during service provisioning. The members of the list may be used by EES as targets for common EAS announcements. |
| EEC Service Continuity Support (NOTE 2) | O | Indicates if the EEC supports service continuity or not. The IE indicates which ACR scenarios are supported by the EEC, also indicates the EEC ability of handling bundled EAS ACR. |
| NOTE 1: The IE may be present only if Selected EASID(s) and Selected EAS Endpoint(s) are present and Request type is "ACR scenario selection announcement" NOTE 2: The IEs are present only if request type is “ACR scenario selection request” NOTE 3: The IE is present if AC Profile is not shared to EES previously NOTE 4: This IE may be present only if the request type is “EAS selection”. |  |  |

* * * NEXT Change * * * *

## 8.X	Common EAS announcement

### 8.X.1	General

Common EAS announcement procedure enables an EES to exchange selected common EAS information with other EES(s).

### 8.X.2	Procedure

Pre-conditions:

1.	The announcing EES may be pre-configured with the information for communicating with the receiving EES, which belongs to the same EDN; or

2.	The announcing EES may receive the list of EES information (e.g. address information) from an EEC during the EAS information provisioning procedure.

Figure 8.X.2-1: Announce common EAS procedure

- The announcing EES sends Announce common EAS request message to the receiving EES. The request message includes the requestor identifier [EESID], the security credentials and the selected common EAS information, along with Application group information.

- Upon receiving the request from the announcing EES, the receiving EES checks if the announcing EES is authorized to provide the selected common EAS. The receiving EES stores the EES ID of the announcing EES and the received common EAS information along with Application group information.

- If the processing of the request was successful, the receiving EES sends Announce common EAS response to the announcing EES indicating a successful status; otherwise, the receiving EES shall indicate a failure status and include an appropriate cause.

* * * NEXT Change * * * *

### 8.X.3	Information flows

#### 8.X.3.1	General

The information flows are specified for Announce common EAS request and response.

#### 8.X.3.2	Announce common EAS request

Table 8.X.3.2-1 describes the information elements for Announce common EAS request from the announcing EES to the receiving EES.

Table 8.Y.3.2-1: Announce common EAS request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the announcing EES. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Selected common EAS ID | M | The identifier of the Selected Common EAS |
| Application group ID | M | Identity of the Application group associated with the common EAS. |

Editor's note: It is FFS whether and how the receiving EES redirects the EEC to the announcing EES, as well as whether and how it directly provides the common EAS details.

* * * NEXT Change * * * *

#### 8.X.3.3	Announce common EAS response

Table 8.X.3.2-1 describes the information elements for Announce common EAS response from the receiving EES to the announcing EES.

Table 8.X.3.2-1: Announce common EAS response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the request was successful. |
| Failure response | O | Indicates that the request has failed. |
| Cause | O | Indicates the failure cause. Only included when Failure Response is included. |

* * * END of Changes * * * *