3GPP TSG-SA WG6 Meeting #54-e	S6-231601

17th – 26th April 2023	(revision of S6-231413)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.255 | CR | 0047 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Unsubscribe UAV dynamic information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UASAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The unsubscription procedure for host UAV dynamic information is missing. UASS should be expanded to UAS Application Specific Server in text and figures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the unsubscription procedure and fixed the UASS terminology issue in text and figures |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature will not be complete and stage 3 specification will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.8.2.x (new), 7.8.3.x (new), 7.8.3.y (new), 7.8.2.1, 7.8.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Fixed UASS to UAS Application Specific Server in clauses 7.8.2.x, 7.8.2.1 and 7.8.2.4 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

7.8.2.x	Unsubscription for host UAV dynamic information

Figure 7.8.2.x-1 describes the procedure for unsubscription for host UAV’s dynamic information.

Pre-condition:

-	The UAS Application Specific Server or UAE client of the host UAV have performed the subscription procedure as specified in clause 7.8.2.1

Figure 7.8.2.x-1: Unsubscription for host UAV dynamic information

1.	The UAS Application Specific Server or UAE client of host UAV sends a unsubscribe host UAV dynamic information request to the UAE server 1. The request includes subscription ID.

2.	The UAE server 1 removes the previously stored subscription information.

3.	The UAE server 1 sends unsubscription response to the UAS Application Specific Server.

4.	The UAE server 1 stops tracking the host UAV location from the location management server 1 as specified in 3GPP TS 23.434 [5].

* * * Next Change * * * *

7.8.3.x	Unsubscribe host UAV dynamic information request

Table 7.8.3.x-1 describes the information flow for a UAS application specific server to unsubscribe the host UAV dynamic information at the UAE server.

Table 7.8.3.x-1: Unsubscribe host UAV dynamic information request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID (NOTE) | M | Identifier of a successful subscription. |

7.8.3.y	Unsubscribe host UAV dynamic information response

Table 7.8.3.y-1 describes the information flow for UAE server to respond for host UAV dynamic unsubscription request from the UAS application specific server.

Table 7.8.3.y-1: Unsubscribe host UAV dynamic information response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result from the UAE server in response to unsubscription request indicating success or failure |

* * * Next Change * * * *

7.8.2.1	Subscription for host UAV dynamic information

Figure 7.8.2.1-1 describes the procedure for subscription for host UAV’s dynamic information.

Pre-condition:

-	UAS Application Specific Server has registered with UAE server 1 which is responsible for the host UAV.

-	The UAV ID and application defined proximity range information are configured on the host UAV.

Figure 7.8.2.1-1: Subscription for host UAV dynamic information

1.	The UAS Application Specific Server or UAE client of host UAV sends a subscribe host UAV dynamic information request to the UAE server 1. The request includes the UAV ID of the host UAV, application defined proximity range information.

2.	The UAE server 1 stores the subscription information.

3.	The UAE server 1 sends subscription response to the UAS Application Specific Server.

4.	The UAE server 1 obtains and initiates tracking the host UAV location from the location management server 1 as specified in 3GPP TS 23.434 [5].

* * * Next Change * * * *

7.8.2.4	Notification of host UAV dynamic information

Pre-conditions:

-	UAS Application Specific Server has performed subscription as per procedure in clause 7.8.2.1 with UAE server 1.

-	UAE server 1 has prepared the host UAV dynamic information as per procedure in clause 7.8.2.3.3.

Figure 7.8.2.4-1: Notification for host UAV dynamic information

1.	The UAE server 1 sends notification of host UAV dynamic information to the subscribed entity (i.e. UAS Application Specific Server and/or to the subscribed UAE client of the host UAV). The notification includes the aggregated information of all the UEs in the application defined proximity range of the host UAV and the location of the host UAV.

2.	The UAS Application Specific Server or the UAE client of the host UAV updates the host UAV dynamic information with the host UAV dynamic information received in step 1. The UAE client provides the host UAV dynamic information to the UAS Client.