3GPP TSG-SA WG6 Meeting #54-e	S6-231615

17th – 26th April 2023	 (revision of S6-231592 Rev2 was S6-231412)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0215 | rev | 2 | Current version: | 18.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Coordinated application-level direct UE-to-UE communications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2023-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Provides necessary changes to include TR 23.745 solutions 1 and 11 from FFApp to SEAL |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduces UE-to-UE coordination based on application requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FFApp functionality not supported in SEAL |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.2.x1 (new); 14.3.2.x2 (new); 14.3.2.x3 (new); 14.3.2.x4 (new); 14.3.2.x5 (new); 14.3.Y.1 (new); 14.3.Y.2 (new); 14.3.Y.3 (new); |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | n | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | n | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | n | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S6-231592 provides corrections to several steps of the procedure in 14.3.Y and to the figure. It also clarifies that the UE-to-UE direct communications enabled are not PC5 but realized over Uu. This corresponds to the requirements and solutions for FFApp, as described in TR 23.745 (see FFA-2 reference point) . S6-231592 rev 1 merges the new procedure in clause 14.3.9 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[3]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[4]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[5]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[6]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[7]	3GPP TS 23.286: "Application layer support for V2X services; Functional architecture and information flows".

[8]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[9]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[10]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[11]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[12]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[13]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[14]	3GPP TS 23.002: "Network Architecture".

[15]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[16]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[17]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[18]	3GPP TS 23.203: "Policy and charging control architecture".

[19]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[20]	3GPP TS 26.348: "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point".

[21]	3GPP TS 29.214: "Policy and charging control over Rx reference point".

[22]	3GPP TS 29.468: "Group Communication System Enablers for LTE (GCSE_LTE); MB2 Reference Point; Stage 3".

[23]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[24]	IETF RFC 6733 (October 2012): "Diameter Base Protocol".

[25]	ETSI TS 102 894-2 (V1.2.1): "Intelligent Transport Systems (ITS); Users and applications requirements; Part 2: Applications and facilities layer common data dictionaryMultimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[26]	ETSI TS 102 965 (V1.4.1): "Intelligent Transport Systems (ITS); Application Object Identifier (ITS-AID); Registration".

[27]	ISO TS 17419: "Intelligent Transport Systems - Cooperative systems - Classification and management of ITS applications in a global context".

[28]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[29]	3GPP TS 33.434: "Service Enabler Architecture Layer (SEAL); Security aspects for Verticals".

[30]	3GPP TS 29.549: "Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification; Stage3".

[31]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[32]	IETF RFC 7252: "The Constrained Application Protocol (CoAP)".

[33]	IETF RFC 8323: "CoAP (Constrained Application Protocol) over TCP, TLS, and WebSockets".

[34]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[35]	IEEE Std 802.1Qcc-2018: "Standard for Local and metropolitan area networks - Bridges and Bridged Networks - Amendment: Stream Reservation Protocol (SRP) Enhancements and Performance Improvements".

[36]	IEEE 802.1Q-2018: "IEEE Standard for Local and Metropolitan Area Networks—Bridges and Bridged Networks".

[37]	IEEE Std 802.1CB-2017: "Frame Replication and Elimination for Reliability".

[38]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[39]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[40]		3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[41]		3GPP TS 28.531: "Management and orchestration; Provisioning".

[42]		3GPP TS 28.533: "Management and orchestration; Architecture framework".

[43]		3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[44]		3GPP TS 28.532: "Management and orchestration; Generic management services".

[45]		3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[46]		3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[47]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[48]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".

[49]	3GPP TS 23.436: "Procedures for Application Data Analytics Enablement Service".

[50]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2"

[51]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3"

[52]	3GPP TS 23.256 "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

[x]	3GPP TS 23.545 “Application layer support for Factories of the Future (FF)".

* * * Next Change * * * *

#### 14.3.2.35	Application connectivity request

Table 14.3.2.35-1 describes the information flow for application connectivity request from a NRM client to the NRM server.

Table 14.3.2.35-1: Application connectivity request

| Information element | Status | Description |
| --- | --- | --- |
| Source VAL UE ID | M | Identifier of the requestor VAL UE |
| Source IP address | M | IP address of the requestor VAL UE |
| VAL service ID | M | Identifier of the VAL service |
| List of destination VAL UE IDs | M | Identifiers of the destination VAL UEs |
| Application service requirements | M | Details of service requirements |
| >Packet size (see NOTE) | O | Size of the packet to be transmitted. |
| >Packet transmission interval (see NOTE) | O | Intervals at which the packet is to be transmitted |
| >Packet E2E latency (see NOTE) | O | E2E Latency for packet transmission |
| >Packet error KPI (see NOTE) | O | The KPIs related to packet error (e.g. allowed packet loss rate, packet loss amount, packet error rate) |
| >Bitrate (see NOTE) | O | The bitrate required |
| Application connectivity context | O | Application connectivity context of the requester, used for determining the destination and/or resource parameters. If this IE is included, at least one of the information elements in Table 14.3.2.35-2 shall be provided. |
| NOTE: At least one of information elements of the application service requirements shall be included. |  |  |

Table 14.3.2.35-2: Application connectivity context

| Information element | Status | Description |
| --- | --- | --- |
| Location | O | Location information of the client. The element may also indicate whether network-determined location is to be used instead. |
| Speed | O | Speed information for the client. |
| Direction | O | Direction information for the client. |

#### 14.3.2.36	Application connectivity response

Table 14.3.2.36-1 describes the information flow for application connectivity response from the NRM server to the NRM client.

Table 14.3.2.36-1: Application connectivity response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result from the NRM server in response to request indicating success or failure for accepting the request. |
| Cause | O | The cause of failure, if the result indicates failure. |

#### 14.3.2.37	Application connectivity notification

Table 14.3.2.37-1 describes the information flow for application connectivity notification from the NRM server to the NRM client.

Table 14.3.2.37-1: Application connectivity notification

| Information element | Status | Description |
| --- | --- | --- |
| Session information | M | Session information for the established connectivity for application communication |
| VAL service ID | M | Identifier of the VAL service |
| Requestor VAL UE ID | M | The identity of the source VAL UE requesting the connectivity |
| List of accepted destination VAL UE IDs (see NOTE) | O | The list of destination VAL UEs which are accepted for the connectivity established for the application service communication |
| NOTE: The List of accepted destination VAL UE IDs IE shall be included when the application connectivity notification is sent to the initiator of the application connectivity request. |  |  |

* * * Next Change * * * *

#### 14.3.2.x4	Get application connectivity context request.

Table 14.3.2.x4-1 describes the information flow for application connectivity context request from the NRM server to the NRM client, the context applying to UE-to-UE application-level direct communications (e.g. over FFA-2 reference point see TS 23.545[x]).

.

Table 14.3.2.x4-1: Get Application connectivity context request.

| Information element | Status | Description |
| --- | --- | --- |
| Requester VALUE ID | M | The identity of the source VAL client for which the NRM client performs the request. |
| VAL service ID | M | Identify of the VAL service for which the information is requested. |
| VAL-specific connection coordination context information | O | Additional information required to identify the context data (e.g. device type, device vendor, etc) |

#### 14.3.2.x5	Get application connectivity context response.

Table 14.3.2.x5-1 describes the information flow for application connectivity context response from the NRM client to the NRM server, the context applying to UE-to-UE application-level direct communications (e.g. over FFA-2 reference point see TS 23.545[x]).

Table 14.3.2.x5-1: UE-to-UE connection coordination context response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of the UE-to-UE resource coordination response operation. |
| Application connectivity context (see NOTE) | O | Application connectivity context used for determining connectivity parameters. If this IE is included, at least one of the information elements in Table 14.3.2.35-2 shall be provided. |
| NOTE: When this information element is not included, the NRM server considers default or pre-provisioned values. |  |  |

* * * Next Change * * * *

### 14.3.9	Establishing communication with application service requirements

#### 14.3.9.1	General

The NRM client and the NRM server (acting as an AS) are involved in the exchange and analysis of the desired service requirements (e.g. packet size, packet transmission interval, reliability, packet error rate) for the E2E communication amongst the Vertical UEs. The NRM server triggers the establishment of application-level direct service connectivity between two UEs via Uu, based on the information provided by the UEs and static configuration information available to the NRM server prior to the UE interaction. Note that service connectivity among VAL clients is established over the Uu, without device-to-device direct radio connectivity (e.g. PC5) requirement.

#### 14.3.9.2	Procedures

##### 14.3.9.2.1	Procedure triggered by correlated source and destination requests

The procedure for establishing Uu-based application-level direct communications between two UEs,  with application service requirements is as illustrated in figure 14.3.9.2.1-1. In this procedure the source and destination VAL clients correlate their triggering of the procedure establishment before the NRM Server provides the service.

Pre-conditions:

-	NRM client 1 and NRM client 2 are provided configuration information for the VAL clients served e.g. connectivity requirements, which destination UEs to connect to over Uu, etc.

-	The NRM client 1 and NRM client 2 are configured with the information of the NRM server and have connectivity enabled to communicate with the NRM server. The information is provided via pre-configuration.

-	The NRM server is configured with policies and information of the UEs to determine authorization of the UEs requesting connectivity via Uu.

-	The VAL clients associated with NRM client 1 and NRM client 2 have triggered the establishment of connectivity.

Figure 14.3.9.2.1-1: Establishing communication with application service requirements

1a.	The NRM client 1 sends the application connectivity request (source identity and IP address, destination identities, service requirements) to the NRM Server. The service requirement from the source includes packet size, packet transmission interval, packet E2E latency, allowed packet loss rate/packet loss amount/packet error rate, etc. The destination may be multiple UEs (devices). The identity of source and destination may be the application user identity or the MAC address.

1b.	The NRM server determines whether the UE of NRM client 1 is authorized to connect to the destination UEs for direct service communications via Uu. If UE of NRM client 1 is authorized to connect to the destination UEs, then a response is provided to the NRM client 1 indicating acceptance of the request.

2a.	The NRM client 2 sends the application connectivity request (destination identity and IP address, source identity, service requirements) to the NRM server. The service requirements from the destination includes the service requirements as described in step 1a.

2b.	The NRM server determines whether the UE of NRM client 2 is authorized to connect to the destination UEs for direct service communications via Uu. If UE of NRM client 2 is authorized to connect to the destination UEs, then a response is provided to the NRM client 2 indicating acceptance of the request.

3.	Based on the service requirements received in step 1 and step 2, the NRM server determines the parameters and patterns for direct service connectivity between the UEs via Uu and also the transport requirements, i.e., QoS requirements for the 3GPP system (e.g. 5GS). This step may also include retrieving the direct link status of the UEs (e.g. PDU Session Status, UE reachability). If the NRM server determines that direct service connectivity via Uu is not authorized or not possible with the given connectivity requirements, it skips step 4 and proceeds to steps 5 and 6, informing each NRM client accordingly.

NRM server will process E2E connectivity establishment between NRM client 1 and NRM client 2 only after it receives the request from NRM client 2. There can be several NRM clients (destinations) which will perform step 2 and NRM server will process their E2E connectivity with NRM client 1 (source) as and when the requests are received by the NRM server.

4.	The NRM server triggers 3GPP system to establish Uu connectivity between the UE of NRM client 1 and UE of NRM client 2 with required QoS as specified in 3GPP TS 23.501 [10].

5.	The NRM server sends the application connectivity notification (connectivity/session information) to NRM client 1 indicating successful establishment of the connectivity. The connectivity/session information may contain the accepted destination identities.

6.	The NRM server sends the application connectivity notification (connectivity/session information) to NRM client 2 indicating successful establishment of the connectivity.

* * * Next Change * * * *

##### 14.3.9.2.2	Procedure triggered by source request and coordinated with destination

This procedure is used for establishing Uu-based application-level direct communications between two UEs, based on a single client initiating and providing  application requirements. The procedure is as illustrated in figure 14.3.9.2.2-1. The NRM Server provides the service by coordinating with the destination client.

Pre-conditions:

-	NRM client 1 and NRM client 2 are provided configuration information for the VAL clients served e.g. connectivity requirements, etc.

-	Pre-processing determines that network assisted UE-to-UE communications is required. VAL application policies and destination information for NRM clients are available at the NRM server.

-	The VAL client associated with NRM client 1 triggers the establishment of connectivity and provides information about (one or more) destination VAL client(s).

Figure 14.3.Y.2-1: Coordination UE-to-UE communications with VAL application requirements

1.	The NRM client 1 sends the application connectivity request (source identity and IP address, destination identities, application requirements) to the NRM server to establish connectivity for VAL client 1 on UE 1. The destination VAL client(s) may be hosted on one or multiple UEs (devices).

2.	The NRM server determines whether VAL client 1 is authorized to connect to VAL client 2 for application-level direct UE-to-UE communications. If VAL client 1 is authorized to connect to VAL client 2, the NRM server performs the get application connectivity context request to  retrieve  VAL UE-to-UE connection coordination context procedure as described in clause 14.3.2.x4. This step can be skipped if the NRM server is already aware of VAL client 2's context information.

NOTE:	The signaling and functionality for handling the cases when NRM client 2 will be temporarily unavailable for establishing the direct service connection are implementation dependent.

3. The NRM client 2 sends the get application connectivity context response to the NRM server, 2.	Using the request information and local policies, the NRM client 2 determines whether context information is to be provided for establishing application-level direct connectivity to the counterpart UE for the VAL application indicated. If the NRM client determines that context information is to be provided, it responds to the NRM server and provides the VAL connection coordination context data for the VAL client served.

4.	The NRM server sends the application connectivity response to NRM client 1.

5.	The NRM server uses VAL client 1's and VAL client 2's context information, their application-level direct UE-to-UE connectivity requirements, location information, and network context as input, checks connectivity service policies, and determines the parameters and patterns for application-level direct UE-to-UE connectivity between  the VAL clients. The NRM server may also determine transport requirements, e.g. QoS requirements, for the 3GPP system (e.g. 5GS).

If network provided location information is used, location information may be obtained from the SEAL location management server. Alternatively, Location Reporting monitoring as described in 23.502[12] may be used. This step may also include a request for direct link status (e.g. PDU Session Status, UE reachability, etc. as described in 23.502 [12]).  This action may be skipped if  the clients provide location information or if there are no location requirements for establishing the application-level direct UE-to-UE connectivity.

If the NRM server determines that UE-to-UE application-level direct connectivity is not authorized or not possible with the given connectivity requirements, it skips step 5 and proceeds to steps 6 and 7, informing each NRM client accordingly.

6.	The NRM server may request the 3GPP system to establish or modify the 3GPP system level connectivity that enables the application-level direct UE-to-UE connection for VAL client 1 and VAL client 2 services, e.g. via modification of existing radio bearers. NRM server provides the necessary information (e.g. identifiers of VAL client 1 and VAL client 2, transport requirements) in this request message.

7.	a. The NRM server notifies NRM client 1 of the established UE-to-UE connection b. The NRM server notifies the NRM client 2 of the established UE-to-UE connection.

Each NRM client notifies the corresponding VAL client of the established application-level direct UE-to-UE connection.

* * * End of Changes * * * *