3GPP TSG-SA WG6 Meeting #54-e	S6-231636

17th – 26th April 2023	(revision of S6-231585, 1281)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0306 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve the EN in the EEC executed ACR via T-EES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This contribution is provided to resolve the EN in the service provisioning request clause: Editor's Note: it is FFS how EEC context pull request will work if S-EES does not support "EEC executed ACR via T-EES" scenario. When the EEC service continuity support indicates that T-EES must support "EEC executed ACR via T-EES" scenario but the S-EES or T-EES does not support the EEC context pull capability, then the EEC determines to register to the T-EES after the ACR. Thus the related clause are enhanced to solve the issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Enhance EES profile with the EEC context transmission capability, then the EEC can determine whether the registration is required or not - Fix some editorial issues |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The above EN will remain open. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.6, 8.3.3.3.2, 8.3.3.3.3, 8.3.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.3.3.3.2	Service provisioning request

Table 8.3.3.3.2-1 describes the information elements for service provisioning request from the EEC to the ECS.

Table 8.3.3.3.2-1: Service provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| EECID | M | Unique identifier of the EEC. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| AC Profile(s) | O | Information about services the EEC wants to connect to, as described in Table 8.2.2-1. |
| EEC Service Continuity Support | O | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC. When requesting service provisiojing for T-EES discovery, if the EEC requires that T-EES must support "EEC excuted ACR via T-EES" scenario, then EEC includes only "EEC excuted ACR via T-EES" in this IE |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Connectivity information | O | List of connectivity information for the UE, e.g. PLMN ID, SSID. |
| UE location | O | The location information of the UE. The UE location is described in clause 7.3.2. |
| ECSP identifiers | O | The list of desired EES provider IDs, by the EEC. |

Editor's Note: [SA3] Whether the EECID and the UE ID included in request of EDGE-1 & 4 interactions is part of the security credential is SA3's responsibility.

* * * END of Change * * * *