3GPP TSG-SA WG6 Meeting #54e	S6-231642

e-meeting,  17th – 26th April 2023		(revision of S6-231225, S6-231451)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0192 | rev | 2 | Current version: | 18.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add “location information unsubscribe” and “Monitor location unsubscribe” information flow, procedure and APIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GFLS,eSEAL2 |  |  |  |  |  | Date: |  |  | 2023-04-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are “location information subscription” and “Monitor location subscription” related information flow, procedure and APIs, however there are not related unsubscribe related information flow, procedure and APIs in SEAL LM of TS 23.434. And the general description of SEAL APIs for SEAL LM is also not completed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add “location information unsubscribe” and “Monitor location unsubscribe” information flow, procedure and APIs for SEAL LM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.x(new), 9.3.2.y(new), 9.3.z(new), 9.3.2.p(new), 9.3.2.q(new), 9.3.r(new),9.4.1,9.4.3.a(new),9.4.6.b(new),9.4.6.2,9.4.6.3,9.4.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3.2.x	Location information unsubscribe request

Table 9.3.2.x-1 describes the information flow from the VAL server or location management client to the location management server for location information unsubscribe request.

Table 9.3.2.x-1: Location information unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the VAL server or VAL UE’s requesting ID. |

9.3.2.y	Location information unsubscribe response

Table 9.3.2.y-1 describes the information flow from the location management server to the VAL server or location management client for location information unsubscribe response.

Table 9.3.2.y-1: Location information unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Request status | M | It indicates the unsubscribe request result |

* * * Next Change * * * *

9.3.z	Location information unsubscribe procedure

Figure 9.3.z-1 illustrates the high level procedure of location information unsubscribe request. The same procedure can be applied for the location management client and other entities that would like to unsubscribe to the VAL UE’s location information.

Figure 9.3.z-1: Location information unsubscribe procedure

1.	The VAL server sends location information unsubscribe request to the location management server to unsubscribe the subscription to the VAL UE’s location information.

2.	The location management server replies with location information unsubscribe response indicating the unsubscribe status.

* * * Next Change * * * *

9.3.2.p	Monitor location unsubscribe request

Table 9.3.2.p-1 describes the information flow from the VAL server to the LM server for monitor location unsubscribe request.

Table 9.3.2.p-1: Monitor location unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the VAL server’s requesting ID. |

9.3.2.q	Monitor location unsubscribe response

Table 9.3.2.q-1 describes the information flow from LM server to the VAL server for monitor location unsubscribe response.

Table 9.3.2.q-1: Monitor location unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Request status | M | It indicates the unsubscribe request result |

* * * Next Change * * * *

9.3.r	Monitor location unsubscribe procedure

Figure 9.3.r-1 illustrates the high level procedure of location monitoring unsubscribe request. The same procedure can be applied for other entities that would like to unsubscribe the subscription to monitor the VAL UE's location in a given area of interest.

Figure 9.3.r-1: Monitor location unsubscribe procedure

1.	The VAL server sends Monitor location unsubscribe request to the location management server to unsubscribe the subscription to monitor the VAL UE's location in a given area of interest.

2.	The location management server replies with Monitor location unsubscribe response indicating the unsubscribe status.

* * * Next Change * * * *

9.4	SEAL APIs for location management

9.4.1	General

Table 9.4.1-1 illustrates the SEAL APIs for location management.

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting Update_Trigger_Location_Reporting Cancel_Trigger_Location_Reporting | VAL server | Request /Response |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Info | VAL server |  |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |
| SS_LocationAreaInfoRetrieval | Obtain_UEs_Info | VAL server | Request/Response |
| SS_LocationMonitoring | Subscribe_Location_Monitoring | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Monitoring | VAL server |  |
|  | Notify_Location_Monitoring_Events | VAL server |  |
| SS_LocationAreaMonitoring | Subscribe_Location_Area_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Location_Area_Monitoring_Events |  |  |
|  | Update_Location_Area_Monitoring_Subscribe |  |  |
|  | Unsubscribe_Location_Area_Monitoring |  |  |

* * * Next Change * * * *

9.4.3	SS_LocationInfoEvent API

9.4.3.1	General

API description: This API enables the VAL server to subscribe, receive and unsubscribe the UEs location information from the location management server over LM-S.

* * * Next Change * * * *

9.4.3.a	Unsubscribe_Location_Info operation

API operation name: Unsubscribe_Location_Info

Description: Unsubscribe to the location information.

Known Consumers: VAL server.

Inputs: Refer subclause 9.3.2.x

Outputs: Refer subclause 9.3.2.y

See subclause 9.3.z for the details of usage of this API operation.

* * * Next Change * * * *

9.4.6	SS_LocationMonitoring API

9.4.6.b	Unsubscribe_Location_ Monitoring operation

API operation name: Unsubscribe_Location_Monitoring

Description: Unsubscribe to the location Monitoring request.

Known Consumers: VAL server.

Inputs: Refer subclause 9.3.2.p

Outputs: Refer subclause 9.3.2.q

See subclause 9.3.r for the details of usage of this API operation.

9.4.6.2	Subscribe_Location_Monitoring operation

API operation name: Subscribe_Location_Monitoring

Description: Subscription to location monitoring.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.11

Outputs: 9.3.2.12.

See subclause 9.3.11.2 for the details of usage of this API operation.

9.4.6.3	Notify_Location_Monitoring_Events operation

API operation name: Notify_Location_Monitoring_Events

Description: Notifying the VAL server with the location monitoring events related to VAL UE.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.13

Outputs: None.

See subclause 9.3.11.2 for the details of usage of this API operation.

* * * Next Change * * * *

9.4.2.4	Cancel_Trigger_Location_Reporting operation

API operation name: Cancel_Trigger_Location_Reporting

Description: Cancels the trigger to report location information.

Known Consumers: VAL server.

Inputs: Refer subclause 9.3.2.8

Outputs: Refer subclause 9.3.2.25

See subclause 9.3.6 for the details of usage of this API operation.

* * * End of Changes * * * *