3GPP TSG-SA WG6 Meeting #54-e	S6-231185

17th – 26th April 2023

Source:	Huawei

Title:	Pseudo-CR on add procedures, information flows and APIs of slice requirements verification and alignment

Spec:	3GPP TS 23.435 1.1.0

Agenda item:	8.10

Document for:	Approval

Contact:	JIA Xiaoqian, jiaxiaoqian@huawei.com

1. Introduction

The Solutions of QoS(slice requirements) verfication and slice requirements alignment are discussed and agreed in NSCE study phase. This contribution proposes to add the procedures, informatin flows and APIs of slice requirements verfication and alignment capability exposed by NSCE.

2. Reason for Change

Add the slice requirements verification and alignment capability exposed by NSCE.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.435 1.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[4]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[5]	GSMA NG.116 - Generic Network Slice Template.

[6]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[7]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[8]		3GPP TS 28.531: "Management and orchestration; Provisioning".

[9]	3GPP TS 28.537: "Management and orchestration; Management capabilities".

[10]		3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[11]	3GPP TS 28.535: "Management and orchestration; Management services for communication service assurance; Requirements".

[12]		3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[13]		3GPP TS 29.536: "5G System; Network Slice Admission Control Service; Stage 3".

[14]	3GPP TS 33.501: "Security architecture and procedures for 5G System"

[15]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements"

[16]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[17]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[18]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[19]		3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[20]		3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[21]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".[22]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[22]	3GPP TS 33.434: "Service Enabler Architecture Layer (SEAL); Security aspects for Verticals".

[x]	3GPP TS 26.114: " IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction ".

* * * Next Change * * * *

## 9.X	Procedure on slice requirements verification and alignment capability exposure

### 9.X.1	General

Verticals can compare the QoS achieved by the provider with the QoS/slice requirements and its own experience of the QoS to verify if the QoS/Slice requirements is reasonably configured. To order a slice with certain slice requirements parameters and their values, the verticals will put their best effort into slice requirements translation. However, they are not able to guarantee that all the potential factors will be considered to generate the optimal slice requirements parameters on the first try.

In some cases, there may be some unforeseen exceptions (e.g., unexpected traffic changes) and the current configured slice requirements parameters are not able to fulfil the verticals requirements (e.g., more resources are required to address the exceptions), the VAL client may also suffer unsatisfied experience. Or in some other cases, when the service is executed on the required slice, the slice may not fully match the service real-time running conditions, for example, maybe only 60% of the slice resource is used to support the service, and rest of the slice resource is always idle, or the slice resource is insufficient due to under-provisioning

Hence, NSCE is able to provide the capability of comparing the QoS achievement status together with the OAM QoS data versus real customer QoS data (e.g., MOS) collected from VAL client to check whether the existing QoS/Slice related data is able to satisfy the VAL clients and alignment between the network performance statistics and slice requirements.

### 9.X.2	Procedure

#### 9.X.2.1	Procedures on slice requirements verification and alignment capability exposure

Figure 9.X.2.1-1 provides a possible procedure of slice requirements verification and alignment capability exposure.

Pre-conditions:

1.	The VAL Server has subscribed to the slice requirements verification and alignment capability.

2.	The NSCE Server has subscribed to the service of network slice performance management provided by EGMF.

3.	The NSCE server has subscribed to the service of network slice provisioning provided by EGMF.

Figure 9.X.2.1-1: Slice requirements verification and alignment process

1.	The VAL server subscribe to slice requirements verification and alignment services provided by NSCE server to require the NSCE server to check whether the slice requirements (by configuring the attributes of serviceProfile) matches the real network slice usage status, the request may include the S-NSSAI, the ID of the VAL server, the slice requirements parameters (attributes of serviceProfile) which is requested to be aligned.

2.	The NSCE server checks if the VAL server is authorized to trigger the service of slice requirements alignment.

3.	The NSCE server response to VAL server with the result of slice requirements verification and alignment request, e.g., the slice requirements alignment request is accepted.

4.	On receiving the request from VAL server, the NSCE server may optionallycollect the QoE data from the NSCE client. The QoE data can refer to the data defined in clause 16, TS 26.114[x].

Note: The QoE data collection from NSCE client follows the mechanism defined in EVEX work in SA4.

5.	On receiving the request from VAL server, NSCE server retrieves the network slice related performance data and KPIs (e.g., the average PRB usage, the distribution of the PRB usage) defined in TS 28.552[19] by utilize the APIs defined in clause 11.3, TS 28.532[7].

6.	NSCE server checks the experiences from the end users and compares the slice requirement parameters of network slice (the values of attributes of serviceProfile, e.g., radioSpectrum, the maxNumberofUEs) with network slice performance statistics (e.g., active number of users, the average PRB usage, the distribution of the PRB usage of the S-NSSAI) to generate the optimal slice requirements for the required service (represented by S-NSSAI) of the vertical and determine whether the network slice modification operations should be taken. This step depends on implementation (e.g., if the VAL client experience is satisfied while the network slice resources are with low utilization, the resources required in the slice requirements could be reduced).

7.	If NSCE decides to update the slice requirements, the NSCE server performs the network slice modification operation, as defined in clause 7.6, TS 28.531[5] exposed by EGMF defined in SA5, the serviceProfile will be re-configured by utilizing the modifyMOIAttributes operation.

8.	NSCE server sends the notifications to VAL server to notify the change of the slice requirements parameters.

### 9.X.3	Information flows

#### 9.X.3.1	General

The following information flows are specified for slice requirements verification and alignment capability exposure.

#### 9.X.3.2	Slice requirements verification and alignment subscription

Table 9.X.3.2-1 describes information elements for slice requirements verification and alignment subscription.

Table 9.X.3.2-1: Slice requirements verification and alignment subscription

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server |
| VAL service ID | M | The identifier of the VAL service for which the requirement request applies |
| VAL UE ID list | O | The list of VAL UE IDs for which the requirement request applies |
| Slice identifier | M | The slice identifier (S-NSSAI, NSI ID or ENSI) which is mapped to the VAL application, if known by the VAL server |
| Slice Requirements List | M | The list of the slice requirements which need to be aligned. |
| >sliceRequirement | M | The requirement which need to be aligned, this parameter refers to the attribute of serviceProfile defined in TS 28.531[5] |

#### 9.X.3.3	Response of slice requirements verification and alignment subscription

Table 9.X.3.3-1 describes information elements for response of slice requirements verification and alignment subscription.

Table 9.X.3.3-1: Response of slice requirements verification and alignment subscription

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the subscription (positive or negative acknowledgement) |

#### 9.X.3.4	Slice requirements verification and alignment notification

Table 9.X.3.4-1 describes information elements for slice requirements verification and alignment notification.

Table 9.X.3.4-1: Slice requirements verification and alignment notification

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server |
| VAL service ID | M | The identifier of the VAL service for which the requirement request applies |
| VAL UE ID list | O | The list of VAL UE IDs for which the requirement request applies |
| Slice identifier | M | The slice identifier (S-NSSAI, NSI ID or ENSI) which is mapped to the VAL application, if known by the VAL server |
| Updated Slice requirements info | M | The attributes and values of slice requirements |

### 9.X.4	APIs

#### 9.X.4.1	General

Table 9.X.4.1-1 illustrates the APIs for the network slice verification and alignment capability exposure.

Table 9.X.4.1-1: List of APIs for network slice verification and alignment

| API Name | API Operations | Communication Type | Consumer(s) |
| --- | --- | --- | --- |
| SS_NSCE_SliceReq_VerifyAndAlign | SliceReq_VerifyAndAlign_Subscribe | Subscription/Notification | VAL server |
|  | SliceReq_VerifyAndAlign_Notification |  |  |

#### 9.X.4.2	SS_NSCE_SliceReq_VerifyAndAlign API

##### 9.X.4.2.1	General

API description: This API enables the VAL server to communicate with the network slice capability enablement server to consume the network slice requirements verification and alignment service.

##### 9.X.4.2.2	SliceReq VerifyAndAlign_Subscribe

API operation name: SliceReq_VerifyAndAlign

Description: The consumer subscribe to the network slice requirements verification and alignment.

Known Consumers: VAL server

Inputs: See table 9.X.3.2-1

Outputs: See table 9.X.3.3-1

##### 9.X.4.2.3	SliceReq_VerifyAndAlign_Notification

API operation name: SliceReq_VerifyAndAlign_Notification

Description: The consumer is notified with the updated network slice requirements information according to the slice requirements verification and alignment.

Known Consumers: VAL server

Inputs: None

Outputs: See table 9.X.3.4-1