3GPP TSG-SA WG6 Meeting #55	S6-231700

Munich, Germany 22nd – 26th May 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0396 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Syncing Criteria changes with 23.379 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T, FirstNet, Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Enh4MCPTT |  |  |  |  |  | Date: |  |  | 2023-02-13 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR is to align TS23.280 with TS23.379. During SA6 meeting #54 S6-231573 was agreed that added criteria to the preconfigured regroup request and associated procedure in TS23.379, it also added a NOTE2 in the preconfired regroup request message. This CR adds the NOTE2 to the appropriate sections in TS23.280. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the “Note2: This information element is used only for the user regroup procedures” and the “Note2” reference in the IE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS23.280 and TS23.379 will not be in-sync for the use of the Criteria IE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.15.2.1 10.15.2.2, and 10.15.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.15.2.1	Preconfigured regroup request (MC service client – MC service server)

Table 10.15.2.1-1 describes the information flow preconfigured regroup request from the MC service client to the MC service server.

Table 10.15.2.1-1 Preconfigured regroup request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requester |
| MC service group ID | O | MC service group ID of the regroup group |
| MC service group ID | M | MC service group ID of the MC service group from which configuration is to be taken |
| MC service group ID list | O (see NOTE1) | List of MC service groups to be regrouped into the group regroup group |
| MC service user ID list | O (see NOTE1) | List of MC service users to be regrouped into the user regroup group |
| MC service user criteria | O (see NOTE1, NOTE2) | Carries the details of criteria which will be used by the MC service server for determining the participants. For example, it can be a location based criteria to determine the MC service user ID list in a particular area, or it could be tags (e.g. "fire", "medical", "police", etc.), or a combination of tags and location, which is left to implementation. |
| NOTE1: One and only one of these shall be present. NOTE2: This information element is used only for the user regroup procedures |  |  |

* * * Second Change * * * *

#### 10.15.2.2	Preconfigured regroup request (MC service server – MC service client)

Table 10.15.2.2-1 describes the information flow preconfigured regroup request from the MC service server to the MC service client.

Table 10.15.2.2-1 Preconfigured regroup request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the target MC service group member |
| MC service group ID | O | MC service group ID of the regroup group |
| MC service group ID | M | MC service group ID of the MC service group from which configuration is to be taken |
| MC service group ID list | O (see NOTE1) | List of MC service groups to be regrouped into the group regroup group |
| MC service user ID list | O (see NOTE1) | List of MC service users to be regrouped into the user regroup group |
| MC service user criteria | O (see NOTE1, NOTE2) | Carries the details of criteria which will be used by the MC service server for determining the participants. For example, it can be a location based criteria to determine the MC service user ID list in a particular area, or it could be tags (e.g. "fire", "medical", "police", etc.), or a combination of tags and location, which is left to implementation. |
| NOTE1: One and only one of these shall be present. NOTE2: This information element is used only for the user regroup procedures |  |  |

* * * Third Change * * * *

#### 10.15.2.3	Preconfigured regroup request (MC service server – MC service server)

Table 10.15.2.3-1 describes the information flow preconfigured regroup request from the MC service server to the MC service server.

Table 10.15.2.3-1 Preconfigured regroup request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requester |
| MC service group ID | O | MC service group ID of the regroup group |
| MC service group ID | M | MC service group ID of the MC service group from which configuration is to be taken |
| MC service group ID list | O (see NOTE1) | List of MC service groups to be regrouped into the group regroup group |
| MC service user ID list | O (see NOTE1) | List of MC service users to be regrouped into the user regroup group |
| MC service user criteria | O (see NOTE1, NOTE2) | Carries the details of criteria which will be used by the MC service server for determining the participants. For example, it can be a location based criteria to determine the MC service user ID list in a particular area, or it could be tags (e.g. “fire”, “medical”, “police”, etc.), or a combination of tags and location, which is left to implementation. |
| NOTE: One and only one of these shall be present. NOTE2: This information element is used only for the user regroup procedures |  |  |

* * * End of Changes * * * *