3GPP TSG-SA WG6 Meeting #55	S6-231702

Berlin, Germany 22nd – 26th May 2023	(revision of S6-23xxxx)

Source:	Samsung

Title:	Procedures and APIs for Network Slice Information delivery

Spec:	3GPP TS 23.435 v1.2.0

Agenda item:	8.10

Document for:	Approval

Contact:	Hyesung Kim <hs1207.kim@samsung.com>

1. Introduction

This pCR proposes procedures for subscription and notification for NS information delivery.

2. Reason for Change

The EN “Subscribe-notify procedure for Network Slice Information delivery will be defined” needs to be resolved. Additionally, the API and information flows need to be specified accordingly so that the following Editor's note: “Information flows of Network Slice Information delivery to NSCE client will be specified” can be also addressed.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.435 v1.2.0.

* * * First Change * * * *

9.17.2.1	Network Slice Information delivery request

This subclause depicts the procedure of the Network Slice Information delivery to the VAL server via NSCE server, when the VAL server requests the Network Slice Information after registration.

Pre-condition:

1. 	The NSCE server should have the agreement with MNO (NOP) for retrieval of ServiceProfile, if the NSCE server is the external entity.

Figure 9.17.2.1-1: Network Slice Information delivery request

1.	The NSCE server retrieves the Network Slice ServiceProfile from 5GS (e.g., NSMF) when the NSCE server acting as a NSP prepares a Network Slice to be provided. The NSCE server follows the procedure to request/receive the Network Slice Service Profile with 'getMOIAttributes' operation as specified in 3GPP TS 28.532[7].

NOTE:	If NSCE server and NSMF are in same operator, then the NSCE server gets access directly to NSMF.      The delivered Network Slice Service Profile contains the values of attributes such as PLMN, S-NSSAI, SST, maximum number of UEs, maximum number of PDU sessions, Slice Coverage Area, Latency, and, Data volume, which specify the Network Slice characteristics, as specified in clause of ServiceProfile in 3GPP TS 28.541 [10].

2.	The NSCE server, as Network Slice as a Service, creates and stores the Network Slice information. When NSCE server retrieves the Network Slice Information, it is necessary for NSCE server to convert the attributes in Network Slice ServiceProfile to the Network Slice information for readable information and to compose the Network Slice information, according to the NSP's policy.

In order to reduce to request often the Network Slice Information Retrieval, the NSCE server stores the Network Slice information.

3.	The VAL server requests the Network Slice Information to the NSCE server. If the VAL server needs to know the specific attribute value for its service, then the attribute name of Network Slice Information (e.g., S-NSSAI, SST, Slice Coverage Area, etc.) can be added in the Request message.

4.	The NSCE server performs to check whether the requesting VAL server is registered or not. The NSCE server identifies which the Network Slice Customer is authorized to use.

5.	The NSCE server sends the Network Slice Information, if the VAL server is registered and authorized. The NSCE server rejects to the request of the Network Slice Information, if not registered.

9.17.2.2	Network Slice Information delivery subscription

This subclause depicts the subscription procedure of the Network Slice Information delivery to the VAL server via NSCE server, when the VAL server requests the Network Slice Information after registration.

Pre-condition:

1. 	The NSCE server should have the agreement with MNO (NOP) for retrieval of ServiceProfile, if the NSCE server is the external entity.

Figure 9.17.2.2-1: Network Slice Information delivery subscribe request

1.	The NSCE server retrieves the Network Slice ServiceProfile from 5GS as described in the step 1 of clause 9.17.2.1.

2.	The NSCE server, as Network Slice as a Service, creates and stores the Network Slice information as described in the step 2 of clause 9.17.2.1.

3.	The VAL server sends the Network Slice Information Subscribe Request to the NSCE server. If the VAL server needs to know the specific attribute value for its service, then the attribute name of Network Slice Information (e.g., S-NSSAI, SST, Slice Coverage Area, etc.) can be added in the Subscribe Request message.

4.	The NSCE server performs to check whether the requesting VAL server is registered or not. The NSCE server identifies which the Network Slice Customer is authorized to use.

5.	The NSCE server sends the Network Slice Information Subscribe Response that contains the subscription ID and optionally the expiration time, if the VAL server is registered and authorized. The NSCE server rejects to the request of the Network Slice Information, if not registered.

9.17.2.3	Network Slice Information delivery notification

This subclause depicts the subscription procedure of the Network Slice Information delivery to the VAL server via NSCE server, when the VAL server requests the Network Slice Information after registration.

Pre-condition:

1. 	The NSCE server should have the agreement with MNO (NOP) for retrieval of ServiceProfile, if the NSCE server is the external entity.

2. 	The VAL Server has subscribed to the NSCE server for Network Slice Information delivery.

Figure 9.17.2.3-1: Network Slice Information delivery notification

1.	The NSCE server retrieves the updated Network Slice ServiceProfile from 5GS (e.g., NSMF) when the NSCE server acting as a NSP prepares a Network Slice to be provided. The NSCE server follows the procedure to request/receive the Network Slice Service Profile with 'getMOIAttributes' operation as specified in 3GPP TS 28.532[7]. The NSCE determines to notify the updated Network Slice ServiceProfile to the VAL server which has subscribed for Network Slice Information delivery.

2.	The NSCE server sends the Network Slice Information Delivery notification Response, which contains subscription ID and Network Slice Information.

9.17.2.4	Network Slice Information delivery subscription update

This subclause depicts the subscription update procedure of the Network Slice Information delivery to the VAL server via NSCE server.

Pre-condition:

1. 	The NSCE server should have the agreement with MNO (NOP) for retrieval of ServiceProfile, if the NSCE server is the external entity.

2.	The VAL server has subscribed to NSCE for Network Slice Information delivery.

Figure 9.17.2.4-1: Network Slice Information delivery subscription update request

1.	The VAL server sends the Network Slice Information delivery subscription update request to the NSCE server. The subscription update request message shall contain the subscription ID. If the VAL server needs to update the subscription in order to know the specific attribute value for its service, then the attribute name of Network Slice Information (e.g., S-NSSAI, SST, Slice Coverage Area, etc.) can be added in the Network Slice Information delivery subscription update request message.

2.	The NSCE server performs the authorization for the VAL server. If successful, the NSCE server updates the subscription information corresponding to the received subscription ID.

3.	The NSCE server may retrieve the Network Slice ServiceProfile from 5GS as described in the step 1 of clause 9.17.2.1.

4.	The NSCE server sends the Subscribe Update Response that contains the subscription ID and optionally the expiration time, if the VAL server is registered and authorized. The NSCE server rejects to the request of the Network Slice Information, if not registered.

9.17.2.5	Network Slice Information delivery unsubscribe

This subclause depicts the unsubscription procedure of the Network Slice Information delivery to the VAL server via NSCE server.

Pre-condition:

1. 	The NSCE server should have the agreement with MNO (NOP) for retrieval of ServiceProfile, if the NSCE server is the external entity.

2.	The VAL server has subscribed to NSCE for Network Slice Information delivery.

Figure 9.17.2.4-1: Network Slice Information delivery unsubscription

1.	The VAL server sends the Network Slice Information delivery unsubscription request to the NSCE server. The subscription update request message shall contain the subscription ID.

2.	The NSCE server performs the authorization for the VAL server. If successful, the NSCE server delete the subscription information corresponding to the received subscription ID.

3.	The NSCE server sends the Subscribe Update Response that contains the subscription ID and the result of unsubscription.

9.17.2.6	Network Slice Information delivery to NSCE client request

This subclause depicts the procedure of the Network Slice Information delivery to the NSCE client via NSCE server.

Pre-condition:

1. 	The network slice has been allocated/created to VAL server.

Figure 9.17.2.5-1: Network Slice Information delivery to NSCE client

1.	The VAL server sends a request to NSCE server to deliver the Network Slice Information to the NSCE Clients. The request includes the network slice information and indication of the UE notification with VAL UE's ID List.

2.	The NSCE server delivers the Network Slice Information to the NSCE Clients of VAL UEs based on the VAL UE's ID list from step 1, in case the Network Slice Allocation is successful, if the NSCE server does not perform AF-driven guidance for URSP determination to 5GS.

The Network Slice Information contains the VAL service ID, S-NSSAI, and DNN.

3.	The NSCE client stores and applies the Network Slice Information.

NOTE:	If the UE is provisioned with URSP rules by the network operator, the UE handles the precedence between the delivered network slice info via NSCE layer info and URSP rules as defined in clause 6.1.2.2.1 of 3GPP TS 23.503 [17]. How the UE uses the Network Slice info delivered via NSCE layer in relation to the URSP is implementation dependent.

4.	The NSCE client sends the Network Slice Allocation Information response to the NSCE server.

5.	The NSCE server sends the response to the Network Slice Information delivery to NSCE client request.

* * * Next Change * * * *

9.17.3.1	Network Slice Information delivery request

Table 9.z.3.1-1 describes the information flows for Network Slice Information delivery from the VAL server to the NSCE server.

Table 9.17.3.1-1: Network Slice Information delivery request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server |
| Security credentials | M | Security credentials resulting from a successful authorization. |
| VAL service ID | M | The VAL service ID of the VAL application |
| Requested network slice information list | O | Network slice information that is requested by the VAL server |
| > S-NSSAI information request | O | Indicates to request S-NSSAI |
| > SST information request | O | Indicates SST |
| > Slice Coverage Area information request | O | Indicates slice coverage area |

9.17.3.2	Network Slice Information delivery response

Table 9.17.3.2-1 describes the information flows for Network Slice Information delivery response from the NSCE server to the VAL server.

Table 9.17.3.2-1: Network slice information response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the Network Slice Information delivery request |
| > Network slice Information | O (see NOTE 1) | Network slice information |
| >> S-NSSAI | O | S-NSSAI |
| >> SST | O | Slice/Service Type |
| >> Slice Coverage Area | O | Coverage area of the network slice |
| > Cause | O (see NOTE 2) | Indicates the cause of Network Slice Information delivery request failure. |
| NOTE 1: Shall be present if the result is success and shall not be present otherwise. NOTE 2: Shall be present if the result is failure and shall not be present otherwise. |  |  |

9.17.3.3	Network Slice Information delivery subscribe request

Table 9.17.3.3-1 describes the information flows for Network Slice Information delivery subscribe request from VAL server to the NSCE server.

Table 9.17.3.3-1: Network slice information subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server |
| Security credentials | M | Security credentials resulting from a successful authorization. |
| VAL service ID | M | The VAL service ID of the VAL application |
| Requested network slice information list | O | Network slice information that is requested by the VAL server |
| > S-NSSAI information request | O | Indicates to request S-NSSAI |
| > SST information request | O | Indicates SST |
| > Slice Coverage Area information request | O | Indicates slice coverage area |

9.17.3.4	Network Slice Information delivery subscribe response

Table 9.17.3.4-1 describes the information flows for Network Slice Information delivery response from the NSCE server to the VAL server.

Table 9.17.3.4-1: Network slice information subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the Network Slice Information delivery request |
| > Subscription ID | O (see NOTE 1) | Subscription identifier corresponding to the subscription |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| > Cause | O (see NOTE 2) | Indicates the cause of Network Slice Information delivery subscription request failure. |
| NOTE 1: Shall be present if the result is success and shall not be present otherwise. NOTE 2: Shall be present if the result is failure and shall not be present otherwise. |  |  |

9.17.3.5	Network Slice Information delivery notification

Table 9.17.3.5-1 describes the information flows for Network Slice Information delivery notification from the NSCE server to the VAL server.

Table 9.17.3.5-1: Network slice information notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the notification |
| > Network slice Information | M | Network slice information |
| >> S-NSSAI | O (see NOTE) | S-NSSAI |
| >> SST | O (see NOTE) | Slice/Service Type |
| >> Slice Coverage Area | O (see NOTE) | Coverage area of the network slice |
| NOTE : At least, one of them shall be present. |  |  |

9.17.3.6	Network Slice Information delivery subscribe update request

Table 9.17.3.6-1 describes the information flows for Network Slice Information delivery subscribe request from VAL server to the NSCE server.

Table 9.17.3.6-1: Network slice information subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription requested to be updated |
| VAL server ID | M | The identifier of the VAL server |
| Security credentials | M | Security credentials resulting from a successful authorization. |
| VAL service ID | M | The VAL service ID of the VAL application |
| Requested network slice information list | O | Network slice information that is requested by the VAL server |
| > S-NSSAI information request | O | Indicates to request S-NSSAI |
| > SST information request | O | Indicates SST |
| > Slice Coverage Area information request | O | Indicates slice coverage area |

9.17.3.7	Network Slice Information delivery subscribe update response

Table 9.17.3.7-1 describes the information flows for Network Slice Information delivery response from the NSCE server to the VAL server.

Table 9.17.3.7-1: Network slice information subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the Network Slice Information delivery request |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| > Cause | O (see NOTE) | Indicates the cause of Network Slice Information delivery subscription update request failure. |
| NOTE: Shall be present if the result is failure and shall not be present otherwise. |  |  |

9.17.3.8	Network Slice Information delivery unsubscribe request

Table 9.17.3.8-1 describes the information flows for Network Slice Information delivery unsubscribe request from VAL server to the NSCE server.

Table 9.17.3.8-1: Network slice information unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription requested to be updated |
| VAL server ID | M | The identifier of the VAL server |
| Security credentials | M | Security credentials resulting from a successful authorization. |
| VAL service ID | M | The VAL service ID of the VAL application |

9.17.3.9	Network Slice Information delivery subscribe update response

Table 9.17.3.7-1 describes the information flows for Network Slice Information delivery response from the NSCE server to the VAL server.

Table 9.17.3.7-1: Network slice information subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the Network Slice Information delivery request |
| > Cause | O (see NOTE) | Indicates the cause of Network Slice Information delivery subscription update request failure. |
| NOTE: Shall be present if the result is failure and shall not be present otherwise. |  |  |

* * * Next Change * * * *

9.17.4	APIs

9.17.4.1	General

Table 9.17.4.1-1 illustrates the APIs for the Network Slice Information delivery.

Table 9.17.4.1-1: List of APIs for Network Slice Information delivery

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| SS_NSCE_NSInfoDelivery | Get | Request/Response | VAL Server |
|  | Subscribe | Subscribe/Notify | VAL Server |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |

Table 9.17.4.1-1: List of APIs for Network Slice Information delivery

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| SS_NSCE_NSInfoDelivery | Get | Request/Response | VAL Server |
|  | Subscribe | Subscribe/Notify | VAL Server |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |

9.17.4.2	SS_NSCE_NSInfoDelivery Get operation

API operation name: SS_NSCE_NSInfoDelivery Get

Description: The consumer requests one time report of a UE's location.

Inputs: See clause 9.17.3.1.

Outputs: See clause 9.17.3.2.

See clause 9.17.2.1 for details of usage of this operation.

9.17.4.3	SS_NSCE_NSInfoDelivery Subscribe operation

API operation name: SS_NSCE_NSInfoDelivery Subscribe

Description: The consumer subscribes for continuous reporting of a UE's location.

Inputs: See clause 9.17.3.3.

Outputs: See clause 9.17.3.4.

See clause 9.17.2.2 for details of usage of this operation.

9.17.4.4	SS_NSCE_NSInfoDelivery Notify operation

API operation name: SS_NSCE_NSInfoDelivery Notify

Description: The consumer is notified with report of the UE's location.

Inputs: See clause 9.17.3.5.

Outputs: None.

See clause 9.17.2.3 for details of usage of this operation.

9.17.4.5	SS_NSCE_NSInfoDelivery UpdateSubscription operation

API operation name: SS_NSCE_NSInfoDelivery UpdateSubscription

Description: The consumer updates an existing subscription for continuous reporting of a UE's location.

Inputs: See clause 9.17.3.6.

Outputs: See clause 9.17.3.7.

See clause 9.17.2.4 for details of usage of this operation.

9.17.4.6	SS_NSCE_NSInfoDelivery Unsubscribe operation

API operation name: SS_NSCE_NSInfoDelivery Unsubscribe

Description: The consumer cancels an existing subscription for continuous reporting of a UE's location.

Inputs: See clause 9.17.3.8.

Outputs: See clause 9.17.3.9.

See clause 9.17.2.5 for details of usage of this operation.

* * * End of Change * * * *