3GPP TSG-SA WG6 Meeting #55	S6-231912

Berlin, Germany 22nd – 26th May 2023	(revision of S6-23xxxx)

Source:	Samsung

Title:	Pseudo-CR on deregistration and update registration of PINE

Spec:	3GPP TS 23.542 V0.3.0

Agenda item:	8.19

Document for:	Approval

Contact:	Arunprasath R, arun.prasath@samsung.com

1. Introduction

3GPP TS 23.542 captures the PINE registration prodecure in section 8.4.2 but the corresponding update registration procedure and deregistration procedure is missing.

2. Reason for Change

This pCR adds the procedures for PINE deregistration and update registration.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.542 V0.3.0.

* * * First Change * * * *

##### 8.4.2.2.X	PINE de-registration directly to PIN server

Figure 8.4.2.2.X-1 illustrates PIN de-registration procedure based on request/response model.

Pre-conditions:

1.	The PINE has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server and is already registered with the PIN server;

2.	The UE Identifier is available;

3.	The PINE has been authorized to communicate with the PIN server;

Figure 8.4.2.2.X-1: PINE de-registration directly to PIN server

1.	The PINE (including PINE/PEMC/PEGC) sends a PINE de-registration Request to the PIN server. The request includes the security credentials of the PINE received during authorization procedure and also the request may include the GPSI, MAC address, vendor name, device description, PIN element ID/PIN client ID, PINE Address.

2.	The PIN server checks whether the UE identified by the GPSI has active registration with the PIN server.

3.	The PIN server responds to the PINE with PINE de-registration response which contains the status of the PINE de-registration request. If the PINE is successfully de-registered, the PIN server deletes all the configuration information it has related to the PIN element that is being de-registered. If the de-registration procedure fails, the PIN server should give the failure response to indicates that indicates the cause of de-registration request failure.

* * * Next Change * * * *

##### 8.4.2.2.Y	PINE de-registration indirectly to PIN server

Figure 8.4.2.2.Y-1 illustrates PINE De-registration request to PIN Server indirectly via PEMC based on request/response model.

Pre-conditions:

1.	The PEMC has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server;

2.	The UE Identifier or PIN client Identifier is available;

3.	The PEMC has been authorized to communicate with the PIN server;

4.	The PINE/PEGC has already received the IP address of PEMC.

Figure 8.4.2.2.Y-1: PINE de-registration indirectly to PIN server

- The PINE (including PINE/PEMC/PEGC) sends a PINE de-registration Request to the PEMC. The request includes the security credentials of the PINE received during authorization procedure and also the request may include the GPSI, MAC address, vendor name, device description, PIN element ID/PIN client ID, PIN ID, PINE Address.

- The PEMC sends the PINE de-registration request to PIN server.

- The PIN server checks whether the UE identified by the GPSI has active registration with the PIN server. The PIN server responds to the PINE with PINE de-registration response which contains the status of the PINE de-registration request. If the PINE is successfully de-registered, the PIN server deletes all the configuration information it has related to the PIN element that is being de-registered. If the de-registration procedure fails, the PIN server should give the failure response that indicates the cause of de-registration request failure.

- The PEMC sends the response received in step 3 to the PINE/PEGC.

* * * Next Change * * * *

##### 8.4.2.2.Z	PINE update registration directly to PIN server

Figure 8.4.2.2.Z-1 illustrates PIN update registration procedure based on request/response model.

Pre-conditions:

1.	The PINE has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server and is already registered with the PIN server;

2.	The UE Identifier is available;

3.	The PINE has been authorized to communicate with the PIN server;

Figure 8.4.2.2.Z-1: PINE update registration directly to PIN server

1.	The PINE (including PINE/PEMC/PEGC) sends a PINE update registration request to the PIN server. The request includes the security credentials of the PINE received during authorization procedure and also the request may include the GPSI, MAC address, vendor name, device description, PIN element ID/PIN client ID, PINE Address.

2.	The PIN server checks whether the UE identified by the GPSI has active registration with the PIN server.

3.	The PIN server responds to the PINE with PINE update registration response which contains the status of the PINE update registration request. If the update registration procedure fails, the PIN server should give the failure response to indicates that indicates the cause of update registration request failure.

* * * Next Change * * * *

##### 8.4.2.3.A	PINE de-registration request

Table 8.4.2.3.A-1 describes information elements in the PINE de-registration request from the PINE (including PEMC, PEGC, PINEs) to the PIN server.

Table 8.4.2.3.A-1: PINE de-registration request

| Information element | Status | Description |
| --- | --- | --- |
| UE Identifier | M | The identifier of the hosting UE (i.e. GPSI or identity token) of PINE/PEMC/PEGC |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |
| MAC address | O | MAC address of the requested PINEs. |
| Vendor name | O | The vendor name of the PINE |
| Device description | O | Give the description of the device. |
| PINE Address | O | The IP address of PINE, if available. |

##### 8.4.2.3.B	PINE de-registration response

Table 8.4.2.3.B-1 describes information elements in the PINE de-registration response from the PIN server to the PINE (including PEMC, PEGC, PINEs).

Table 8.4.2.3.B-1: PINE de-registration response

| Information element | Status | Description |
| --- | --- | --- |
| De-registration status | M | Indicates whether the PINE de-registration request was successful or not. Includes the cause of the failure if the de-registration fails. |

##### 8.4.2.3.C	PINE update registration request

Table 8.4.2.3.C-1 describes information elements in the PINE update registration request from the PINE (including PEMC, PEGC, PINEs) to the PIN server.

Table 8.4.2.3.C-1: PINE update registration request

| Information element | Status | Description |
| --- | --- | --- |
| UE Identifier | M | The identifier of the hosting UE (i.e. GPSI or identity token) of PINE/PEMC/PEGC |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |
| MAC address | O | MAC address of the requested PINEs. |
| Vendor name | O | The vendor name of the PINE |
| Device description | O | Give the description of the device. |
| PINE Address | O | The IP address of PINE, if available. |

##### 8.4.2.3.D	PINE update registration response

Table 8.4.2.3.D-1 describes information elements in the PINE de-registration response from the PIN server to the PINE (including PEMC, PEGC, PINEs).

Table 8.4.2.3.D-1: PINE update registration response

| Information element | Status | Description |
| --- | --- | --- |
| Update registration status | M | Indicates whether the PINE update registration request was successful or not. Includes the cause of the failure if the update registration fails. |