3GPP TSG-SA WG6 Meeting #56	S6-232272

Goteborg, Sweden, 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0150 | rev | - | Current version: | 18.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of profile storing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | one2many |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Normative words are missing. Pre-condition 2 refers to clause 8.1.3 and subsequently repeats what is specified in clause 8.1.3 and also sets a pre-condition for optional elements. Step 2 specifies that if an individual registration request is succesful the MSGin5G Server stores the profiles from all registration requests (even from those that failed). Step 3 requires rewording for readability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Included normative words where appropriate. Removal of repeated text. Storing the profile is only done for the registration requests that are succesful. Step 3 is reworded. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.2.9	Non-MSGin5G UE bulk registration

Non-MSGin5G UEs (i.e., Legacy 3GPP UEs or Non-3GPP UEs) are connected to the MSGin5G Server through a Message Gateway. The Message Gateway may decide to use bulk registration procedure specified in this clause based on service policy. The procedure for Non-MSGin5G UE bulk registration is illustrated in figure 8.2.9-1.

Pre-conditions:

1.	The Message Gateway has been pre-configured with the MSGin5G Server address.

2.	The Message Gateway has been configured with the necessary information as specified in clause 8.1.3.

3.	A secured connection has been established between the Message Gateway and the MSGin5G Server.

Figure 8.2.9-1: Non-MSGin5G UE bulk registration

1.	The Message Gateway determines to perform Non-MSGin5G UE bulk registration.

NOTE:	The conditions of whether the Non-MSGin5G UE bulk registration is used are implementation specific and out of the scope of the current specification.

2.	The Message Gateway shall send a Non-MSGin5G UE bulk registration request to the MSGin5G Server. The Information Elements specified in table 8.2.9-1 shall be included in the Non-MSGin5G UE bulk registration request.

Table 8.2.9-1: Non-MSGin5G UE bulk registration request

| Information element | Status | Description |
| --- | --- | --- |
| Number of individual MSGin5G UE registration requests | M | Indicates total number of Non-MSGin5G UE registration requests which are bulked in this Non-MSGin5G UE bulk registration request |
| List of individual Non-MSGin5G UE information | M | Each element in this list contains information as specified in Table 8.2.3-1. |

3.	Upon receiving the Non-MSGin5G UE bulk registration request, the MSGin5G Server shall process each of the UE registrations from the list individually as specified in clause 8.2.3. For each individual Non-MSGin5G UE  registration that is successful, the MSGin5G Server shall store the UE Service ID and associated Non-MSGin5G UE Profile information..

Editor's note:	Security aspects of Non-MSGin5G UE bulk registration is the responsibility of SA3.

4.	The MSGin5G Server shall include all corresponding MSGin5G UE registration responses in a Non-MSGin5G UE bulk registration response and shall send it to the Message Gateway. The Information Elements specified in table 8.2.9-2 shall be included in the Non-MSGin5G UE bulk registration response.

Table 8.2.9-2: MSGin5G UE bulk registration response

| Information element | Status | Description |
| --- | --- | --- |
| Number of individual Non-MSGin5G UE registration responses | M | Indicates total number of Non-MSGin5G UE registration responses which are bulked in this MSGin5G UE bulk registration response |
| List of individual Non-MSGin5G UE registration response | M | Each element in this list contains information as specified in Table 8.2.1-4. |