3GPP TSG-SA WG6 Meeting #56	S6-232275

Goteborg, Sweden, 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0153 | rev | - | Current version: | 18.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on message segmentation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | one2many |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Normative words are missing. In 8.5.2: Step 2: It is unclear where the size restriction comes from. Step 5: "Status request" is the wrong name for the request. It is unclear to which message NOTE 3 refers to; no such message is specified or shown in the figure. In 8.5.3 and 8.5.4: Step 2: It is unclear where the size restriction comes from. It is unclear to which message NOTE 2 refers to; no such message is specified or shown in the figure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Inserted normative words where appropriate. In 8.5.2: Step 2: Clarification where the size restriction is imposed. Step 5: renamed request. Note 3 is deleted. In 8.5.3 and 8.5.4: Step 2: Clarification where the size restriction is imposed. Note 2 is deleted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification may lead to implementation problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.2, 8.5.3, 8.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.5.2	Application-to-Point Segmentation and Reassembly

Figure 8.5.2-1 shows the MSGin5G message segmentation and reassembly procedure for Application-to-Point MSGin5G message use cases (e.g. AOMT).

NOTE 1:	Segmentation can also be done by the Application Server, in this case the Application Server will create a segmented message and send it as a regular MSGin5G message. In this case it is implementation specific and out of the scope of the current specification.

Pre-conditions:

1.	A UE hosts an MSGin5G Client and an Application Client.

2.	The MSGin5G Client has registered with the MSGin5G Server.

3.	An Application Server needs to deliver application data to the Application Client on the UE and the size of the application data exceeds the maximum allowed packet size (e.g. due to limitation by the UE's access network transport).

Figure 8.5.2-1: Application-to-Point MSGin5G Message Segmentation and Reassembly

1.	An Application Server shall send a message to an MSGin5G Server that targets an Application Client on a UE.

2.	The MSGin5G Server shall compare the size of the received message to the maximum allowed packet size that is supported for message delivery to the targeted UE and may detect that the size exceeds the limit. As a result, the MSGin5G Sever shall segment the received message into a set of segmented messages. Within each segmented message, the information elements defined in Table 8.3.3-1 shall be included to enable the MSGin5G Client on the targeted UE to reassemble the segmented messages, with following clarifications.

a)	The MSGin5G message request shall include following information elements from Table 8.3.3-1:

i)	Originating AS Service ID, Recipient UE Service ID, Message ID, Segmentation Set Identifier and Message segment number in each segmented message

ii)	Delivery status required and Total number of message segments, only if it is the first segment of the message

iii)	Last Segment Flag, only if it is the last segment of the message.

3.	The MSGin5G Server shall send each segmented message to the targeted UE. If any segment is not received within the expected time (based on configuration) then proceed to step 4.

4.	If MSGin5G Client has received all segments (determined based on First segment and Last Segment), the MSGin5G Client shall reassemble all the segmented messages into a single MSGin5G message based on the information elements mentioned in step 2. If not all segments are received within expected time, then the MSGin5G Client shall recover the segments as described in clause 8.5.6 MSGin5G message segment recovery procedure, before continuing with rest of the steps.

NOTE 2:	When no further segments are received within expected time and if both first segment and last segment are missing, recovery can be initiated as described in clause 8.5.6 MSGin5G message segment recovery procedure for recovering the first segment and then for remaining segments.

5.	The MSGin5G Client shall send Message received confirmation request to the MSGin5G Server. The information elements defined in Table 8.5.2-2 shall be included in the request. The result information element shall contain "success" if the reassembly of the segments is success. Otherwise, the result information element shall contain "failure".

Table 8.5.2-2: Message Received Confirmation Request Information Elements

| Information element | Status | Description |
| --- | --- | --- |
| Segmentation Set Identifier | M | The Segmentation Set Identifier as received in segments. |
| Result | M | Indicates the "success" or "failure" |

6.	If reassembly of segments is successful, the MSGin5G Client shall deliver the contents of the MSGin5G message to the targeted Application Client. If MSGin5G Client has not received all messages (even after recovery procedure) or reassembly of segments failed for any reason (e.g. corrupt data) then the MSGin5G Client will notify receiving of a failed message to the Application Client.

### 8.5.3	Point-to-Application Message Segmentation and Reassembly

Figure 8.5.3-1 shows the MSGin5G message segmentation and reassembly procedure for Point-to-Application MSGin5G message use cases (e.g. MOAT).

Pre-conditions:

1.	A UE hosts an MSGin5G Client and an Application Client.

2.	The MSGin5G Client registered with the MSGin5G Server.

3.	An Application Client on the UE needs to deliver application data to an Application Server and the size of the application data exceeds the maximum allowed packet size (e.g. due to limitation by the UE's access network transport).

Figure 8.5.3-1: Point-to-Application MSGin5G Message Segmentation and Reassembly

1.	An Application Client on a UE sends a message to an MSGin5G Client that targets an Application Server and that has a size that exceeds the maximum allowed packet size.

2.	The MSGin5G Client shall compare the size of the received message to the maximum allowed packet size that is supported for message delivery to the MSGin5G Server and may detect that the size exceeds the limit. As a result, the MSGin5G Client shall segment the received message into a set of segmented messages such that each segmented message can fit within the maximum allowed packet size. Within each segmented message, the information elements defined in Table 8.3.2-1 shall be included to enable the MSGin5G Server to reassemble the segmented messages, with following clarifications.

a)	The MSGin5G message request shall include following information elements from Table 8.3.2-1:

i)	Originating UE Service ID, Recipient AS Service ID, Message ID, Segmentation set identifier and Message segment number in each segmented message

ii)	Delivery status required and Total number of message segments, only if it is the first segment of the message

iii)	Last segment flag, only if it is the last segment of the message.

3.	The MSGin5G Client shall send each segmented message to the MSGin5G Server. If any segment is not received within the expected time (based on configuration) then proceed to step 4.

4.	If MSGin5G Server has received all segments (determined based on first segment and last Segment), the MSGin5G Server shall reassemble all the segmented messages into a single MSGin5G message based on the information elements mentioned in step 2. If not all segments are received within expected time, then the MSGin5G Server shall recover the segments as described in clause 8.5.6 MSGin5G message segment recovery procedure, before continuing with rest of the steps.

NOTE 1:	When no further segments are received within expected time and if both first segment and last segment are missing, recovery can be initiated as described in clause 8.5.6 MSGin5G message segment recovery procedure for recovering the first segment and then for remaining segments.

5.	The MSGin5G Server shall send a Message received confirmation to the MSGin5G Client. The information elements defined in Table 8.5.2-2 shall be included in the request. The result information element shall contain "success" if the reassembly of the segments is success. Otherwise, the result information element shall contain "failure".

6.	If reassembly of segments is successful, the MSGin5G Server shall deliver the contents of the MSGin5G message to the targeted Application Server. If MSGin5G Server has not received all messages (even after recovery procedure) or reassembly of segments failed for any reason (e.g. corrupt data) then the MSGin5G Server shall notify receiving of a failed message to the Application Server.

### 8.5.4	Point-to-Point Message Segmentation and Reassembly

Figure 8.5.4-1 shows the MSGin5G message segmentation and reassembly procedure for Point-to-Point MSGin5G message use cases (e.g. MOMT).

If the recipient UE is a non-MSGin5G UE, the reassembly is performed by the Message Gateway.

This procedure assumes that a UE is only aware of the maximum payload size of the delivery mechanism it is currently using, and it is not aware of the maximum payload size of the recipient UE.

Pre-conditions:

1.	Both UEs host an MSGin5G Client and an Application Client.

2.	The MSGin5G Clients registered with the MSGin5G Server.

3.	An Application Client on the UE needs to deliver application data to an Application Client on another UE and the size of the application data exceeds the allowed maximum packet size (e.g. due to limitation by the UE's access network transport).

Figure 8.5.4-1: Point-to-Point MSGin5G Message Segmentation and Reassembly

1.	An Application Client on UE 1 sends a message to MSGin5G Client 1 that targets Application Client on UE 2.

2.	The MSGin5G Client 1 shall compare the size of the received message to the maximum allowed packet size that is supported for message delivery to the MSGin5G Server and may detect that the size exceeds the limit of the originating UE. As a result, the MSGin5G Client shall segment the received message into a set of segmented messages such that each segmented message can fit within the maximum allowed packet size. Within each segmented message, the information elements defined in Table 8.51.2-1 shall be included to enable reassembly of the segmented messages, with following clarifications.

a)	The MSGin5G message request shall include following information elements from Table 8.3.2-1:

i)	Originating UE Service ID, Recipient UE Service ID, Message ID, Segmentation set identifier and Message segment number in each segmented message

ii)	Delivery status required and Total number of message segments, only if it is the first segment of the message

iii)	Last segment flag, only if it is the last segment of the message.

3.	The MSGin5G Client 1 shall send each segmented message to the MSGin5G Server. If any segment is not received within the expected time (based on configuration) then proceed to step 4.

4.	The MSGin5G Server shall check if each segment does not exceed the configured maximum packet size of the targeted UE. If the maximum packet size is not exceeded, then the MSGin5G Server proceeds with step 6.

If the maximum packet size is exceeded, the MSGin5G Server shall perform the following operations:

a)	If all segments are received within expected time, then the MSGin5G Server shall reassemble subsequent segmented messages into a single MSGin5G message until the Last segment flag indication is received. The re-assembled message shall then be segmented such that each segment is smaller than the maximum allowed packet size of the targeted UE. Within each segmented message, the information elements as mentioned in step 2 shall be included to enable reassembly at the target, then proceed with step 6.

b)	If not all segments are received within expected time, then the MSGin5G Server shall act as Message Receiver to recover the segments as described in clause 8.5.6 MSGin5G message segment recovery procedure, before continuing with rest of the steps. If all segments are received after recovery procedure, then the MSGin5G Server skips to step 4-a, otherwise proceed to step 5.

5.	The MSGin5G Server shall send Message received confirmation to the MSGin5G Client 1. The information elements defined in Table 8.5.2-2 shall be included in the request, the Result information element is "failure", and further steps are not executed.

6.	The MSGin5G Server shall send each segmented message to the MSGin5G Client 2. If any segment as a separate message is not received within the expected time (based on configuration) then proceed to step 8.

7.	The MSGin5G Client 2 shall reassemble all the segmented messages into a single MSGin5G message based on the information elements defined mentioned in step 2. If not all segments are received within expected time, then the MSGin5G Client 2 shall act as Message receiver to recover the segments as described in clause 8.5.6 MSGin5G message segment recovery procedure, before continuing with rest of the steps.

NOTE 1:	Steps 7, 8 and 9 can also be performed by the MSGin5G Client functionality in the Message Gateway in case the recipient UE is a non-MSGin5G UE.

8.	The MSGin5G Client 2 shall send Message received confirmation to the MSGin5G Client. The information elements defined in Table 8.5.2-2 shall be included in the request. The Result information element shall contain "success" if the reassembly of the segments is successful. Otherwise, the Result information element shall contain "failure".

9.	The MSGin5G Client 2 shall deliver the contents of the MSGin5G message to the targeted Application Client. If MSGin5G Client has not received all messages (even after recovery procedure) or reassembly of segments failed for any reason (e.g. corrupt data) then the MSGin5G Client shall notify receiving of failed message to Application Client.