3GPP TSG-SA WG6 Meeting #56	S6-232276

Goteborg, Sweden, 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0154 | rev | - | Current version: | 18.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Segment recovery procedure doesn't work |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | one2many |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The recovery procedure applies equally well to Topic messages as it applies to Group messages, or any other scenarios. If the Message receiver received a segmented message with failed segments, the procedure in 8.5.6 specifies a recovery procedure. However, the Message receiver doesn't know which entity performed the segmentation of the message and furthermore addressing any Message sender is not possible with the IEs from table 8.5.6-1 (sender Service ID is missing). If the segmented message was a group message or a topic message, the sender wouldn't know which receiver asked for the missing segments to be resent (receiver Service ID is missing). In short: the procedure doesn't work. In step 4 of the recovery procedure all segments are resent, not only those listed in the List of Segment range. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposed in the CR is a staged recovery procedure: The sender sends the segmented message to the MSGin5G Server and the MSGin5G Server ensures that all segments are received correctly. The Message receiver at the end-point requests any missing segments from the MSGin5G Server. Only the listed segments are resent, not all segments. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The recovery procedure doesn't work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.5.6	MSGin5G Message Segment Recovery

Figure 8.5.6-1 illustrates an MSGin5G message segmentation recovery procedure. The procedure is applicable to Application-to-Point messages, Point-to-Application messages, Point-to-Point message, Topic messages and Group messages.

The procedure applies when:

- The Message receiver is an MSGin5G UE, and the Message sender is the MSGin5G Server, and when

- The Message receiver is the MSGin5G Server, and the Message sender is an MSGin5G UE.

Pre-condition:

1.	The Message sender has delivered segmented messages to Message receiver with one or more missing segments.

Figure 8.5.6-1: MSGin5G Message Segmentation and Reassembly

1.	The Message Receiver detects that one or more segments are missing to reassemble the complete message.

2.	The Message receiver shall send the Segment recovery request to the Message sender. The information elements defined in Table 8.5.6-1 shall be included in the request message.

Table 8.5.6-1: Segment Recovery Request Information Elements

| Information element | Status | Description |
| --- | --- | --- |
| Message receiver Service ID | M | Service ID of the Message receiver that sends the segment recovery request. |
| Message sender Service ID | M | Service ID of the Message sender that needs to resend the requested segments. |
| Segmentation Set Identifier | M | The Segmentation Set Identifier as received in segments. |
| List of Segment range | M | List of Segment range which the client wants to recover, each segment range consist of start and end sequence number of missing segments e.g. (5-7, 10-10, 15-19) |

3.	The Message sender shall send a Segment recovery acknowledgement to the Message receiver.

4.	The Message sender shall send each segmented message listed in the List of Segment range to the Message receiver within an individual access network transport packet. If any segment is not received within the expected time (based on configuration) then the Message receiver may consider recovery as failed or may initiate the procedure again with updated list of segment range.

NOTE:	The MSGin5G message segment recovery procedure may repeat based on the configuration.

5.	If Message receiver has received all segments (determined based on First segment and Last Segment), the Message receiver shall reassemble all the segmented messages into a single MSGin5G message.