3GPP TSG-SA WG6 Meeting #56	S6-232281

Goteborg, Sweden, 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0159 | rev | - | Current version: | 18.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to procedure for inter-PLMN message exchange |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | one2many |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Text does not contain normative wording. The protocol for message exchange between MSGin5G Servers in different PLMNs is unspecified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Insert normative wording where appropriate. Specify the protocol for message exchange between MSGin5G Servers. For Topic messages it is not as simple as forwarding the message the Server has received from the sender to the other Server 1-on-1. In case Server 1 serves the Service ID of the endpoint, Server 1 delivers the message to the endpoint as specified in clause 8.3.3. However, in case Server 1 has subscribed to a Topic on behalf of end-points that it serves, the message that is received by Server 2 shall still include the Topic ID and cannot include the Service ID of the endpoints; hence this message shall be as specified in clause 8.3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing protocol for message exchange between MSGin5G servers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.5.2, 8.7.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 8.7.5.2	Inter-PLMN message exchange procedure

Pre-condition:

1.	The Message Sender (e.g. MSGin5G Client 1 in MSGin5G UE 1) is registered to the MSGin5G Server 1 in one PLMN.

2.	The Message Receiver (e.g. MSGin5G Client 2 in MSGin5G UE 2) is registered to the MSGin5G Server 2 in another PLMN.

3.	MSGin5G Server 1 and MSGin5G Server 2 have established a secured connection.

Editor's Note: TS 33.501 does not specify how the connection is secured.

Figure 8.7.5.2-1 shows message delivery between MSGin5G endpoints in different PLMNs, where the Message Sender is registered in MSGin5G Server 1 and the Message Receiver is registered in MSGin5G Server 2.

Figure 8.7.5.2-1: Message delivery between MSGin5G UEs in different PLMNs

1.	The Message Sender sends an MSGin5G message request to MSGin5G Server 1 in PLMN 1 as specified in clause 8.3.2.

2.	The MSGin5G Server 1 shall analyse the target UE Service ID and determines that the message is targeted to the Message Receiver in PLMN 2, shall authenticate that the Message Sender is allowed to send a message to the Message Receiver, and then the MSGin5G Server 1 shall forward the MSGin5G message request to MSGin5G Server 2 in PLMN 2. The MSGin5G message request shall contain the Information Elements as specified in table 8.3.3-1.

3.	MSGin5G Server 2 forwards the MSGin5G message request to the Message Receiver as specified in clause 8.3.3.

4-6.	If the message delivery status report is requested, the Message Receiver sends a message delivery status report to Message Sender as per procedure specified in clause 8.3.4 and 8.3.5.

#### 8.7.5.3	Inter-PLMN message exchange procedure based on Messaging Topic

Figure 8.7.5.3-1 shows the inter-PLMN Message delivery based on Messaging Topic when MSGin5G Server 2 forwards the Messaging Topic subscription request from the MSGin5G UE/Application Server served by it to MSGin5G Server 1 as specified in clause 8.8.4.

Pre-condition:

1.	The Message Sender (e.g. an MSGin5G UE, a Message Gateway on behalf of a Non-MSGin5G UE or an Application Server) is registered to the MSGin5G Server 1 in PLMN 1.

2.	The Message Receiver (e.g. an MSGin5G UE, a Message Gateway on behalf of a Non-MSGin5G UE or an Application Server) is registered to the MSGin5G Server 2 in PLMN 2.

3.	MSGin5G Server 1 and MSGin5G Server 2 have established a secured connection.

4.	MSGin5G Client or Application Server served by MSGin5G Server 2 has subscribed to a Messaging Topic with the MSGin5G Server 1 as specified in clause 8.8.4 via MSGin5G Server 2.

Figure 8.7.5.3-1: Message delivery between MSGin5G UEs in different PLMNs

1.	The Message Sender sends an MSGin5G message request (if the sender is an MSGin5G UE) or API request (if the sender is an Application Server or a Message Gateway on behalf of a Non-MSGin5G UE) to MSGin5G Server 1 in PLMN 1 as specified in clause 8.3.2. A Messaging Topic is included in this request.

2.	The MSGin5G Server 1 shall check the subscriptions of the Messaging Topic included in the request and obtains the UE Service ID/AS Service ID of the Message Receivers. The MSGin5G Server 1 determines that the message with this Messaging Topic is needed to be delivered to an MSGin5G UE/Application Server served by Message Server 2 in PLMN 2 based on the UE Service ID/AS Service ID of the Message Receiver. The MSGin5G Server 1 then shall send the MSGin5G message to MSGin5G Server 2. The MSGin5G message request shall contain the Information Elements as specified in table 8.3.3-1.

3	The MSGin5G Server 2 delivers the MSGin5G message request to Messager Receiver based on the UE Service ID/AS Service ID in the message as specified in clause 8.3.3.

4-6.	If the message delivery status report is requested, the Message Receiver sends message delivery status report to Message Sender as per procedure specified in clause 8.3.4 and 8.3.5.

Figure 8.7.5.3-2 shows the inter-PLMN Message delivery to subscribing service endpoints based on Messaging Topic when MSGin5G Server 2 subscribes the Messaging Topic on behalf of all MSGin5G UEs/Application Servers served by it as specified in clause 8.8.4.

Pre-condition:

1.	The Message Sender (e.g. an MSGin5G UE, a Message Gateway on behalf of a Non-MSGin5G UE or an Application Server) is registered to the MSGin5G Server 1 in PLMN 1.

2.	The Message Receiver (e.g. an MSGin5G UE, a Message Gateway on behalf of a Non-MSGin5G UE or an Application Server) is registered to the MSGin5G Server 2 in PLMN 2.

3.	MSGin5G Server 1 and MSGin5G Server 2 have established a secured connection.

4.	MSGin5G Server 2 subscribed to a Messaging Topic with the MSGin5G Server 1 as specified in clause 8.8.4. A Messaging Topic with the MSGin5G Server 2 address of the subscriber has been created on MSGin5G Server 1.

5.	The MSGin5G Client or Application Server served by MSGin5G Server 2 subscribed to the Messaging Topic with the MSGin5G Server 2. A Messaging Topic with the UE Service ID/AS Service ID of the subscriber has been created on MSGin5G Server 2.

Figure 8.7.5.3-2: Message delivery between MSGin5G UEs in different PLMNs

1.	The Message Sender sends an MSGin5G message request (if the sender is an MSGin5G UE) or API request (if the sender is an Application Server or a Message Gateway) to MSGin5G Server 1 in PLMN 1 as specified in clause 8.3.2. A Messaging Topic is included in this request.

2.	The MSGin5G Server 1 shall check the subscription of the Messaging Topic included in the request. The MSGin5G Server 1 determines that the message with this Messaging Topic is needed to be delivered to the Message Server 2 in PLMN 2 and shall send the MSGin5G message to MSGin5G Server 2. The MSGin5G message request shall contain the Information Elements as specified in table 8.3.2-1.

3a	The MSGin5G Server 2 shall check the subscription of the Messaging Topic included in the inbound request. The MSGin5G Server 2 shall deliver the MSGin5G message request to Messager Receiver 1 based on its UE Service ID/AS Service ID as specified in clause 8.8.2.

3b	The MSGin5G Server 2 also delivers the MSGin5G message request to Messager Receiver 2 which subscribed to this Messaging Topic as specified in clause 8.8.2.

4a-6a.	If the message delivery status report is requested, the Message Receiver 1 sends message delivery status report to Message Sender as per procedure specified in clause 8.3.4 and 8.3.5.

4b-6b.	If the message delivery status report is requested, the Message Receiver 2 sends message delivery status report to Message Sender as per procedure specified in clause 8.3.4 and 8.3.5.