3GPP TSG-SA WG6 Meeting #56	S6-232287

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0408 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for group de-affiliation for specific UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh4MCPTT |  |  |  |  |  | Date: |  |  | 2023-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Information flow “MC service group de-affiliation request (MC service server – MC service server)” (clause 10.8.2.7) does not contain the information element “Scope of De-affiliation request”. This information element is provided from the MC service client to the MC service server (clause 10.8.2.6) but, if another MC service server is involved, it needs also to be provided to this partner MC service server (see also procedure 10.8.4.3). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Information element “Scope of De-affiliation request” is added to the “MC service group de-affiliation request” between two MC service servers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The information element “Scope of De-afflilation request” is not provided to the partner MC service server, which will give a wrong guidance to stage 3 development. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.8.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * 1st Change * * * *

#### 10.8.2.7	MC service group de-affiliation request (MC service server – MC service server)

Table 10.8.2.7-1 describes the information flow MC service group de-affiliation request between the MC service servers.

Table 10.8.2.7-1: MC service group de-affiliation request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the originator who triggers the MC service group de-affiliation request. |
| MC service group ID list | M | A list of one or more MC service group IDs to which the originator intends to de-affiliate and is defined in the destination MC system. |
| Scope of De-affiliation request | M | Indicating, whether de-affiliation is requested for this specific MC service client only, or for all MC service clients where this MC service ID has logged on. |

* * * End of Change * * * *