3GPP TSG-SA6 Meeting #56	S6-232292

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0318 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of location Information Element to FD message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Solutions UK Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2023-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, SDS message has provision to set location. This CR proposes to add the location Information Element in FD Message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of location IE in the following tables: 7.5.2.1.5-1, 7.5.2.1.5-2, 7.5.2.1.5-3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Location cannot be set in FD messages |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 7.5.2.1.5	MCData FD request (using HTTP)

Table 7.5.2.1.5-1 describes the information flow for the MCData FD request (in subclause 7.5.2.4.2) sent from the MCData client to the MCData server.

Table 7.5.2.1.5-1: MCData FD request (using HTTP) from MCData client to MCData server

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending the file |
| Functional alias | O | The functional alias associated with MCData user sending the file |
| MCData ID (see NOTE) | O | The identity of the MCData user receiving the file |
| Functional alias (see NOTE) | O | The associated functional alias of the MCData user identity towards which the data is sent. |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Disposition indication | O | Indicates whether file download completed report is expected or not |
| Download indication | O | Indicates mandatory download |
| Application metadata container | O | Implementation specific information that is communicated to the recipient |
| Content reference | M | URL reference to the content and file metadata information |
| Emergency indicator | O | Indicates that the data request is for MCData emergency communication |
| Deposit file indication | O | Indicates whether the file to be stored into the MCData message store account of the MCData user |
| Location | O | Location of the Originating MCData user sending the FD message |
| NOTE: Either the MCData ID or the functional alias must be present. |  |  |

Table 7.5.2.1.5-2 describes the information flow for the MCData FD request (in clause 7.5.2.4.2) sent from an MCData server to a partner MCData server.

Table 7.5.2.1.5-2: MCData FD request (using HTTP) from an MCData server to MCData server

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending the file |
| Functional alias | O | The associated functional alias of the MCData user identity sending the file |
| MCData ID | M | The identity of the MCData user receiving the file |
| Functional alias | O | The associated functional alias of the MCData user identity towards which the data is sent. |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Disposition indication | O | Indicates whether file download completed report is expected or not |
| Download indication | O | Indicates mandatory download |
| Application metadata container | O | Implementation specific information that is communicated to the recipient |
| Content reference | M | URL reference to the content and file metadata information |
| Emergency indicator | O | Indicates that the data request is for MCData emergency communication |
| Location | O | Location of the Originating MCData user sending the FD message |

Table 7.5.2.1.5-3 describes the information flow for the MCData FD request (in clause 7.5.2.4.2) sent from the MCData server to the MCData client.

Table 7.5.2.1.5-3: MCData FD request (using HTTP) from MCData server to MCData client

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending the file |
| Functional alias | O | The associated functional alias of the MCData user sending the file |
| MCData ID | M | The identity of the MCData user receiving the file |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Disposition indication | O | Indicates whether file download completed report is expected or not |
| Download indication | O | Indicates mandatory download |
| Application metadata container | O | Implementation specific information that is communicated to the recipient |
| Content reference | M | URL reference to the content and file metadata information |
| Emergency indicator | O | Indicates that the data request is for MCData emergency communication |
| Location | O | Location of the Originating MCData user sending the FD message |

* * * End of Changes * * * *