3GPP TSG-SA WG6 Meeting #56	S6-232302

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0412 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MCGWUE initial configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2023-08-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently, the list of permitted MC gateway UE parameter is added to MCPTT, MCVideo and MCData user profile configuration data in 3GPP TS 23.379, 3GPP TS 23.281, and 3GPP TS 23.282, respectively. However, this parameter is required to enable the MC gateway client hosted by a non-3GPP device to perform connection authorisation procedure towards an MC gateway UE correctly. Please refer to pre-condition number 2 in clause 11.5.1.2.3. At this point, the MC service client has not received its user profile to be able to obtain this information from. Additionally, this information is utilized by the MC gateway client and not the MC service client. Please refer to clause 10.1.1.1. Therefore, clause A.9 is added to include this parameter. Furthermore, clause A.7 is reformulated to reflect the initial configuration data of an MC gateway UE. In table A.7-1, the GW MC service ID is described to be a parameter for permitted gateway MC services. However, the MC gateway UE is not permitted to gateway MC services. It is rather the parameter to support gateway MC services. Please refer to step 2 in clause 11.5.12.3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - adding clause A.9 describing the initial MC gateway client configuration data. - updating clause A.7 to have the list of supported gateway MC services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The initial configuration data of the MC gateway client is essential to perform the initial connection towards MC gateway UE, namely, the connection authorisation. If the initial configuration data is not correctly described, stage 3 work will be delayed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7, A.9 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# A.7	Initial MC gateway UE configuration data

The initial MC gateway UE configuration data is essential to the MC gateway UE to successfully connect MC clients to the MC system. The initial MC gateway UE configuration data can be the same or different across MC gateway UEs.

Data in table A.7-1 is provided to the MC gateway UE during the bootstrap process and can be configured on the MC gateway UE offline using the CSC-11 reference point or via other means.

Table A.7-1: Initial MC gateway UE configuration data (on-network)

| Reference | Parameter description |
| --- | --- |
| Subclause 5.15 of 3GPP TS 22.280 [3] | List of supported GW MC service IDs for MCPTT |
|  | > GW MC service ID |
| Subclause 5.15 of 3GPP TS 22.280 [3] | List of supported GW MC service IDs for MCVideo |
|  | > GW MC service ID |
| Subclause 5.15 of 3GPP TS 22.280 [3] | List of supported GW MC service IDs for MCData |
|  | > GW MC service ID |

NOTE:	Configured MC service IDs indicate the support of an MC service.

# * * * * Second change * * * *

# A.9	Initial MC gateway client configuration data

The initial MC service gateway client configuration data is essential to the MC gateway client, either hosted by or associated to a non-3GPP device to be able to perform connection authorisation and connect to an MC gateway UE.

Table A.9-1: Initial MC gateway client configuration data (on-network)

| Reference | Parameter description |
| --- | --- |
| Subclause 5.15 of 3GPP TS 22.280 [3] | List of permitted GW MC service ID to enable connecting to MC gateway UE for MCPTT |
|  | > GW MC service ID |
| Subclause 5.15 of 3GPP TS 22.280 [3] | List of permitted GW MC service ID to enable connecting to MC gateway UE for MCVideo |
|  | > GW MC service ID |
| Subclause 5.15 of 3GPP TS 22.280 [3] | List of permitted GW MC service ID to enable connecting to MC gateway UE for MCData |
|  | > GW MC service ID |

# * * * * End of changes * * * *