3GPP TSG-SA WG6 Meeting #56	S6-232311

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0418 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Complete ECS-ER API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-06-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | ECS-ER API is not specified for EAS info storage and retrieval. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Complete ECS-ER API for EAS info storage and retrieval. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete TS to support common EAS discovery and EAS sync. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2, 8.20.x (new), 8.20.x.1 (new), 8.20.x.2 (new), 8.20.x.2.1 (new), 8.20.x.2.2 (new), 8.20.x.2.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.7.2	APIs provided by the Edge Enabler Layer

Table 6.7.2-1 summarizes the APIs exposed by the ECS.

Table 6.7.2-1: APIs provided by the ECS

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eecs_ServiceProvisioning | EEC | 8.3 |
| Eecs_EESRegistration | EES | 8.4.4 |
| Eecs_TargetEESDiscovery | EES | 8.8.3.3 |
| Eecs_EASInfoManagement | EES | 8.20.2 |

Table 6.7.2-2 summarizes the APIs exposed the EES.

Table 6.7.2-2: APIs provided by the EES

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eees_EECRegistration | EEC | 8.4.2 |
| Eees_EASRegistration | EAS | 8.4.3 |
| Eees_EASDiscovery | EEC | 8.5 |
| Eees_UELocation | EAS | 8.6.2 |
| Eees_ACRManagementEvent | EAS | 8.6.3 |
| Eees_AppClientInformation | EAS | 8.6.4 |
| Eees_UEIdentifier | EEC, EAS | 8.6.5 |
| Eees_SessionWithQoS | EAS | 8.6.6 |
| Eees_TargetEASDiscovery | EAS, EES | 8.8.3.2 |
| Eees_AppContextRelocation | EEC, EAS | 8.8.3.4 |
| Eees_ACREvents | EEC | 8.8.3.5 |
| Eees_EELManagedACR | EAS | 8.8.3.6 |
| Eees_EECContextPull | EES | 8.9.4.2 |
| Eees_EECContextPush | EES | 8.9.4.3 |
| Eees_SelectedTargetEAS | EAS | 8.8.3.7 |
| Eees_ACRStatusUpdate | EAS | 8.8.3.8 |

NOTE:	The event exposure related APIs (e.g. Eees_EASDiscovery and Eees_ACREvents) can be realized as single event subscription API.

* * * Next Change * * * *

### 8.20.x	APIs

#### 8.20.x.1	General

Table 8.20.x.1-1 illustrates the API for EAS information management.

Table 8.20.x.1-1: EAS information management APIs

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eecs_EASInfoManagement | Store | Request/Response | EES |
|  | Get | Request/Response | EES |

#### 8.20.x.2	Eecs_EASInfoManagement API

##### 8.20.x.2.1	General

This clause describes the Eecs_EASInfoManagement API and its operations.

##### 8.20.x.2.2	Eecs_EASInfoManagement_Store operation

API operation name: Eecs_EASInfoManagement_Store

Description: The consumer requests EAS information storage service from the ECS.

Inputs: See clause 8.20.3.4.

Outputs: See clause 8.20.3.5.

See clause 8.20.2.3 for details of usage of this operation.

##### 8.20.x.2.3	Eecs_EASInfoManagement_Get operation

API operation name: Eecs_EASInfoManagement_Get

Description: The consumer obtains EAS information from the ECS.

Inputs: See clause 8.20.3.2.

Outputs: See clause 8.20.3.3.

See clause 8.20.2.2 for details of usage of this operation.

* * * END of Change * * * *