SA WG6 Meeting #56	S6-232325

Goteborg, Sweden, August 21 – 25, 2023	(revision of S6-23xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0006 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of PIN Modification after local PEMC failure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2023-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In section 8.5.10.2.2, it describes the case that how to PIN server to arrange another PINE-1 to be the new PEMC. Due to in section 8.5.12, the heartbeat procedure is designed to detect that the PEMC 2 is unavailable. So, step 1 can be replaced. And for step 6, that PIN notification procedure is the same usage as PIN status notify. For the ENs, it can be removed that to update the dynamic PIN profile to alige with the information flow. At last, due to the PIN server can also consume the heart beat to detect that the PEMC 2 is failure. So, the PIN server can directly trigger the step 4 and the step 1 – step 3 can be in dash line that as optional. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the ENs, and replace the PIN notification to PIN status notify. Set step 1 to step 3 as optional procedure, and the PIN server can directly decide the PEMC newly arrange. Reusing the heart beat mechanism to replace the step 1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some of the procedure in Figure 8.5.10.2.2-1 is unclear and misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.10.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

8.5.10.2.2	PIN modification after local PEMC failure

Figure 8.5.10.2.2-1 describes the PIN modification procedure to perform a PEMC role change due to the failure of the PEMC. An authorised administrator is the owner of the PIN and accesses PIN configuration using an application on a UE, which is one of the PEMC for the PIN. The authorised administrator can manage the PIN locally or through the 5G network. This procedure describes a PEMC (e.g. an authorized administrator on a UE) managing the PIN remotely via the 5G network.

The procedure may used e.g. when a PEMC is available on a UE for PIN management by an authorised administrator. When there is a local PEMC failure, the authorized administrator can be enabled to manage the PIN remotely, via the 5G network, using the following steps.

Pre-conditions:

1.	The PIN server has authorized the creation of the PIN.

2.	The authorized administrator is the owner of the PIN and has created the PIN.

3.	The authorized administrator configures the active PEMC to provide PIN management for the PIN. UE/PEMC1 is the inactive PEMC.

4.	The active PEMC, PEGC, PINE-1, PINE-2, and the UE/PEMC1 are members of the PIN. PINE-1 has PEMC capability.

5.  The authorized administrator leaves the local area of the PIN (e.g. in a home) and is able to access the PIN remotely through the 5G network. As a result, the authorized administrator is able to manage the PIN through the 5G network

Figure 8.5.10.2.2-1: PIN Modification after local PEMC failure

Step 1 to Step 3, and step 7 are optional and can be skipped if the PIN server detect the PEMC 2 is unavailable directly in step 4 by PIN heartbeat and decides the PINE 1 can be the new PEMC.

1.	 UE/PEMC1 detects PEMC2 is unavailable by PIN heartbeat.

2.	An authorized administrator on UE/PEMC1 sends a PIN configuration request to the PIN server through the 5G network. The request includes the security credentials of the authorized administrator, the UE ID, the PIN ID, PIN member ID, authorization type indicating the role change, and a request that PINE-1 be assigned the new PEMC.

3.	The PIN server processes the modification request and checks if the authorized administrator is allowed to modify the PIN. The PIN server verifies PINE-1 has the capability to serve as a PEMC using information in the PIN profile.

4. 	If the authorized administrator is allowed to perform PIN modification, the PIN server sends a PIN management request to PINE-1 to assign PINE-1 as the new PEMC and provides dynamic profile information to PINE-1.

5.	PINE-1 returns an accept response in the PIN management response to the PIN server.

6.	The PIN server notifies the other members of the PIN that PINE-1 will be the new PEMC for the PIN and updates t dynamic profile information.

7.	The PIN server sends a PIN configuration response to the PIN configuration request with updated information for the dynamic profile information to the UE/PEMC1.

8.	PIN communications resume with PINE-1 serving as the new PEMC.

* * * End of Change * * *