SA WG6 Meeting #56	S6-232330

Goteborg, Sweden, August 21 – 25, 2023	(revision of S6-23xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0011 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of PINE update for port number |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2023-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Section 8.4.2.3.6, some of the information, for example the port number can also be updated. So, this part should be aligned with section 8.4.2.3.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add port number into PINE update registration request |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some of the information is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

8.4.2.3.6	PINE update registration request

Table 8.4.2.3.6-1 describes information elements in the PINE update registration request from the PINE (including PEMC, PEGC, PINEs) to the PIN server.

Table 8.4.2.3.6-1: PINE update registration request

| Information element | Status | Description |
| --- | --- | --- |
| UE Identifier | M | The identifier of the hosting UE (i.e. GPSI or identity token) of PINE/PEMC/PEGC |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |
| MAC address | O | MAC address of the requested PINEs. |
| Vendor name | O | The vendor name of the PINE |
| Device description | O | Give the description of the device. |
| PINE Address | O | The IP address of PINE, if available. |
| Port number | M | Port number of PIN client on PINE/PEMC/PEGC to support PIN enabler layer communication. The port number is the port used by a PINE to expose a service within the PIN. |
| PINE Capabilities | O | Identify whether the PINE is capable of becoming a PEMC, a PEGC or both. |
| > Maximum number of PINEs (see NOTE) | O | Indicates the maximum number of PINE that can be managed by the PEMC or PEGC |
| NOTE: Only present if PINE Capabilities is present. |  |  |

* * * End of Change * * *