3GPP TSG-SA WG6 Meeting #56	S6-232345

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

Source:	Samsung

Title:	Pseudo-CR on EDGE-10 mapping to EWBI

Spec:	3GPP TR 23.958

Agenda item:	7.9

Document for:	Approval

Contact:	Sapan Shah (sapan.shah@samsung.com)

1. Introduction

This pCR introduces mapping of EDGE-10 interface to EWBI.

2. Reason for Change

EDGE-10 interface is defined in TS 23.558 as follows:

EDGE-10 reference point enables interactions between the ECS and another ECS. It supports: a) service provisioning information retrieval. b) registration and de-registration of the ECS to the ECS acting as edge repository; and c) retrieval or discovery of information about other ECS(s) of the federation.

EWBI allows an OP to itneract with another OP. So, based on description and usage of EDGE-10 interface, EDGE-10 can be mapped to EWBI as specifed in GSMA OPG PRD.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.958 (v0.6.0).

* * * First Change * * * *

## 6.2 Relationship between EDGEAPP architecture and GSMA OPG reference architecture

Figure 6.2-1 illustrates the relationship between EDGEAPP architecture and GSMA OPG reference architecture [3].

Figure 6.2-1: Relationship between EDGEAPP architecture and GSMA OPG reference architecture

EDGE-1 and EDGE-4 reference points can support similar function(s) as OP's User-Network interface (UNI), providing the Edge Enabler Client (corresponding to Edge/User Client in OP) with the information required to access the edge services. EDGE-1/EDGE-4 neither impact nor overlap with other existing 3GPP interfaces between the UE and the network, catering to the OP's requirements on UNI.

EDGE-2 and EDGE-8 reference points can support similar function(s) as OP's Southbound interface (SBI), through which the edge enabler layer (corresponding to the operator platform) access the 3GPP network capabilities and services. Specifically, EDGE-2 and EDGE-8 cater to the requirements of the SBI-NetworkResource interface. ECSP management system as specified in 3GPP TS 28.538 [8] caters to the requirements of OP's SBI-CloudResource interface.

EDGE-3 reference point can support similar function(s) as OP's Northbound interface (NBI), exposing the capabilities of Edge Enabler Server to the Edge Application Servers (EAS) hosted on the edge. OP's NBI also expands capabilities exposure to ASPs, for example to on-board applications to be deployed as EASs based on specific criteria.

EDGE-9 and EDGE-10 reference point can support similar function(s) as OP's East/Westbound interface (E/WBI), allowing the edge enabler layer to interact within and beyond its domains e.g., between operator platforms. OP's E/WBI focuses on use cases like user and application roaming or resource sharing across domains.

* * * End of Change * * * *