3GPP TSG-SA WG6 Meeting #56	S6-232349

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0433 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | APIs for Roaming and Federation and ENS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 8.19 specifies procedure for Common EAS announcement. It is required to define API for the same. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added API for Common EAS announcement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Common EAS announcement procedure will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.17.4 (new), 8.18.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.17.4	APIs

#### 8.17.4.1	General

Table 8.17.4.1-1 illustrates the API for AC registration.

Table 8.17.4.1-1: Eees_EECRegistration API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eecs-er_ECSRegistration | Request | Request/Response | ECS |
|  | Update |  |  |
|  | Deregister |  |  |
| Eecs-er_ECSDiscovery | Request | Request/Response | ECS |
|  | Subscribe | Subscribe/Notify |  |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |
| Eecs-er_ServiceProvInfoRetrieval | Request | Request/Response | ECS (Partner ECSP), ECS-ER (Partner ECSP) |
|  | Subscribe | Subscribe/Notify |  |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |

#### 8.17.4.2	Eecs-er_ECSRegistration API

##### 8.17.4.2.1	Eecs-er_ECSRegistration_Request operation

API operation name: Eecs-er_ECSRegistration_Request

Description: The consumer requests to register the ECS on the ECS-ER.

Inputs: See clause 8.17.3.2.

Outputs: See clause 8.17.3.3.

See clause 8.17.2.2.2 for details of usage of this operation.

##### 8.17.4.2.2	Eecs-er_ECSRegistration_Update operation

API operation name: Eecs-er_ECSRegistration_Update

Description: The consumer requests to update the registered information of the ECS on the ECS-ER.

Inputs: See clause 8.17.3.4.

Outputs: See clause 8.17.3.5.

See clause 8.17.2.2.3 for details of usage of this operation.

##### 8.17.4.2.3	Eecs-er_ECSRegistration_Deregister operation

API operation name: Eecs-er_ECSRegistration_Deregister

Description: The consumer requests to de-register the ECS from the ECS-ER.

Inputs: See clause 8.17.3.6.

Outputs: See clause 8.17.3.7.

See clause 8.17.2.2.4 for details of usage of this operation.

#### 8.17.4.3	Eecs-er_ECSDiscovery API

##### 8.17.4.3.1	Eecs-er_ECSDiscovery_Request operation

API operation name: Eecs-er_ECSDiscovery_Request

Description: The consumer requests to discover ECS of partner ECSP.

Inputs: See clause 8.17.3.8.

Outputs: See clause 8.17.3.9.

See clause 8.17.2.3.2 for details of usage of this operation.

##### 8.17.4.3.2	Eecs-er_ECSDiscovery_Subscribe operation

API operation name: Eecs-er_ECSDiscovery_Subscribe

Description: The consumer subscribes for ECS discovery of partner ECSP related events.

Inputs: See clause 8.17.3.10.

Outputs: See clause 8.17.3.10.

See clause 8.17.2.3.3.2 for details of usage of this operation.

##### 8.17.4.3.3	Eecs-er_ECSDiscovery_Notify operation

API operation name: Eecs-er_ECSDiscovery_Notify

Description: The consumer is notified about ECS discovery of partner ECSP related events.

Inputs: See clause 8.17.3.12.

Outputs: None.

See clause 8.17.2.3.3.3 for details of usage of this operation.

##### 8.17.4.3.4	Eecs-er_ECSDiscovery_UpdateSubscription operation

API operation name: Eecs-er_ECSDiscovery_UpdateSubscription

Description: The consumer updates an existing subscription for ECS discovery of partner ECSP related events.

Inputs: See clause 8.17.3.13.

Outputs: See clause 8.17.3.14.

See clause 8.17.2.3.3.4 for details of usage of this operation.

##### 8.17.4.3.5	Eecs-er_ECSDiscovery_Unsubscribe operation

API operation name: Eecs-er_ECSDiscovery_Unsubscribe

Description: The consumer unsubscribes for the previously subscribed ECS discovery of partner ECSP related events.

Inputs: See clause 8.17.3.15.

Outputs See clause 8.17.3.16.

See clause 8.17.2.3.3.5 for details of usage of this operation.

#### 8.17.4.4	Eecs-er_ServiceProvInfoRetrieval API

##### 8.17.4.4.1	Eecs-er_ServiceProvInfoRetrieval_Request operation

API operation name: Eecs-er_ServiceProvInfoRetrieval_Request

Description: The consumer requests to retrieve Service provisioning information from a partner ECS.

Inputs: See clause 8.17.3.17.

Outputs: See clause 8.17.3.18.

See clause 8.17.2.4.2.2 for details of usage of this operation.

##### 8.17.4.4.2	Eecs-er_ServiceProvInfoRetrieval_Subscribe operation

API operation name: Eecs-er_ServiceProvInfoRetrieval_Subscribe

Description: The consumer subscribes for Service provisioning information from a partner ECS related events.

Inputs: See clause 8.17.3.19.

Outputs: See clause 8.17.3.20.

See clause 8.17.2.4.2.3.2 for details of usage of this operation.

##### 8.17.4.4.3	Eecs-er_ServiceProvInfoRetrieval_Notify operation

API operation name: Eecs-er_ServiceProvInfoRetrieval_Notify

Description: The consumer is notified about Service provisioning information from a partner ECS related events.

Inputs: See clause 8.17.3.21.

Outputs: None.

See clause 8.17.2.4.2.3.3 for details of usage of this operation.

##### 8.17.4.4.4	Eecs-er_ServiceProvInfoRetrieval_UpdateSubscription operation

API operation name: Eecs-er_ServiceProvInfoRetrieval_UpdateSubscription

Description: The consumer updates an existing subscription for Service provisioning information from a partner ECS related events.

Inputs: See clause 8.17.3.22.

Outputs: See clause 8.17.3.23.

See clause 8.17.2.4.2.3.4 for details of usage of this operation.

##### 8.17.4.4.5	Eecs-er_ServiceProvInfoRetrieval_Unsubscribe operation

API operation name: Eecs-er_ServiceProvInfoRetrieval_Unsubscribe

Description: The consumer unsubscribes for the previously subscribed Service provisioning information from a partner ECS related events.

Inputs: See clause 8.17.3.24.

Outputs See clause 8.17.3.25.

See clause 8.17.2.4.2.3.5 for details of usage of this operation.

* * * Next Change * * * *

##### 8.18.2.3.3	EAS discovery via Partner ECSP, EEC triggered

If the EEC has the EES information of the partner ECSP and the required security credentials to communicate with the partner ECSP, the EEC can use the EAS discovery procedures specified in clause 8.5 to directly obtain the EAS information from the EES of the partner ECSP. Upon receiving the request from the EEC, the EES of the partner ECSP identifies the EAS(s) considering the allowed MNO information of the registered EASs and UE’s serving MNO information.

* * * Next Change * * * *

### 8.18.4	APIs

#### 8.18.4.1	General

The APIs as defined in clause 8.5.4 are used to perform EAS discovery in partner ECSP.

See clause 8.18.2.3.2, clause 8.18.2.3.3 and clause 8.18.2.3.4 for details of usage of this operation.

* * * End of Change * * * *