3GPP TSG-SA WG6 Meeting #56	S6-232250

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0224 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VAL service area ID based triggering criteria for location reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA6 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Samsung |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2023-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA6 received a LS from CT3 seeking clarification on the usage of VAL service area ID based triggering criteria in the location reporting. This CR aims to add more clarification in the specification to give guidance to stage 3 on the usage of VAL service area ID based trigger criterias. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification in the existing procedures on how to handle the VAL service area ID based triggers |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect implementation in stage 3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 9.3.5	Client-triggered or VAL server-triggered location reporting procedure

Figure 9.3.5-1 illustrates the high level procedure of client-triggered or VAL server-triggered location reporting.

Figure 9.3.5-1: Client-triggered location reporting procedure

1.	Location management client 2 (authorized VAL user or VAL UE) or VAL server sends a location reporting trigger to the location management server to activate a location reporting procedure for obtaining the location information of location management client 1. The location reporting event triggers e.g. minimum time between consecutive reports, SAI changes, access RAT changes, or ECGI changes for reporting the location of the VAL UE, are included. VAL service area ID based location reporting event triggers can be included when the VAL server sends a location reporting trigger to the location management server.

NOTE:	Step 1 can be performed when Location management client 2 or VAL server require to update the location reporting trigger.

2.	Location management server checks whether location management client 2 or VAL server is authorized to send a location reporting trigger. Depending on the information specified by the location reporting trigger, location management server initiates an on-demand location reporting procedure or an event-triggered location reporting procedure for the location of location management client 1.

3.	Once the location information of the location management client 1 is available in the location management server by the on-demand location reporting procedure, a location information report is sent to the location management client 2 or VAL server.

* * * Next Change * * * *

#### 9.3.3.4	Update Location reporting configuration

The location management server may update the location reporting configuration information and inform the location management client of the update ones at any time by triggering the location reporting configuration update procedure.

Figure 9.3.3.4-1 illustrates the procedure for location reporting configuration update.

Figure 9.3.3.4-1: Location reporting configuration update procedure

1.	Based on the local configurations or the location information request from other entities (e.g., another location management client, VAL server) or if the VAL server updates the geographical location information of the existing VAL service area ID and there is triggering criteria corresponding to the updated VAL service area ID, the location management server decides to update the location reporting configuration and informs the updated information to the location management client.

2.	The location management server sends location reporting configuration update request message to the location management client(s) which may contain the updated location reporting configuration, e.g. the event triggers, the minimum time between consecutive reports, the requested location access type and positioning methods when VAL UE fetches the UE location report. This message can be sent over a unicast bearer to a specific location management client or as a group message over an MBMS bearer to update the location reporting configuration for multiple location management clients at the same time. If the location reporting trigger from VAL server contains the triggering criteria based on the VAL service area ID, then the location management server converts those triggers into geographical area based triggers while sending it to the location management client.

NOTE:	Different location management clients may be given different location reporting criteria.

3.	The location management client(s) reply with location reporting configuration update response to the location management server with the update result.

4. The location management client stores or updates the new location reporting configuration received in step 2.

* * * Next Change * * * *

### 9.3.13	VAL Service Area configuration

#### 9.3.13.1	General

The VAL service area refers to the geographical area served by a VAL server. The VAL server may have more than one VAL service area defined. These VAL service areas are configured by the VAL server to the location management server and and are assigned with the unique identifier. When the VAL service area configurations in place the VAL server may use the VAL service area ID as part of the location information in the SEAL APIs requiring location information. These configurations can be updated by the VAL server in-order to modify the geographical area associated with the VAL service area ID and when is updated, the SEAL internally manage the updates without requiring the VAL server to re-invoke the APIs where the updated VAL service area ID are being used.

The VAL server may configure the location reporting trigger with the triggering criteria based on the VAL service area ID. These triggers could be to receive the location report from the VAL client when it enters a particular VAL service area or when it exits a particular VAL service area or whenever the VAL service area of the VAL client changes. Since the VAL service area ID is not configured at the VAL client, the location management server shall convert the VAL service area ID based triggers to the corresponding mapped geographical area while configuring the location management client for location reporting.

* * * End Changes * * *