3GPP TSG-SA WG6 Meeting #56	S6-232354

Goteborg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0185 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modify list of participants by changing the criteria during an ongoing MCVideo ad hoc group call |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Kontron Transportation France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2023-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current standard does not allow to update the participants list of an ongoing communication by changing the criteria, only adding and removing of specific participants is supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The modify ad hoc group call participants request and modify ad hoc group call participants response messages are updated. The procedure on modification of ad hoc group call participants by the initiator is enhanced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The list of ad hoc group call participants cannot be modified during an ongoing communication by changing the criteria. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.19.2.11, 7.19.2.12, 7.19.3.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 7.19.2.11	Modify ad hoc group call participants request (MCVideo client – MCVideo server)

Table 7.19.2.11-1 describes the information flow Modify ad hoc group call participants request from the MCVideo client to the MCVideo server.

Table 7.19.2.11-1: Modify ad hoc group call participants request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo ad hoc group ID | M | The MCVideo group ID of ad hoc group call whose participants list needs to be modified |
| MCVideo ID list (see NOTE 1) | O | List of additional MCVideo users to be added to the on-going ad hoc group call |
| MCVideo ID list (see NOTE 1) | O | List of MCVideo users to be removed from the on-going ad hoc group call |
| Criteria to determine the list of participants (see NOTE 2) | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCVideo server for determining the participants e.g., it can be a location-based criteria to invite participants in a particular area |
| NOTE 1: Either one or both information elements can be present. If this information element is present the IE Criteria to determine the list of participants is not present. NOTE 2: If this information element is present, none of the MCVideo ID list IEs are present. |  |  |

#### 7.19.2.12	Modify ad hoc group call participants response (MCVideo server – MCVideo client)

Table 7.19.2.12-1 describes the information flow Modify ad hoc group call participants response from the MCVideo server to the MCVideo client.

Table 7.19.2.12-1 Modify Ad hoc group call participants response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo ad hoc group ID | M | The MCVideo group ID of ad hoc group call whose participants list needs to be modified |
| Result | M | Result of the modify ad hoc group call participants request (success of failure) |
| MCVideo ID list (see NOTE) | O | List of MCVideo users who are not allowed to be added to the on-going ad hoc group call. This list is provided if the operation is partially success. |
| NOTE: This information element is only present when the request contains a list of additional MCVideo users to be added to the ad hoc group call. |  |  |

# * * * * Second change * * * *

##### 7.19.3.1.4	Modification of ad hoc group call participants by the initiator

Figure 7.19.3.1.4-1 below illustrates the modification of ad hoc group call participants procedure by the initiator of the ad hoc group call.

Pre-conditions:

1.	The MCVideo client 1 is the initiator of the ad hoc group call.

2.	The MCVideo users on MCVideo client 1, MCVideo client 3 to MCVideo client n are on an ongoing ad hoc group call.

3.	The MCVideo user 1 requests to remove the user of MCVideo client 3 from the ad hoc group call and add user of MCVideo client 2 into the on-going ad hoc group call. Alternatively, MCVideo user 1 requests to modify the criteria for determining the list of participants. MCVideo client 3 does not meet the modified criteria while MCVideo client 2 meets the modified criteria.

Figure 7.19.3.1.4-1: Modification of ad hoc group call participants by the initiator

1.	The MCVideo client 1 sends the modify ad hoc group call participants request to the MCVideo server in order to remove MCVideo client 3 from the ongoing ad hoc group call and add MCVideo client 2 into it.

2.	The MCVideo server verifies whether the MCVideo client 1 is authorized to add or remove (modify) the participants of the on-going ad hoc group call. If the request to modify the ad hoc group call participants contains the criteria to determining the list of participants, the MCVideo server determines the list of ad hoc group call participants based on the criteria provided

3.	The MCVideo server sends modify ad hoc group call participants response to the MCVideo client 1.

4.	The MCVideo server sends the ad hoc group call leave request to the MCVideo client 3 in order to remove it from the on-going ad hoc group call.

5.	The MCVideo client 3 notifies the user of the ad hoc group call leave request.

6.	The MCVideo client 3 sends the ad hoc group call leave response to the MCVideo server.

7.	The MCVideo server removes the MCVideo client 3 from the ongoing ad hoc group call.

8.	The MCVideo server sends the ad hoc group call request towards MCVideo client 2.

NOTE 1:	Steps 8 to 10 can occur at any time following step 3.

9.	The receiving MCVideo client 2 notifies the user about the incoming ad hoc group call.

10.	The MCVideo client 2 accepts the ad hoc group call request and send ad hoc group call responses to the MCVideo server. The response may also contain a functional alias of the responding MCVideo user, which is verified (valid and activated for the user) by the MCVideo server.

11.	The MCVideo server may notify the initiating MCVideo user of all the users who are added to the on-going ad hoc group call. This notification may be sent to the initiating MCVideo user by the MCVideo server more than once during the call when MCVideo users join or leave the ad hoc group call.

12.	The MCVideo server may notify the participants about the change in the participants list of on-going ad hoc group call.

NOTE 2:	If the ad hoc group call is associated with an ad hoc group emergency alert and the change of criteria caused the modification of ad hoc group call participant list then the ongoing ad hoc group emergency alert is modified accordingly.

# * * * * End of changes * * * *