3GPP TSG-SA WG6 Meeting #56	S6-232365

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0165 | rev | - | Current version: | 18.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to clause 8.2.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | at&t |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | R-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | UE-2b in clause 8.2.11 should be UE-2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace UE-2b with UE-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will create confusion and misguide stage 3 development. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.2.11	MSGin5G UE bulk de-registration over MSGin5G-6 reference point

The MSGin5G Gateway UE may decide to use bulk de-registration procedure specified in this clause based on the de-registration request sent from MSGin5G Clients in MSGin5G UEs or, if allowed by service policy. The procedure for MSGin5G UE bulk de-registration is illustrated in figure 8.2.11-1.

Pre-conditions:

1.	The MSGin5G Gateway UE has registered to the MSGin5G Server successfully.

2.	MSGin5G UE-1 and MSGin5G UE-2 have registered to the MSGin5G Server via MSGin5G Gateway UE successfully as specified in clause 8.2.8.

Figure 8.2.11-1: MSGin5G UE bulk de-registration

1.	The MSGin5G Client-1 sends an MSGin5G UE de-registration request to the MSGin5G Server as specified in clause 8.2.2, In addition to the Information Elements specified in table 8.2.2-1, the MSGin5G Client-1 may also add the Information Elements specified in table 8.2.11-1 in the MSGin5G UE de-registration request.

Table 8.2.11-1: MSGin5G UE de-registration request

| Information element | Status | Description |
| --- | --- | --- |
| De-registration urgent degree | O | Indicates whether the de-registration request is urgent or not. If a MSGin5G UE de-registration request is indicated as urgent, the MSGin5G UE de-registration request cannot be bulk-handled. |
| De-registration request expiration time | O | Indicates the maximum processing time of the de-registration request allowed by the MSGin5G Client-2. |

2.	Upon receiving the MSGin5G UE de-registration request, the MSGin5G Gateway Client checks whether this request can be bulk-handled based on the service policy and the information included in the MSGin5G UE registration request, e.g.

a)	whether the MSGin5G UE de-registration request is urgent; or

b)	whether bulk de-registration is supported by the MSGin5G service provider;

NOTE 1:	Whether bulk de-registration is supported can be configured in the MSGin5G Gateway Client.

NOTE 2:	The bulk de-registration conditions are specified by application provider or MSGin5G Service provider and are out of scope of this document.

3.	Based on step 2:

a)	if the MSGin5G UE de-registration request cannot be bulk-handled, MSGin5G Gateway Client forwards the de-registration request to MSGin5G Server; or

b)	if the MSGin5G UE de-registration request can be bulked, MSGin5G Client-1 sends a bulk de-registration notification to MSGin5G Client-1 and caches/stores the MSGin5G UE de-registration request until the bulk de-registration conditions can be fulfilled. The Information Elements specified in table 8.2.11-2 are included in the bulk de-registration notification. The steps 4 and 5 are skipped.

Table 8.2.11-2: bulk de-registration notification

| Information element | Status | Description |
| --- | --- | --- |
| UE Service ID | M | UE service identifier assigned to the requesting MSGin5G UE. |
| Expected de-registration time | O | The expected time when the bulk de-registration can be handled and the MSGin5G UE de-registration response can be received. |

4.	The MSGin5G Server handles the MSGin5G UE de-registration request as specified in clause 8.2.2 and sends the corresponding MSGin5G UE de-registration response to MSGin5G Gateway Client.

5.	The MSGin5G Gateway Client sends the MSGin5G UE de-registration response to MSGin5G Client-1. The MSGin5G Client-1 handles the MSGin5G UE de-registration response as specified in clause 8.2.2 and skips all the remaining steps in this clause.

6.	MSGin5G Client-2 sends an MSGin5G UE de-registration request and the MSGin5G UE de-registration request is handled as in steps 2-5.

7.	The MSGin5G Gateway Client finds that the conditions of sending a bulk de-registration request can be fulfilled, e.g.

a)	the de-registration request expiration time in the MSGin5G UE de-registration request sent from MSGin5G Client-1 or MSGin5G Client-2 is reached; or

b)	the periodic bulk de-registration interval is reached; or

c)	the maximum MSGin5G UE de-registration request number in a MSGin5G UE bulk de-registration request is reached.

NOTE 3:	The maximum de-registration time, periodic bulk de-registration interval and maximum MSGin5G UE de-registration request number are implementation specific and out of the scope of the current specification.

8.	The MSGin5G Gateway Client includes all cached/stored MSGin5G UE de-registration requests in an MSGin5G UE bulk de-registration request and sends it to the MSGin5G Server. The Information Elements specified in table 8.2.11-3 are included in the MSGin5G UE bulk de-registration request.

Table 8.2.11-3: MSGin5G UE bulk de-registration request

| Information element | Status | Description |
| --- | --- | --- |
| Number of individual MSGin5G UE de-registration requests | M | Indicates total number of MSGin5G UE de-registration requests which are bulked in this MSGin5G UE bulk de-registration request |
| List of individual MSGin5G UE de-registration request | M | Each element in this list contains information as specified in Table 8.2.2-1. |

9.	Upon receiving the MSGin5G UE bulk de-registration request, the MSGin5G Server processes each of the MSGin5G UE de-registration from the list individually as specified in clause 8.2.2.

Editor's note:	Security aspects of MSGin5G UE bulk de-registration is the responsibility of SA3.

10.	The MSGin5G Server includes all corresponding MSGin5G UE de-registration responses in an MSGin5G UE bulk de-registration response and sends it to MSGin5G Gateway Client. The Information Elements specified in table 8.2.11-4 are included in the MSGin5G UE bulk de-registration response.

Table 8.2.11-4: MSGin5G UE bulk de-registration response

| Information element | Status | Description |
| --- | --- | --- |
| Number of individual MSGin5G UE de-registration responses | M | Indicates total number of MSGin5G UE de-registration responses which are bulked in this MSGin5G UE bulk de-registration response |
| List of individual MSGin5G UE de-registration response | M | Each element in this list contains information as specified in Table 8.2.2-2 |

11.	Upon receiving the MSGin5G UE bulk de-registration response, the MSGin5G Gateway Client splits the MSGin5G UE bulk de-registration response into multiple individual MSGin5G UE de-registration response, each individual MSGin5G UE de-registration response contains information as specified in Table 8.2.2-2.

12.	MSGin5G Gateway Client sends the MSGin5G UE de-registration Response 1 to MSGin5G Client-1.

13.	MSGin5G Gateway Client sends the MSGin5G UE de-registration Response 2 to MSGin5G Client-2.