3GPP TSG-SA WG6 Meeting #56	S6-232376

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0018 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PIN Service Switch subscription |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2023-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The PIN Service Switch Configure procedure in clause 8.7.2.1.3 defines a request sent from the PIN Server to the AS and contains the following EN. Editor's note: Whether subscription/notification mechanisms are used for informing the AS over PIN-9 reference point is FFS. Because AS API is not standardized, it is proposed to replace the request sent to the AS in this procedure by a notification and define the realted AS subscription mechanism. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Service Switch Configuration subscription/notification is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure cannot be realized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.2.1.3, 8.7.2.1.4 (new), 8.7.2.1.4.1 (new), 8.7.2.1.4.2 (new) 8.7.2.1.4.2.1 (new), 8.7.2.1.4.2.2 (new), 8.7.2.1.4.2.3, (new), 8.7.2.1.4.2.4 (new), 8.7.3.1, 8.7.3.10 (new), 8.7.3.11 (new), 8.7.3.12 (new), 8.7.3.13 (new), 8.7.3.14 (new), 8.7.3.15 (new), 8.7.3.16 (new), 8.7.4, 8.7.4.1 (new), 8.7.4.2 (new),8.7.4.3 (new), 8.7.4.4 (new), 8.7.4.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 8.7.2.1.3	PIN Service Switch Configure procedure

Pre-conditions:

1.	The PIN Client successfully requested a service switch with the PIN Server

2.	The AS has successfully registered to the PIN Server to receive PIN service switch notifications

Figure 8.7.2.1.3-1: PIN Configure Service Switch procedure

1.	The PIN Server sends the PIN Configuration Service Switch Configure request to the PIN Management Client. The PIN Configuration Service Switch Configure request includes the requestor identifier, security credential, Application Client identifier, Application Server identifier, target PIN Client, application session identifier and can include IP 4 tuple that descibes the traffic of the application session.

NOTE 1:	The PIN Server first performs PIN Configure Service Switch procedure with the PIN Management Client, then if successful, notifies the Application Server.

2.	Upon receiving the request from the PIN Server, the PIN Management Client checks if the PIN Server is authorized to request service switch configuration and validates the request.

If the request is authorized and valid, the PIN Management Client prepares for sending PIN Configure Service Switch request to the PIN Gateway Client or the target PIN Client. The PIN Management Client can use the PIN ID to identify the PIN Gateway Client instance and the target PIN Client identifier to identify the target PIN Client.

3.	The PIN Management Client sends the PIN Management Service Switch Configure request to the PIN Gateway Client or the target PIN Client including the information defined in step 1.

NOTE 2:	The PIN Management Client first performs PIN Configure Service Switch procedure with the PIN Gateway Client, then if successful, with the target PIN Client.

4.	Upon receiving the request from the PIN Management Client, the PIN Gateway Client or the target PIN Client checks if the PIN Managment Client is authorized to request service switch configuration and validates the request. If the PIN Management Client is authorized and the request is valid, the PIN Gateway Client or the target PIN Client use the information provided in the request to switch the application session to the target PIN Client.

NOTE 3:	The target Application Client needs to receive the application context prior to re-establishing a switched application session with the Application Server.

NOTE 4:	How the target Application Client recovers the switched application session is out of scope of this specification.

5.	If the processing of the request was successful, the PIN Gateway Client or the target PIN Client send a PIN management Service Switch Configure response to the PIN Management Client indicating the processing was successful. Otherwise, the PIN Gateway Client or target PIN Client send a PIN Management Service Switch Configure response to the PIN Management Client indicating that processing the request failed and can include appropriate reasons.

6.	If the processing of the request was successful, the PIN Management Client or Application Server send a PIN Configuration Service Switch Configure response to the PIN Server that indicates the request was successfully processed. Otherwise, the PIN Management Client or Application Server send a PIN Configuration Service Switch Configure response to the PIN Server indicating the processing the request failed and can include appropriate reasons.

7.	The PIN Server sends the PIN service switch notification to the Application Server if the procedure with the PIN Management Client was successful in step 6. The PIN service switch notification includes the PIN server identifier, PIN identifier, Application Client identifier, target PIN Client, application session identifier and can include IP 4 tuple that descibes the traffic of the application session.

8.	Upon receiving the PIN service switch notification, the Application Server uses the information provided in the notification to switch the application session to the target PIN client.

* * * Next Change * * * *

##### 8.7.2.1.4	PIN Service Switch subscription

###### 8.7.2.1.4.1	General

The PIN service switch subscription is used by the Application Server to be notified of PIN service switch events by the PIN Server.

The PIN service switch notification events includes PIN service switch configuration information.

###### 8.7.2.1.4.2	Procedure

8.7.2.1.4.2.1	PIN service switch subscribe

Figure 8.7.2.1.4.2.1-1 illustrates the PIN service switch subscribe procedure.

Pre-conditions:

1.	The application server has already received the address of the PIN server;

2.	The application server has been authorized to communicate with PIN server;

Figure 8.7.2.1.4.2.1-1: PIN service switch subscribe

1.	The application server sends the PIN service switch subscribe request to the PIN server. The PIN service switch subscribe request includes the application server identifier along with the security credentials.

2.	Upon receiving the request from the application server, the PIN server checks if the application server is authorized to subscribe. The authorization check may apply to an individual PIN. If the request is authorized, the PIN server creates and stores the subscription for application server.

3.	If the processing of the request is successful, the PIN server sends a PIN service switch subscribe response to the application server, which includes the subscription identifier and may include the expiration time, indicating when the subscription will automatically expire. To maintain the subscription, the application server shall send a PIN service switch update request prior to the expiration time. If a new PIN service switch update request is not received prior to the expiration time, the PIN server shall treat the application server as implicitly unsubscribed.

8.7.2.1.4.2.2	PIN service switch notify

Figure 8.7.2.1.4.2.2-1 illustrates the PIN service switch notify procedure.

Pre-conditions:

1.	The application server has subscribed with the PIN server as specified in clause 8.7.2.1.4.2.1.

Figure 8.7.2.1.4.2.2-1: PIN service switch notify procedure

1.	When a service switch event occurs at the PIN server that satisfies triggering conditions, the PIN server notifies subscribed application servers.

2.	The PIN server sends an PIN service switch notification to the subscribed application server(s) related to the service switch event and includes the PIN service switch configuration infromation.

The PIN service switch configuration information includes PIN server identifier, Application Client identifier, Application Server identifier, target PIN Client, application session identifier and can include IP 4 tuple that descibes the traffic of the application session.

3.	Upon receiving the notification, the application server uses the information provided in the notification to switch the application session to the target PIN client.

8.7.2.1.4.2.3	PIN service switch update

Figure 8.7.2.1.4.2.3-1 illustrates the PIN service switch update procedure.

Pre-conditions:

1.	The application server has subscribed with the PIN server as specified in clause 8.7.2.1.4.2.1.

Figure 8.7.2.1.4.2.3-1: PIN service switch update

1.	The application server sends a PIN service switch update request to the PIN server. The PIN service switch update request includes the security credentials and the subscription identifier. It may also include notification target address and proposed expiration time.

2.	Upon receiving the request from the application server, the PIN server checks if the application server is authorized to update the subscription information. If the request is authorized, the PIN server updates the stored subscription.

3.	If the processing of the request is successful, the PIN server sends a PIN service switch update response to the application server, which may include the expiration time, indicating when the subscription will automatically expire. To maintain the subscription, the application server shall send a PIN service switch subscription update request prior to the expiration time. If a PIN service switch update request is not received prior to the expiration time, the PIN server shall treat the application server as implicitly unsubscribed.

8.7.2.1.4.2.4	PIN service switch unsubscribe

Figure 8.7.2.1.4.2.4-1 illustrates the PIN service switch unsubscribe procedure.

Pre-conditions:

1.	The application server has subscribed with the PIN server as specified in clause 8.7.2.1.4.2.1.

Figure 8.7.2.1.4.2.4-1: PIN service switch unsubscribe

1.	The application server sends a PIN service switch unsubscribe request to the PIN server. The PIN service switch unsubscribe request includes the security credentials and the subscription identifier.

2.	Upon receiving the request from the application server, the PIN server checks if the application server is authorized to unsubscribe. If the request is authorized, the PIN server cancels the subscription indicated by the subscription identifier.

3.	If the processing of the request is successful, the PIN server sends a PIN service switch unsubscribe response to the application server.

* * * Next Change * * * *

#### 8.7.3.1	General

The following information flows are specified for Service Switch:

-	PIN service switch request and response

-	PIN configuration service switch configure request and response

-	PIN management service switch configure request and response

-	PIN service discovery request and response

-	PIN service switch subscribe

-	PIN service switch notify

-	PIN service switch update

-	PIN service switch unsubscribe

* * * Next Change * * * *

#### 8.7.3.10	PIN service switch subscribe request

Table 8.7.3.10-1: PIN service switch subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Application server ID | M | Unique identifier of the application server. |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |
| PIN ID | M | The identifier of PIN. |
| Notification Target Address | M | The Notification target address (e.g., URL) where the notifications destined for the application server should be sent. |
| Proposed expiration time | O | Proposed expiration time for the subscription |

#### 8.7.3.11	PIN service switch subscribe response

Table 8.7.3.11-1: PIN service switch subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (see NOTE) | O | Indicates that the subscription request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response (see NOTE) | O | Indicates that the subscription request failed. |
| > Cause | O | Indicates the cause of subscription request failure |
| NOTE: One IE is included in the response. |  |  |

#### 8.7.3.12	PIN service switch notify

Table 8.7.3.12-1 describes information elements in the PIN service switch notification from the PIN server to application server.

Table 8.7.3.12-1: PIN service switch notification

| Information element | Status | Description |
| --- | --- | --- |
| PIN server identifier | M | Identifier of the PIN server sending the notification. |
| PIN identifier | M | Identifier of the PIN. |
| Application client identifier | M | Identifier of the application client. |
| Application session identifier | M | Identifier of the application session. |
| Application session descriptor | O | Descriptor of application traffic flows (e.g., IP 4 tuple) |
| Target PIN client identifier | M | Target PIN client identifier. |

#### 8.7.3.13	PIN service switch update request

Table 8.7.3.13-1: PIN service switch update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription. |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |
| Notification Target Address | O | The Notification target address (e.g., URL) where the notifications destined for the application server should be sent to. |
| Proposed expiration time | O | Proposed expiration time for the subscription |

#### 8.7.3.14	PIN service switch update response

Table 8.7.3.14-1: PIN service switch update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (see NOTE) | O | Indicates that the subscription update request was successful. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response (see NOTE) | O | Indicates that the subscription update request failed. |
| > Cause | M | Indicates the cause of subscription update request failure |
| NOTE: One IE is included in the response. |  |  |

#### 8.7.3.15	PIN service switch unsubscribe request

Table 8.7.3.15-1: PIN service switch unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription. |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |

#### 8.7.3.16	PIN service switch unsubscribe response

Table 8.7.3.16-1: PIN service switch unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (see NOTE) | O | Indicates that the unsubscribe request was successful. |
| Failure response (see NOTE) | O | Indicates that the unsubscribe request failed. |
| > Cause | M | Indicates the cause of unsubscribe request failure |
| NOTE: One IE is included in the response. |  |  |

* * * Next Change * * * *

### 8.7.4	APIs

#### 8.7.4.1	General

Table 8.9.4.1-1 illustrates the API for AS service switch subscription.

Table 8.9.4.1-1: Ppinserver_ASServiceSwitchSubscription API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Ppinserver_ASServiceSwitch | Subscribe | Subscribe/Notify | AS |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |

#### 8.9.4.2	Ppinserver_ ASServiceSwitch_Subscribe operation

API operation name: Ppinserver_ASServiceSwitch_Subscribe

Description: The consumer subscribes for service switch information.

Inputs: See clause 8.7.3.10.

Outputs: See clause 8.7.3.11.

See clause 8.7.2.1.4.2.1 for details of usage of this operation.

#### 8.7.4.3	Ppinserver_ ASServiceSwitch_Notify operation

API operation name: Ppinserver_ASServiceSwitch_Notify

Description: The consumer is notified with service switch information.

Inputs: See clause 8.7.3.12.

Outputs: None.

See clause 8.7.2.1.4.2.2 for details of usage of this operation.

#### 8.7.4.4	Ppinserver_ ASServiceSwitch_ UpdateSubscription operation

API operation name: Ppinserver_ASServiceSwitch_UpdateSubscription

Description: The consumer updates an existing subscription for service switch information.

Inputs: See clause 8.7.3.13.

Outputs: See clause 8.7.3.14.

See clause 8.7.2.1.4.2.3 for details of usage of this operation.

#### 8.7.4.5	Ppinserver_ ASServiceSwitch_ Unsubscribe operation

API operation name: Ppinserver_ASServiceSwitch_Unsubscribe

Description: The consumer cancels an existing subscription for service switch information.

Inputs: See clause 8.7.3.15.

Outputs: See clause 8.7.3.16.

See clause 8.7.2.1.4.2.4 for details of usage of this operation.

* * * End of Change(s) * * * *