3GPP TSG-SA WG6 Meeting #56	S6-232385

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0440 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | API definition of roaming and federation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | API definition for roaming and federation is undefined in clause 8.17 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add API definition for roaming and federation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2, 8.17.4 (new), 8.17.4.1 (new), 8.17.4.2 (new), 8.17.4.2.1 (new), 8.17.4.2.2 (new), 8.17.4.2.3 (new), 8.17.4.2.4 (new), 8.17.4.3 (new), 8.17.4.3.1 (new), 8.17.4.3.2 (new), 8.17.4.3.3 (new), 8.17.4.3.4 (new), 8.17.4.3.5 (new), 8.17.4.3.6 (new), 8.17.4.4 (new), 8.17.4.4.1 (new), 8.17.4.4.2 (new), 8.17.4.4.3 (new), 8.17.4.4.4 (new), 8.17.4.4.5 (new), 8.17.4.4.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.7.2	APIs provided by the Edge Enabler Layer

Table 6.7.2-1 summarizes the APIs exposed by the ECS.

Table 6.7.2-1: APIs provided by the ECS

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eecs_ServiceProvisioning | EEC | 8.3 |
| Eecs_EESRegistration | EES | 8.4.4 |
| Eecs_TargetEESDiscovery | EES | 8.8.3.3 |
| Eecs_ECSRegistration | ECS | 8.17.2.2 |
| Eecs_ECSDiscovery | ECS | 8.17.2.3 |
| Eecs_ECSServiceProvisioning | ECS | 8.17.2.4 |

Table 6.7.2-2 summarizes the APIs exposed the EES.

Table 6.7.2-2: APIs provided by the EES

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eees_EECRegistration | EEC | 8.4.2 |
| Eees_EASRegistration | EAS | 8.4.3 |
| Eees_EASDiscovery | EEC | 8.5 |
| Eees_UELocation | EAS | 8.6.2 |
| Eees_ACRManagementEvent | EAS | 8.6.3 |
| Eees_AppClientInformation | EAS | 8.6.4 |
| Eees_UEIdentifier | EEC, EAS | 8.6.5 |
| Eees_SessionWithQoS | EAS | 8.6.6 |
| Eees_TargetEASDiscovery | EAS, EES | 8.8.3.2 |
| Eees_AppContextRelocation | EEC, EAS | 8.8.3.4 |
| Eees_ACREvents | EEC | 8.8.3.5 |
| Eees_EELManagedACR | EAS | 8.8.3.6 |
| Eees_EECContextPull | EES | 8.9.4.2 |
| Eees_EECContextPush | EES | 8.9.4.3 |
| Eees_SelectedTargetEAS | EAS | 8.8.3.7 |
| Eees_ACRStatusUpdate | EAS | 8.8.3.8 |

NOTE:	The event exposure related APIs (e.g. Eees_EASDiscovery and Eees_ACREvents) can be realized as single event subscription API.

* * * Next Change * * * *

### 8.17.4	APIs

#### 8.17.4.1	General

Table 8.17.4.1-1 illustrates the APIs for roaming and federation.

Table 8.17.4.1-1: Roaming and federation APIs

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eecs_ECSRegistration | Request | Request/Response | ECS |
|  | Update |  |  |
|  | Deregister |  |  |
| Eecs_ECSDiscovery | Request | Request/Response | ECS |
|  | Subscribe | Subscribe/Notify | ECS |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |
| Eecs_ECSServiceProvisioning | Request | Request/Response | ECS |
|  | Subscribe | Subscribe/Notify | ECS |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |

#### 8.17.4.2	Eecs_ECSRegistration API

##### 8.17.4.2.1	General

This clause describes the Eecs_ECSRegistration API and its operations.

##### 8.17.4.2.2	Eecs_ECSRegistration_Request operation

API operation name: Eecs_ECSRegistration_Request

Description: The consumer requests to register the ECS on the ECS-ER.

Inputs: See clause 8.17.3.2.

Outputs: See clause 8.17.3.3.

See clause 8.17.2.2.2 for details of usage of this operation.

##### 8.17.4.2.3	Eecs_ECSRegistration_Update operation

API operation name: Eecs_ECSRegistration_Update

Description: The consumer requests to update the registered information of the ECS on the ECS-ER.

Inputs: See clause 8.17.3.4.

Outputs: See clause 8.17.3.5.

See clause 8.17.2.2.3 for details of usage of this operation.

##### 8.17.4.2.4	Eecs_ECSRegistration_Deregister operation

API operation name: Eecs_ECSRegistration_Deregister

Description: The consumer requests to deregister the ECS from the ECS-ER.

Inputs: See clause 8.17.3.6.

Outputs: See clause 8.17.3.7.

See clause 8.17.2.2.4 for details of usage of this operation.

#### 8.17.4.3	Eecs_ECSDiscovery API

##### 8.17.4.3.1	General

This clause describes the Eecs_ECSDiscovery API and its operations.

##### 8.17.4.3.2	Eecs_ECSDiscovery_Request operation

API operation name: Eecs_ECSDiscovery_Request

Description: The consumer requests for one time ECS discovery information.

Inputs: See clause 8.17.3.8.

Outputs: See clause 8.17.3.9.

See clause 8.17.2.3.2 for details of usage of this operation.

##### 8.17.4.3.3	Eecs_ECSDiscovery_Subscribe operation

API operation name: Eecs_ECSDiscovery_Subscribe

Description: The consumer subscribes for ECS discovery information.

Inputs: See clause 8.17.3.10.

Outputs: See clause 8.17.3.11.

See clause 8.17.2.3.3.2 for details of usage of this operation.

##### 8.17.4.3.4	Eecs_ECSDiscovery_Notify operation

API operation name: Eecs_ECSDiscovery_Notify

Description: The consumer is notified with ECS discovery information.

Inputs: See clause 8.17.3.12.

Outputs: None.

See clause 8.17.2.3.3.3 for details of usage of this operation.

##### 8.17.4.3.5	Eecs_ECSDiscovery_UpdateSubscription operation

API operation name: Eecs_ECSDiscovery_UpdateSubscription

Description: The consumer updates an existing subscription for ECS discovery information.

Inputs: See clause 8.17.3.13.

Outputs: See clause 8.17.3.14.

See clause 8.17.2.3.3.4 for details of usage of this operation.

##### 8.17.4.3.6	Eecs_ECSDiscovery_Unsubscribe operation

API operation name: Eecs_ECSDiscovery_Unsubscribe

Description: The consumer cancels an existing subscription for ECS discovery information.

Inputs: See clause 8.17.3.15.

Outputs: See clause 8.17.3.16.

See clause 8.17.2.3.3.5 for details of usage of this operation.

#### 8.17.4.4	Eecs_ECSServiceProvisioning API

##### 8.17.4.4.1	General

This clause describes the Eecs_ECSServiceProvisioning API and its operations.

##### 8.17.4.4.2	Eecs_ECSServiceProvisioning_Request operation

API operation name: Eecs_ECSServiceProvisioning_Request

Description: The consumer requests for one time ECS service provisioning information.

Inputs: See clause 8.17.3.17.

Outputs: See clause 8.17.3.18.

See clause 8.17.2.4.2.2 for details of usage of this operation.

##### 8.17.4.4.3	Eecs_ECSServiceProvisioning_Subscribe operation

API operation name: Eecs_ECSServiceProvisioning_Subscribe

Description: The consumer subscribes for ECS service provisioning information.

Inputs: See clause 8.17.3.19.

Outputs: See clause 8.17.3.20.

See clause 8.17.2.4.2.3.2 for details of usage of this operation.

##### 8.17.4.4.4	Eecs_ECSServiceProvisioning_Notify operation

API operation name: Eecs_ECSServiceProvisioning_Notify

Description: The consumer is notified with ECS service provisioning information.

Inputs: See clause 8.17.3.21.

Outputs: None.

See clause 8.17.2.4.2.3.3 for details of usage of this operation.

##### 8.17.4.4.5	Eecs_ECSServiceProvisioning_UpdateSubscription operation

API operation name: Eecs_ECSServiceProvisioning_UpdateSubscription

Description: The consumer updates an existing subscription for ECS service provisioning information.

Inputs: See clause 8.17.3.22.

Outputs: See clause 8.17.3.23.

See clause 8.17.2.4.2.3.4 for details of usage of this operation.

##### 8.17.4.4.6	Eecs_ECSServiceProvisioning_Unsubscribe operation

API operation name: Eecs_ECSServiceProvisioning_Unsubscribe

Description: The consumer cancels an existing subscription for ECS service provisioning information.

Inputs: See clause 8.17.3.24.

Outputs: See clause 8.17.3.25.

See clause 8.17.2.4.2.3.5 for details of usage of this operation.

* * * End of Change(s) * * * *