3GPP TSG-SA WG6 Meeting #56	S6-232389

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS23.558 | CR | 0441 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to 6.3.4, 8.3.3.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA6 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - There are some editorials need to be corrected. Especially for table 8.3.3.3.2, the Application Group profile shall be the child IE of the Application information, otherwise, the table could be misleading. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the editorial in 6.3.4, 8.3.3.3.2； |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The information may be misleading |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.4, 8.3.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.3.4	Edge Configuration Server (ECS)

ECS provides supporting functions needed for the EEC to connect with an EES.

Functionalities of ECS are:

a)	provisioning of Edge configuration information to the EEC. The Edge configuration information includes the following:

1)	the information for the EEC to distinguish amongst the EESs (e.g. EDN service area); and

2)	the information for establishing a connection with EESs (such as URI);

b)	 providing  the T-EES information to the S-EES;

NOTE:	The ECS can be deployed in the MNO domain or can be deployed in 3rd party domain by service provider.

c)	supporting the functionalities of registration (i.e., registration, update, and de-registration) for the EES(s);

d)	supporting the functionalities of API invoker and API exposing function as specified in 3GPP TS 23.222 [6];

e)	interacting with 3GPP Core Network for accessing the capabilities of network functions either directly (e.g. via PCF) or indirectly (i.e. SCEF/NEF/SCEF+NEF).

f)	providing service provisioning information to a partner ECS; and

g)	retrieving service provisioning information from a partner ECS.

If the ECSP is required to support service provisioning for a certain EAS whose information is not within the pre-configuration or OAM configuration, the ECS can use Edge repository functions to support ECS discovery via ECS-ER. To support functions of the edge repository the ECS supports the following functionality(ies):

a)	registering and providing edge deployment information to ECS-ER.

To support functions of the edge repository the functionalities of the enhanced ECS i.e. ECS-ER, in addition to ECS functionalities, are:

a)	supporting the functionalities of registration (i.e., registration, update, and de-registration) for the ECS(s);

b)	receiving and storing information about edge computing resources from the ECS(s);

c)	receiving and storing information about edge computing resources from other ECS-ER(s); and

d)	providing information about Edge computing resources to other ECS-ER(s) of the federation.

An ECS can be enhanced to support discovery of a common EAS for a set of UEs. Such enhanced ECSs have the Central Repository functionality. In addition to ECS functionalities, ECS with Central Repository support the following functions:

a)	storing and providing common EES and common EAS information.

* * * Next Change * * * *

8.3.3.3.2	Service provisioning request

Table 8.3.3.3.2-1 describes the information elements for service provisioning request from the EEC to the ECS.

Table 8.3.3.3.2-1: Service provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| EECID | M | Unique identifier of the EEC. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| AC Profile(s) (NOTE) | O | Information about services the EEC wants to connect to, as described in Table 8.2.2-1. |
| Application information (NOTE) | O | List of information about services the EEC wants to connect to, including the option to provide application group profile information. |
| > AC Profile | M | Application Profile as described in Table 8.2.2-1. |
| >Application Group profile | O | Application Group profile associated with the AC Profile, as defined in Table 8.2.11-1. |
| EEC Service Continuity Support | O | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC. When requesting service provisioning for T-EES discovery, if the EEC requires that T-EES must support "EEC executed ACR via T-EES" scenario, then EEC includes only "EEC executed ACR via T-EES" in this IE |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Connectivity information | O | List of connectivity information for the UE, e.g. PLMN ID, SSID. |
| UE location | O | The location information of the UE. The UE location is described in clause 7.3.2. |
| ECSP identifiers | O | The list of EEC preferred ECSPs that provide the EES. |
| NOTE: Only one of AC Profile(s) or Application information shall be provided. |  |  |